SA WG2 Meeting #S2-127	S2-184315

(was) S2-183588

23 - 27 April, 2018, Sanya, China

Source:	Cisco Systems

Title:	AF Control of Make-before-break PDN Connection

Document for:	Approval

Agenda Item:	6.17

Work Item / Release:	FS_LLC_Mob / Rel-16

Abstract of the contribution: Proposes a "make-before-break" PDN connection relocation that is controlled by AF.

## Introduction

The proposed solution is a variation of existing solution 2 in the TR, with the modification that the trigger is from an AF that subscribes to location update changes from PCRF.

################### TEXT PROPOSAL FOR THE TR 23.739 ########################

## 6.X	Solution #X: AF Control of Make-before-break PDN Connection

### 6.X.1	Introduction

The solution addresses Key Issue #2 and Key Issue of creation of make-before-break PDN connection. It is supplementary to Solution 3, where the AF triggers the IP address reallocation instead of the PGW.

### 6.X.2	Functional Description

An example deployment to which both the solutions apply is the shown in the figure below.

Figure 6.X.2-1 Architecture of LLC Data network.

Details are described in Section 6.2.2.1. In addition to the description in Section 6.3.2.1, the following is assumed:

-	AF is aware that edge compute resources are available in PRA1 (edge data center 1) and PRA2 (edge data center 2) and that there is a PGW-U for breakout to the edge compute resources.

Editor’s Note: If is FFS how the AF is made aware of location corresponding to where edge compute resources are placed.

### 6.X.3	Procedure

Depicted in Figure 6.X.3-1 below is the procedure for AF control of “make-before-break” PDN connection mobility in EPS.

Figure 6.X.3-1: “Make-before-break” PDN connection controlled by AF

1a-1d. 	UE Requested PDN Connectivity procedure or Attach with PDN Establishment from TS 23.401 [3] is performed for LLC APN. The S/PGW-C selects a user-plane function S/PGW-U1 which is in the central data center.

2a-2h.	The mobile communicated with AF. The AF creates an Rx session with the PCRF and subscribes to location update change notification for PRA-list. The PRA-list is the list of edge data center..

3a-3e.	The UE moves (handover or idle-mode) to eNB that is in PRA1. The UE performs tracking area update. The MME then sends location update notification as described in TS 23.401 [3] to the S/PGW-C. The S/PGW-C further notifies the PCRF which then notifies the AF.

4a-4b.	The AF determines that there are edge compute resources in PRA which can benefit from the UE having an IP address allocated from user-plane node in PRA1. The AF provides a new trigger to the PCR that the IP address of the UE should be changed. This trigger is then provided to the S/PGW-C.

5. 	Steps  5-13 from Figure 6.2.3.1 are performed where a new S/PGW-U is selected using  PRA1.

6.	UL/DL traffic flows through S/PGW-U2.

The following steps are new:

1.	Step 2b, AF subscription to PRA changes.

2.	Step 4a and Step 4b, Provision of Traffic Steering Rule to change anchor for the PDN Connection

### 6.X.4	Impacts on existing entities and interfaces

Editor's note:	This clause describes impacts to existing entities and interfaces.

### PCF:

### -	Subscription to location notification on Rx interface

### -	Support of new traffic steering rule to change anchor corresponding to a PDN connection.

### PGW-U Selection

### -	PGW-U selection based on PRA-ID.

### 6.X.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

################### END OF PROPOSAL ########################