SA WG2 Meeting #S2-127	S2-184316

23 - 27 April, 2018, Sanya, China	rev of S2-183802

Source:	Vodafone

Title:	Update to the impacts and evaluation for Solution 3 “PCRF initiated S/PGW-U change based on detection of applications requiring low latency”

Document for:	Approval

Agenda Item:	6.17

Work Item / Release:	FS_LLC_Mob / Rel-16

## Proposal

It is proposed to make the following revision marked changes to section 6.3 of TR 23.739 v0.2.1.

################### Start of changes to TR 23.739 v0.2.1 ########################

## 6.3	Solution #3: PCRF initiated S/PGW-U change based on detection of applications requiring low latency

### 6.3.1	Introduction

The solution addresses Key Issue #1 "How to detect the activation of a service requiring low latency user plane support". It uses components from Solution 6.2 "Low latency PDN Connection mobility initiated by PGW".

This solution targets the use case where an operator has much of their (legacy) SGi-LAN infrastructure centralized and only wishes to use the limited capacity of "Edge" SGi-LAN infrastructure when the UE is actually using an application that will benefit from the low user plane latency.

### 6.3.2	Functional Description

The MME uses the APN to select a combined S/PGW-C.

The operator has prior knowledge of which applications (and which UEs) they want to provide with very low latency. The PCRF requests application detection to detect when those UEs are using those applications. Upon the detection of the application, the PCRF requests the PGW-C to use the low latency S/PGW-U.

The S/PGW-C uses User Location Reporting/Presence Reporting Area to know the UE's current location, and then uses its DNS to determine the correct S/PGW-U. If a change of S/PGW-U is needed, the relevant components of the procedure in Solution 6.2 "Low latency PDN Connection mobility initiated by PGW" can be used to move the S/PGW-U.

### 6.3.3	Procedure

This solution describes a triggering mechanism that, after triggering, can then utilize the components in Solution 6.2 "Low latency PDN Connection mobility initiated by PGW" to move the S/PGW-U. The solution comprises the following steps:

1)	At PDN connection establishment, the PGW-C contacts the PCRF (IMSI, APN). PCRF selection is e.g. based on the IMSI.

2)	The PCRF contacts the SPR/UDR and the SPR/UDR downloads subscription data relevant for that APN and that UE. For this solution, the APN/UE combination is tagged in the SPR/UDR for "application dependent low latency". An alternative is that the local configuration of the PCRF (e.g. based on the APN) is used to determine that "application dependent low latency" is needed.

3)	The PCRF requests the PDN-GW to activate User Location Reporting or Presence Reporting Area reporting.

4)	The PCRF requests the TDF/PCEF to detect (and report) when traffic for the low latency application is detected.

5)	When the low latency application is detected, the TDF/PCEF reports the event to the PCRF.

6)	Step 6a OR step 6b is performed:

6a)	The PCRF then modifies the QoS (of the default bearer) to e.g. QCI 80 ("low latency Mobile BroadBand), or requests the establishment of a dedicated bearer for low latency e.g. with QCI 80.

6b)	A new indication is sent by the PCRF to the PGW-C to request the use of the low latency S/PGW-U for that UE & PDN connection.

7)	The PGW-C uses its DNS (or local configuration) to determine the appropriate S/PGW-U using APN and the UE's last reported eNB ID/TAI as inputs. This type of DNS enquiry is specified in clause 5.10 of TS 29.303 [4].

8)	If the DNS response (or use of the local configuration) indicates that a change of S/PGW-U is needed, then the relevant components of the procedures in Solution 6.x "Low latency PDN Connection mobility initiated by PGW" are used to cause the UE to establish a new PDN connection.

9)	Step 9a OR step 9b is performed

9a)	If "make before break" is used, the PDN connection establishment will be routed to the existing PGW-C, and the PGW-C can use this 'duplicate request' as the trigger to select the low latency S/PGW-U. This procedure might be made easier if a "token" is sent to the UE in the PCO and the UE returns the "token" in the PCO in the subsequent PDN establishment procedure.

9b)	If "break then make" is used, then at PDN connection release, the PCRF stores (in the SPR/UDR or in a local database) the need for the subsequent PDN connection establishment to be immediately established with the low latency S/PGW-U.

After step 9, the PCRF may request the TDF/PCEF at the new S/PGW-U location to detect (and report) the sustained absence of traffic for the low latency application(s). Subsequently, the PCRF can indicate the absence of low latency traffic to the PGW-C (with either a new indication or by modifying the QCI back to its earlier value), and the PGW-C can mark that PDN connection as suitable for being returned to a central S/PGW-U, e.g. in the situation that the low latency S/PGW-U becomes heavily loaded.

### 6.3.4	Impacts on existing entities and interfaces

a)	The SPR/UDR/PCRF needs to be configured with data as to which UEs and which APNs are tagged for "application dependent low latency" service. For the SPR/UDR case, the PCRF needs to understand the subscriber data it receives from the UDR. However, this is in line with the specification in TS 29.335 [7] for the operation of the Ud interface between UDR and PCRF.

b)	For the option in procedure step 6b, a new PCRF-PCEF signalling indication is needed.

c)	The PGW-C needs to react to either the new PCRF-PCEF indication (above), or, to the activation of e.g. QCI 80; re-enquire to its DNS; and subsequently, potentially relocate the S/PGW-U.

d)	In order to return the UE's S/PGW-U to a central location, traffic detection functionality is needed at the 'low latency' S/PGW-U location. This could be a standalone TDF or implemented within the PCEF of the S/PGW-U.

e)	The PGW-C also needs to store the information that the S/PGW-U may be relocated back to a more central location, and utilize e.g. S/PGW-U load information to decide when to perform this movement back.

f)	There is no MME impact.

g)	The use of PCO in step 9a requires updates to EPC Session Management signaling and UE processing.

h)	The UE’s application needs to handle the change of IP address.

### 6.3.5	Evaluation

The solution is similar to the SSC mode 3 functionality in the 5G System (refer to TS 23.502 [5] clause 4.3.5.2, with the difference being that the logic for PGW-U relocation uses the PCRF and PGW-C.

The use of SPR/UDR permits PCRF relocation (if needed).

The solution supports roaming with Local BreakOut.

The solution is aware of the use (or non-use) of applications that require low latency. This avoids the PLMN needing to install ‘low latency’ GW capacity for all UEs that might, occasionally need low latency service.

There is no MME impact.

################### END OF PROPOSAL ########################