3GPP TSG-SA2 Meeting #127	S2-184562

# Sanya, China, 16th Apr 2018 - 20th Apr 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0333 | rev | 4 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UCU procedure and Access Type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1, Allowed NSSAI is now per-access information and the current procedure description didn’t indicate the associated Access Type for the Allowed NSSAI. 2, In current specs, it is not clear whether UCU procedure can be performed on either 3GPP access or Non-3GPP access, and whether the UCU Command sent on one access type can include parameters (e.g. Allowed NSSAI) related to another access type. 3, if there are simultaneous changes of Allowed NSSAI for both 3GPP and Non-3GPP access, there are different options: - Two separate UCU procedure are initiated over 3GPP access and Non-3GPP access respectively. - One single UCU procedure is initiated with two new Allowed NSSAIs included in the same UCU command. It is not clear in the specs which option is allowed. It is also not clear how the UE should perform Registration when two Allowed NSSAI changes are received in a single UCU Command. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, add “associated Access Type” for Allowed NSSAI and mapping of Allowed NSSAI in the procedure; 2, Clarify that UCU procedure can be performed on either access type and the UCU Command sent over one access can include the parameter changes related to another access type. 3, Clarify that the UE may receive two allowed NSSAIs, for 3GPP access and Non-3GPP access respectively, in one single UCU Command. In this case, the UE should initiate Registration over 3GPP access first, then perform another Registration over Non-3GPP access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If simultaneous Allowed NSSAI changes for two access types are sent over two separate UCU Command, the UE may not be able to relate them and perform two independent Registrations over each access (without 5G-GUTI) which may end up selecting two different new AMFs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** Start of Changes ****

### 4.2.4	UE Configuration Update

#### 4.2.4.1	General

UE configuration may be updated by the network at any time using UE Configuration Update procedure. UE configuration includes:

-	Access and Mobility Management related parameters decided and provided by the AMF.

-	UE Policy provided by the PCF.

When AMF wants to change the UE configuration for access and mobility management related parameters the AMF initiates the procedure defined in clause 4.2.4.2. When the PCF wants to change or provide new UE Policies in the UE, the PCF initiates the procedure defined in clause 4.2.4.3.

#### 4.2.4.2	UE Configuration Update procedure for access and mobility management related parameters

This procedure is initiated by the AMF when the AMF wants to update access and mobility management related parameters in the UE configuration.

The procedure applies only to information in the UE that does not require negotiation between the UE and the network.

This procedure is also used to trigger UE to perform, based on network indication, either Registration Update procedure while the UE is in CM-CONNECTED state to modify parameters that require negotiation (e.g. MICO mode) or Registration Update procedure after the UE enters CM-IDLE state (e.g. for changes to Allowed NSSAI that require re-registration).

If the AMF wants to update NAS parameters in the UE which require UE acknowledgement, then the AMF provides an indication to the UE of whether the UE shall acknowledge the command or not. The AMF should not request acknowledgement of the NITZ command. The AMF shall request acknowledgement for NSSAI information (e.g. Allowed NSSAI), 5G-GUTI, TAI List, and Mobility Restrictions.

Figure 4.2.4.2-1: UE Configuration Update procedure for access and mobility management related parameters

0. AMF determines the necessity of UE configuration change due to various reasons (e.g. UE mobility change, NW policy, reception of Subscriber Data Update Notification from UDM, change of Network Slice configuration) or that the UE needs to perform a Registration Procedure. If a UE is in CM-IDLE, the AMF triggers Network Triggered Service Request (in clause 4.2.3.4).

The AMF may include Handover Restriction List in N2 message that delivers UE Configuration Update command to the UE if the service area restriction for the UE is updated.

1.	The AMF sends UE Configuration Update command containing one or more UE parameters (5G-GUTI, TAI List, Allowed NSSAI and the associated Access Type, Mapping Of Allowed NSSAI and the associated Access Type, Configured NSSAI for the Serving PLMN, Mapping Of Configured NSSAI, NITZ, Mobility Restrictions, LADN Information, MICO, Configuration Update Indication) to UE.

The AMF includes one or more of 5G-GUTI, TAI List, Allowed NSSAI and the associated Access Type, Mapping Of Allowed NSSAI and the associated Access Type, Configured NSSAI for the Serving PLMN, Mapping Of Configured NSSAI, NITZ (Network Identity and Time Zone), Mobility Restrictions parameters or LADN Information if the AMF wants to update these NAS parameters without triggering a UE Registration procedure.

The AMF may include in the UE Configuration Update Command also a Configuration Update Indication parameter indicating whether:

-	the UE shall acknowledge the command and,

-	a Registration procedure is requested.

For the latter case, the AMF may additionally indicate that the UE whether the 5G-GUTI shall be used in access stratum signalling for the Registration procedure.

The UE Configuration Update Command may be sent over either 3GPP access or Non-3GPP access and the UE Configuration Update Command may contain the configuration changes (e.g. Allowed NSSAI) associated with other Access Type than the Access Type over which the UE Configuration Update Command is sent.

2.	If the UE Configuration Update Indication requires acknowledgement of the UE Configuration Update Command, then the UE shall send a UE Configuration Update complete message to the AMF. The AMF should request acknowledgement for all UE Configuration Updates, except for NITZ.

If the Configuration Update Indication requires a Registration procedure, depending on the parameters included in the UE Configuration Update command steps 3a or 3b-4 or 3c-3d-4 are executed.

3a.	If MICO is included in the UE Configuration Update Command message, UE shall initiate a Registration procedure immediately after the acknowledgement to re-negotiate MICO mode with the network. Steps 3b, 3c, 3d and step 4 are skipped.

3b. If a new Allowed NSSAI and/or a new Configured NSSAI provided by the AMF to the UE does not affect the existing connectivity to slices (i.e. any S-NSSAI(s) the UE is connected to), the AMF needs not release the NAS signalling connection for the UE after receiving the acknowledgement in step 2, and immediate -registration is not required. The UE cannot connect to an S-NSSAI included in the Configured NSSAI for the Serving PLMN but not included in the current Allowed NSSAI until the UE performs a Registration procedure and includes a Requested NSSAI based on the new Allowed NSSAI. Steps 3c and 3d are skipped.

3c.	If a new Allowed NSSAI and/or a new Configured NSSAI provided by the AMF to the UE affects ongoing existing connectivity to slices, then the AMF also includes in the UE Configuration Update Command message an Allowed NSSAI with the associated mapping of Allowed NSSAI, and an indication that the UE shall not provide the 5G-GUTI in Access Stratum signalling when performing a Registration procedure.

3d.	If an Allowed NSSAI with the associated mapping Of Allowed NSSAI and an indication that the UE shall not provide the current 5G-GUTI for the Registration procedure in the Access Stratum signalling are included in the UE Configuration Update Command message, the AMF shall release the NAS signalling connection of the associated Access Type for the UE after receiving the acknowledgement in step 2.

4.	UE initiates a Registration procedure after the UE enters CM-IDLE state and includes the 5G-GUTI in Access Stratum signalling depending on the indication received from the AMF.

If two Allowed NSSAIs are included in the UE Configuration Update Command for 3GPP and Non-3GPP access respectively, the UE initiates the Registration procedure over the 3GPP access first, and if the AMF is re-allocated as indicated by the new 5G-GUTI received during this Registration, the UE may need to perform another Registration (with the new 5G-GUTI received from the first Registraton) on the Non-3GPP access following the first Registration.

NOTE:	Receiving UE Configuration Update command without an indication requesting to perform re-registration, can still trigger Registration Update procedure by the UE for other reasons.

**** End of Changes ****