SA WG2 Meeting #128BIS	S2-187722

August 20 – 24 2018, Sophia Antipolis , France

Source:	China Mobile

Title:	UPF service introduced in SBA

Document for:	Discussion and Approval

Agenda Item:	6.19

Work Item / Release:	eSBA / Rel-16

Abstract of the contribution: This proposal is to introduce and define UPF services.

- Introduction

This proposal addresses key issue2 “Extend Service Concept into User Plane”

In release 15 SBA design, the N4 interface is realized in PtP interaction method. However the UPF is belonging to 5G core network, and the cloud native concept is more convenient to be extended to UPF. Therefore the N4 interface which supports the communication between 5G user plane and 5G control plane will be designed as SBI to achieve SBA’s flexibility and agility.

One consideration for introducing UPF services is based on the UPF’s functionality.

There are 13 main functionalities of UPF illustrated in 23.501, clause 6.2.3.

- Anchor point for Intra-/Inter-RAT mobility (when applicable).

- External PDU Session point of interconnect to Data Network.

- Packet routing & forwarding (e.g. support of Uplink classifier to route traffic flows to an instance of a data network, support of Branching point to support multi-homed PDU Session).

- Packet inspection (e.g. Application detection based on service data flow template and the optional PFDs received from the SMF in addition).

- User Plane part of policy rule enforcement, e.g. Gating, Redirection, Traffic steering).

- Lawful intercept (UP collection).

- Traffic usage reporting.

- QoS handling for user plane, e.g. UL/DL rate enforcement, Reflective QoS marking in DL.

- Uplink Traffic verification (SDF to QoS Flow mapping).

- Transport level packet marking in the uplink and downlink.

- Downlink packet buffering and downlink data notification triggering.

- Sending and forwarding of one or more "end marker" to the source NG-RAN node.

- ARP proxying as specified in IETF RFC 1027 [53] and / or IPv6 Neighbour Solicitation Proxying as specified in IETF RFC 4861 [54] functionality for the Ethernet PDUs. The UPF responds to the ARP and / or the IPv6 Neighbour Solicitation Request by providing the MAC address corresponding to the IP address sent in the request.

Because in eSBA, the N3 and N9 will not be designed as SBI, therefore the UPF’s functionality 12( i.e. the communication between UPF and RAN) and the UPF’s functionality 13( i.e UPF interaction with other user plane entities) will not be considered.

The other 11 functionalities are all depending on the communication between UPF and 5G control plane. The 11 functionalities of UPF can be categorized into 3 services, i.e. the Traffic Path Control Service (fulfilling UPF functionality 1,2,3), Policy Control Service(fulfilling UPF functionality 4,5,6,8,9,10) and Event Exposure Service(fulfilling UPF functionality 7,11).

- Traffic Path Control Service is the service which can be invoked by SMF to establish the traffic routing path for uplink/downlink data. For example, this UPF service can be used to update the tunnel info when the UPF implements the mobility anchor, or a UPF is inserted into/removed from the existing data path.

- Policy Control Service is the service which can be invoked by 5G control plane NF e.g. SMF to configure corresponding policy on UPF. For example, SMF can invoke the policy control service to transfer the QoS other policies to UPFs.

- Event Exposure Service is the service which can report the UPF information to the 5G control plane NF which has subscribed this event before. For example, the downlink data notification can be sent to corresponding SMF through the Event Exposure Service.

5G control plane NFs can communicate flexibly through invoking the three UPF services described above.

Another consideration for introducing the three UPF services is the independency among the 3 services.

The traffic data path establishment, update and release can be independent with policy data transmission from 5G control plane to UPF. There are some cases in 23.502 e.g. clause 4.3.5.4 that the tunnel information is changed in one UPF while the policy configuration in the UPF (e.g. the PSA1) for the PDU session is not changed. And the clause 4.3.3.2 shows the case that the policy configuration is changed while the traffic data path is not changed.

The independency between traffic data path and policy data transmission enables the introduction of the two UPF services i.e. Traffic Path Control Service and Policy Control Service. The Event Exposure Service is also independent from Traffic Path Control Service and Policy Control Service.

Once the UPF can be invoked through SBI, the interaction between UPF and 5GC will be more flexible. What’s more, some 5G control plane NFs besides SMF can invoke the UPF services directly.

- Conclusion and Proposal

This contribution proposes three UPF services to fulfilment UPF functionalities. 5G control plane NFs can invoke UPF services through SBI to realize the communication between UPF and 5G control plane NFs.

* * * First Change * * *

6.X	Solution X: UPF Services introduced in 5G eSBA

6.X.1	Introduction

This solution addresses key issue 2 “Extend Service Concept into User Plane”.
In Release 15 SBA design, the SBA applies to 5GC control plane only due to time constraints. This leads to both SBI and PtP interfaces exists in the core network, causing the protocol complexity. The user plane function (UPF) is the NF that possesses many functionalities. Therefore, extends the service concept into User Plane helps to modularize the functionalities so that the UPF can be flexible defined/deployed, e.g., per network slicing/edge computing requirement.

There are 13 main functionalities of UPF illustrated in 23.501, clause 6.2.3.

- Anchor point for Intra-/Inter-RAT mobility (when applicable).

- External PDU Session point of interconnect to Data Network.

- Packet routing & forwarding (e.g. support of Uplink classifier to route traffic flows to an instance of a data network, support of Branching point to support multi-homed PDU Session).

- Packet inspection (e.g. Application detection based on service data flow template and the optional PFDs received from the SMF in addition).

- User Plane part of policy rule enforcement, e.g. Gating, Redirection, Traffic steering).

- Lawful intercept (UP collection).

- Traffic usage reporting.

- QoS handling for user plane, e.g. UL/DL rate enforcement, Reflective QoS marking in DL.

- Uplink Traffic verification (SDF to QoS Flow mapping).

- Transport level packet marking in the uplink and downlink.

- Downlink packet buffering and downlink data notification triggering.

- Sending and forwarding of one or more "end marker" to the source NG-RAN node.

- ARP proxying as specified in IETF RFC 1027 [53] and / or IPv6 Neighbour Solicitation Proxying as specified in IETF RFC 4861 [54] functionality for the Ethernet PDUs. The UPF responds to the ARP and / or the IPv6 Neighbour Solicitation Request by providing the MAC address corresponding to the IP address sent in the request.

Some considerations to defines UPF services are the following:

1.	Based on the UPF’s functionality characteristics. There are 13 main functionalities of UPF illustrated in 23.501. Considering the N3 and N9 is not in the scope of eSBA, there are eleven functionalities of UPF can be categorized into three services, i.e. the Traffic Path Control Service (fulfilling UPF functionality 1,2,3), Policy Control Service(fulfilling UPF functionality 4,5,6,8,9,10) and Event Exposure Service(fulfilling UPF functionality 7,11).

2.	Considering the independency among the 3 services

1) The traffic data path establishment, update and release can be independent with policy data transmission from 5G control plane to UPF. In 23.502, the clause 4.3.5.4shows that the tunnel information is changed in one UPF while the policy configuration in the UPF (e.g. the PSA1) for the PDU session is not changed.

2) The independency between traffic data path and policy data transmission enables the introduction of the two UPF services i.e. Traffic Path Control Service and Policy Control Service. The Event Exposure Service is also independent from Traffic Path Control Service and Policy Control Service.

6.X.2	High-level Description

5G user plane NF services are defined as the following:

- Traffic Path Control is the service to establish the traffic routing path for uplink/downlink data. It can be used to update the tunnel info when the UPF implements the mobility anchor, or a UPF is inserted into/removed from the existing data path.

- Policy Control is the service to configure corresponding PCF policy and/or policy trigger information on UPF for proper data traffic handling. SMF can invoke the policy control service to transfer the QoS, Lawful Interception, Charging or other policies to UPFs, and after this configuration, the policy can be enforced when the corresponding uplink/downlink data starts to transfer.

- Event Exposure is the service which can report the UPF information to the consumers subscribed this event. For example, the downlink data notification can be sent to corresponding SMF through the Event Exposure Service.

6.X.3	Services and illustrated Procedures

Table 6.x.3-1: List of UPF Services

| Service Name | Service Operations | Operation Semantic | Known Consumer(s) | Reference |
| --- | --- | --- | --- | --- |
| Nupf_TrafficPathControl service | Create | Request/ Response | SMF | Step 10a and 10b of clause 4.3.2.2.1. Step 4a, 4b of clause 4.3.5.3 |
|  | Update | Request/ Response | SMF | Step 16a and 16b of clause 4.3.2.2.1 Step 5a ,5b and 6a,6b of clause 4.3.5.3 |
|  | Release | Request/ Response | SMF | Step 2a and 2b of clause 4.3.4.2 |
|  | N4AssociationSetup | Request/ Response | SMF | Steps of clause 4.4.3.1 |
|  | N4AssociationUpdate | Request/ Response | SMF | Steps of clause 4.4.3.2 SMF initiated N4 association update procedure |
|  | N4AssociationRelease | Request/ Response | SMF | Steps of clause 4.4.3.3 SMF initiated N4 association release procedure |
| Nupf_PolicyControl service | Create | Request/ Response | SMF, | Step 10a and 10b of clause 4.3.2.2.1 Step 3 and 4 of clause 4.4.4 |
|  | Update | Request/ Response | SMF | Step 11a, 11b of clause 4.3.3.2 |
|  | Release | Request/ Response | SMF | Steps of clause 4.4.1.4 |
| Nupf_EventExposure service | Notify | Subscribe/Notify | SMF, | Step 1 of clause 4.4.4 Step 2a of clause 4.2.3.3 |

Editor’s Note: Whether there are other NFs besides SMF can invoke the UPF services is FFS.

6.X.4	Impacts on existing Services and Interfaces

The SMF needs to be updated to be able to invoke the corresponding UPF services .The N4 PtP interface should be updated to support service based interface.

Editor’s Note: Whether the procedure like Step 10a and 10b of clause 4.3.2.2.1can be divided into two services invocation e.g.Nupf_PathControl_Create and Nupf_PolicyControl_Create is FFS.

6.X.5	Evaluation of the Solution

Editor's note:	This clause provides an evaluation of the solution.

* * * End of Change * * *