# SA WG2 Meeting #128bis	S2-188143

# 20 - 24 August, 2018, Sophia Antipolis, France	(revision of S2-18xxxx)

Source:	Apple France

Title:	23.793: Update of Solution 5

Document for:	Approval

Agenda Item:	6.8

Work Item / Release:	FS_ATSSS / Rel-16

Abstract of the contribution: This contribution proposes an update for Solution 5.

# 1.	Discussion

1. “On-path” MPTCP proxy vs. “off-path” MPTCP proxy:

We believe there is no dependency between the presence of the MA-PDU session and the type of the MPTCP proxy (“on-path” or “off-path”). These two aspects are independent. Even if the UE establishes a Multi-access PDU session and there is a single anchor UPF behaving as an MPTCP proxy, it should be possible for the UE to use an “off-path” (or perhaps a better term would be “addressable") MPTCP proxy. Then, the UE can choose when to use MPTCP end-to-end and when to use a proxy. The issue with the “on-path” (or “transparent”) proxy is that the UE has no control whether the network includes the proxy on the path or not. We prefer the UE to be aware if a proxy is on the path.

2. Support for the standalone MPTCP proxy:

The standalone “off-path” proxy (distinct from the UPF) allows further flexibility for MPTCP deployments and we recommend to specify it for Rel-16. We believe there should a simple protocol in place between the UPF and the MPTCP proxy to select the initial proxy and potentially re-select it, in case there is a change of the PDU Session Anchor (for SSC mode 2 / mode 3).

3. Measurement reports

We propose to refer to solution 1 for access measurements reports that can complement the UE’s steering decision in MPTCP.

4. MPTCP proxy protocol:

We propose to delete the reference to SOCKSv6 as its timeline in IETF is not aligned with Rel-16 and keep the transport converter draft as the main baseline with a potential backup to SOCKSv5/v4. The transport converter draft is already adopted by the TCPM WG in IETF and aims for an Experimental RFC status.

# 2.	Proposal

It is proposed to modify Solution 5 in TR 23.793 as follows:

***** First Change *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[3]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[4]	3GPP TS 23.139: "3GPP system - fixed broadband access network interworking".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[7]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[8]	draft-ietf-mptcp-rfc6824bis-10: "TCP Extensions for Multipath Operation with Multiple Addresses".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[9]	RFC 1928: "SOCKS Protocol Version 5".

[10]	Void

[11]	draft-ietf-tcpm-converters-01: "0-RTT TCP Convert Protocol".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

***** Second Change *****

## 6.5	Solution 5: Multipath TCP (MPTCP) proxy architecture

### 6.5.1	General

This solution addresses Key Issues #1 and #4.

Multipath TCP (MPTCP) defined in [8] is a set of extensions to regular TCP to provide a Multipath TCP service, which enables a transport connection to operate across multiple paths simultaneously.

MPTCP defines how to setup multiple paths ("subflows"), manage these subflows, reassemble of data and terminate sessions.

The simultaneous use of multiple disjoint (or partially disjoint) paths across the network increases the resilience of the connectivity by providing multiple paths, protects UEs from the failure of one path, and it also increases the efficiency of the resource usage, and thus increases the network capacity available to UEs.

### 6.5.2	MPTCP proxy

When communicating with traditional TCP servers, MPTCP capable devices have to fall back to traditional TCP and cannot fully utilize their MPTCP capability. In order to support the use of MPTCP session between an MPTCP host and a TCP host, or to help to aggregate MPTCP subflows, an MPTCP proxy needs to be introduced.

Figure 6.5.2-1: Illustration of an MPTCP proxy

The MPTCP proxy makes it possible to achieve the benefits of MPTCP for access aggregation and route selection without requiring MPTCP support in all TCP servers.

### 6.5.3	Addition of MPTCP proxy to 5GC

In order to introduce the MPTCP proxy to 5GC and allow 3GPP UEs to make use of the MPTCP capability, the UPF shall either incorporate the MPTCP proxy functionality or select a standalone MPTCP proxy for the MPTCP PDU session.

In case of the standalone MPTCP proxy, if there is a change of the UPF during the lifetime of the MPTCP PDU session (for SSC mode 2 / mode 3), the new UPF may select a new MPTCP proxy for the MPTCP PDU session.

Editor’s Note: Further details of the communication between the UPF and the MPTCP proxy are FFS.

Figure 6.5.3-1: Addition of MPTCP proxy to 5GC

The MPTCP proxy shall implement  one of the following protocol options to communicate with the UE:

1. SOCKS v4 or v5 Proxy [9];

- Advantages: Well-known, well-supported.

- Disadvantages: Additional connection-setup latency.

2. Transport converter [11];

- Advantage: Designed specifically for the MPTCP use case, provides 0-RTT protocol conversion service, no overhead.

- Disadvantage: Not yet standardized, not yet implemented

### 6.5.4	MPTCP PDU session establishment procedure

This solution supports the Multi-Access PDU session establishment defined in section 6.2. Each of the PDU sessions may have its own set of UPFs, but both child PDU sessions share a common PDU session anchor (PSA).

This procedure is based on the procedure defined in section 6.2.2.2 with the following exceptions:

1.	The UE requests the network to select an MPTCP proxy on the user plane;

2.	The MPTCP proxy address is transported back to the UE;

3.	The child PDU sessions do not share a common IP address but they are allocated separate IP addresses.

Figure 6.5.4-1: UE-requested MPTCP PDU Session Establishment for non-roaming and roaming with local breakout

Editor's note:	It is FFS whether instead of the above PDU session setup procedure the UE could setup two independent PDU sessions for MPTCP procedures as defined in clause 6.2.2.1. In this case it is also FFS how the UE receives the address of the MPTCP proxy.

1.	The UE initiates the "UE Requested PDU Session Establishment" procedure as specified in TS 23.502 [6] clause 4.3.2.2.1 on 3GPP access with an additional preference to establish an MPTCP PDU Session. The UE can also first initiate the PDU Session Establishment procedure on untrusted non-3GPP access.

2.	If the network supports MPTCP procedures, the AMF selects an SMF that supports MPTCP procedures.

3.	The AMF sends an Nsmf_PDUSession_CreateSMContext Request to SMF.

4.	The SMF registers with the UDM.

5.	The SMF sends an Nsmf_PDUSession_CreateSMContext Response to AMF.

6.	Optional Secondary authorization/authentication.

7a.	If dynamic PCC is deployed, the SMF performs PCF selection.

7b.	The SMF may perform a Session Management Policy Establishment.

8.	The SMF selects an UPF that incorporates an MPTCP proxy functionality. As an alternative, the UPF selects a standalone MPTCP proxy.

9.	SMF may perform a Session Management Policy Modification procedure.

10-17.	The user plane of the PDU session over 3GPP access is established and the UE creates the first MPTCP subflow over the 3GPP access. The first MPTCP subflow can also be created over the non-3GPP access once the PDU Session is established over the untrusted non-3GPP access.

-	In step 11, the SMF indicates to AMF the MPTCP proxy address along with the list of protocols the MPTCP proxy supports (SOCKSv4, v5 or Transport converter).

-	In step 13, the UE receives a PDU Session Establishment Accept message containing the MPTCP proxy address (if the network supports MPTCP procedures) along with the list of protocols the MPTCP proxy supports, thus, the UE determines that the MPTCP PDU session over 3GPP access is established and selects one protocol to contact the MPTCP proxy from the protocol options provided by the network.

19-27.	The user-plane of the PDU session over untrusted non-3GPP access is established and the UE creates an additional MPTCP subflow over the untrusted non-3GPP access.

NOTE:	The steps 10-17 and 19-27 can be executed in parallel.

### 6.5.5	ATSSS rules for MPTCP PDU Session

In Rel-15, the UE is responsible for establishing new PDU session(s) based on URSP rules provided by the network (PCF) and/or preconfigured in the UE as described in TS 23.503 [7] clause 6.6.2. As such, URSP rules provide SDF level steering capability for UE to establish new PDU session(s) over the preferred access network based on traffic descriptor and rule precedence.

The steering decision is performed by taking into account the congestion and delay information of the individual TCP subflows of the MPTCP connection. The UE may also take into account Access Measurement Reports, as defined in
subclause 6.1.5.3, to complement the steering and path-management decisions.

The ATSSS rules defined by this solution are extension of the URSP rules described in TS 23.503 [7] section 6.6.2. The following new fields are created:

-	The MPTCP PDU Session field in the Route Selection Descriptor specifies whether the type of the PDU session is an MPTCP PDU session to be established for the matching application;

-	In case of an MPTCP PDU session:

-	The Allowed MPTCP Service Type(s) field specifies the MPTCP Service Type(s) that are allowed for the MPTCP PDU session:

-	Handover MPTCP Service Type: An MPTCP service that provides seamless handover between non-3GPP and 3GPP access in order to preserve the connection. If the rule specifies Handover MPTCP Service Type, Interactive and Aggregate Service Types shall not be present;

-	Interactive MPTCP Service Type: A service whereby Multipath TCP attempts to use the lowest-latency interface. If the rule specifies Interactive MPTCP Service Type, Handover MPTCP Service Type may be present but Aggregate Service Type shall not be present; or

-	Aggregate MPTCP Service Type: An MPTCP service that aggregates the capacities of the access networks in an attempt to increase throughput and minimize latency. If the rule specifies Aggregate MPTCP Service Type, Handover and Interactive MPTCP Service Types may be present as well;

-	If present, in case of WLAN access, the Limited  SSID(s) for MPTCP PDU Session field specifies a list of SSID(s) that are allowed on the WLAN access for the MPTCP PDU Session;

The PCF may select the ATSSS rules applicable for each UE based on local configuration and operator policies taking into consideration e.g. UE location or time of the day.

The PCF may provide the ATSSS rules at the initial Registration procedure, or when the operator policies indicate that the conditions for updating the UE are met, i.e. at change of UE location or at mobility with change the AMF, or at any time, as defined in TS 23.502 [6]. Operator defined policies in the PCF may depend on input data such as UE location, time of day, information provided by other NFs, etc.

Table 6.5.5-1: UE Route Selection Policy (from TS 23.503 [7] table 6.6.2.1-1)

| Information name | Description | Category | PCF permitted to modify in a URSP | Scope |
| --- | --- | --- | --- | --- |
| URSP rules | 1 or more URSP rules as specified in table 6.6.2.1-2 | Mandatory | Yes | UE context |

Table 6.5.5-2: UE Route Selection Policy Rule (from TS 23.503 [7] table 6.6.2.1-2)

| Information name | Description | Category | PCF permitted to modify in a UE context | Scope |
| --- | --- | --- | --- | --- |
| Rule Precedence | Determines the order the URSP rule is enforced in the UE. | Mandatory (NOTE 1) | Yes | UE context |
| Traffic descriptor | This part defines the traffic descriptors for the policy |  |  |  |
| Application identifiers | Application identifier(s) | Optional |  |  |
| IP descriptors | IP 3 tuple(s) (destination IP address or IPv6 network prefix, destination port number, protocol ID of the protocol above IP) | Optional | Yes | UE context |
| Non-IP descriptors | Descriptor(s) for non-IP traffic | Optional | Yes | UE context |
| DNN |  | This is the DNN information provided by the application | Optional | Yes | UE context |
| List of Route Selection Descriptors | A list of Route Selection Descriptors. The components of a Route Selection Descriptor are described in table 6.6.2.1-3 | Mandatory |  |  |
| NOTE 1: Rules in a URSP shall have different precedence values. |  |  |  |  |

Table 6.5.5-3: : Route Selection Descriptor (from TS 23.503 [7] table 6.6.2.1-3)

| Information name | Description | Category | PCF permitted to modify in a UE context | Scope |
| --- | --- | --- | --- | --- |
| Route Selection Descriptor Precedence | Determines the order in which the Route Selection Descriptors are to be applied. | Mandatory (NOTE 1) | Yes | UE context |
| Route selection components | This part defines the route selection components | Mandatory (NOTE 2) |  |  |
| SSC Mode Selection | One single value of SSC mode. | Optional | Yes | UE context |
| Network Slice Selection | Either a single value or a list of values of S-NSSAI(s). | Optional | Yes | UE context |
| DNN Selection | Either a single value or a list of values of DNN(s). | Optional | Yes | UE context |
| PDU Session Type Selection | One single value PDU Session Type | Optional | Yes | UE context |
| Non-seamless Offload indication | Indicates if the traffic of the matching application is to be offloaded to non-3GPP access outside of a PDU Session. | Optional (NOTE 3) | Yes | UE context |
| Access Type preference | Indicates the preferred Access Type (3GPP or non-3GPP) when the UE establishes a PDU Session for the matching application. | Optional | Yes | UE context |
| MPTCP PDU Session | Indicates whether the type of the PDU session is MPTCP PDU session type or not. | Mandatory (NOTE 4) | No | UE context |
| Allowed MPTCP Service Type(s) | Either a single value or a list of values of Allowed MPTCP Service Type(s). Possible values: Handover, Interactive, Aggregation. | Optional (NOTE 5) | Yes | UE context |
| Limited SSID(s) for MPTCP PDU Session | If present, in case of WLAN access, it indicates a list of SSID(s) that are allowed for the MPTCP PDU Session Type. | Optional (NOTE 5) | Yes | UE context |
| NOTE 1: Every Route Selection Descriptor in the list shall have a different precedence value. NOTE 2: At least one of the route selection component shall be present. NOTE 3: If this indication is present in a Route Selection Descriptor, no other components shall be included in the Route Selection Descriptor. NOTE 4: For the MPTCP PDU Session, the Non-seamless Offload indication and the Access Type preference components shall not be included in the Route Selection Descriptor. NOTE 5: This component shall only be present for the MPTCP PDU Session. |  |  |  |  |

***** End of Changes *****