SA WG2 Meeting #128	S2-188416

Sophia Antipolis, France, August 20-24, 2018	(updated revision of S2-18xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	update of solution 6.3 on UE awareness of coexistence rules.

Document for:	Agreement

Agenda Item:	6.21

Work Item / Release:	FS_eNS / Rel-16

Abstract of the contribution: This contribution proposes a solution update to for the solution in clause 6.3

# 1.	Proposal

It is proposed the following solution update to the solution in clause 6.3 is added to TR 23.740

***********************PROPOSED TEXT********************************

### 6.3.3	Procedures

This solution impacts existing procedures by inclusion of the Mutual Exclusion Class Information in association of every S-NSSAI in the configured NSSAI when this is provided to the UE by the AMF in Registration Accept or in the UE Configuration Update. Other than this, there is no impact except behavioural (as described in clause 6.2.3) in the nodes which have to evaluate this information: e.g. the Mutual Exclusion Class information to e.g. generate the Allowed NSSAI from the Requested and subscribed NSSAI in the network, of the UE that needs to evaluate this information in the configured NSSAI to form a Requested NSSAI with compatible slices only.

Mutual Exclusion Class Information is also provided alongside subscriber information in the interaction between UDM and AMF, and between AMF and NSSF, for the determination of suitable allowed NSSAI which complies to Mutual exclusion constraints that are part of Subscriber data.

If a roaming partner network is known to not support rel-16, the HPLMN shall only provide subscription information to those PLMNs with S-NSSAIs that are mutually compatible.

A Rel-15 UE cannot use the Mutual Exclusion Class Information. If a subscription including this information is used for a Rel-15 UE, then the mutual exclusivity is not respected by this UE.

It is therefore expected that a baseline subscription of compatible slices that can be used in a rel-15 UE or a rel-15 network or UE is defined for the event a rel-15 UE is used or a rel-15 network is used by a roaming partner.

***********************END of PROPOSED TEXT***************************