SA WG2 Meeting #128bis	S2-188423

20 - 24 August 2018, Sophia Antipolis, France          (revision of S2-188265/6672/186618)

Source:	Huawei, HiSilicon, Ericsson, KDDI, Convida Wireless

Title:	Solution for Key Issue 7: NWDAF assisting Future Background Data Transfer

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA

Abstract of the contribution: This contribution proposes solution for Key Issue 7: NWDAF assisting Future Background Data Transfer.

1 Discussion

This solution is for Key Issue 7: NWDAF assisting Future Background Data Transfer.

1.1 Current Status in Release 15

At the moment, as specified in clause 4.16.7 Negotiations for future background data transfer in TS 23.502, H-PCF can provide one or more transfer policies for the future background data transfer based on requests from the 3rd party, before UE establishes the PDU session.

Figure 4.16.7.2-1: Negotiation for future background data transfer

1.2 Solution

Though the H-PCF could adopt the transfer policies stored in the UDR during the PDU Session Establishment, the network condition in the particular location may change after the UE establishes PDU session. Then, there are certain scenarios that the policy becomes unsuitable. Therefore, it is required that the PCF considers the latest network condition and the policy shall be updated based on some condition.

It is considered the network condition per UE as follows could directly impact the future background data transfer:

- UE Moving Trajectory;

- NW performance information of 5G NF which serves the UE.

It is obvious that NWDAF could derive the network condition related by collecting the UE level information as listed in Table 1.

Table 1:  NWDAF input data

| Information | Presence | Source | Description |
| --- | --- | --- | --- |
| UE ID | M | AMF | Could be e.g. SUPI, which is used by NWDAF to correlate the information from AMF/OAM. |
| Location info |  |  |  |
| >Timestamp | M | AMF | The timing for the UE |
| >Location Info | M | AMF | The location info for the UE e.g. Cell ID or TA ID |
| 5G NW performance information |  |  |  |

Given a UE ID list provided by PCF, with the UE level information per UE, the NWDAF could cluster the heterogeneous UEs into multiple UE group(s), where each UE group has the same network condition i.e.

- UE Moving Trajectory;

- NW performance information of 5G NF which serves the UE.

Based on these network condition per UE group from NWDAF, the H-PCF could more accurately provide/update the transfer policies for the future background data transfer.

2 Proposal

It is proposed solution for Key Issue 7: NWDAF assisting Future Background Data Transfer.

* * * * Start of Changes * * * *

6.X	Solution X: NWDAF assisting Future Background Data Transfer

6.X.1	Description

This solution is for Key Issue 7: NWDAF assisting Future Background Data Transfer.

At the moment, as specified in clause 4.16.7 Negotiations for future background data transfer in TS 23.502 [3], H-PCF can provide one or more transfer policies for the future background data transfer based on requests from the 3rd party, The transfer policy is generated by the H-PCF based on e.g. analytics information, and some other information listed in TS 23.503. The transfer policy is stored in UDR, at the time the ASP indicates that transfer of background data to the UE starts, to the PCF, the PCF retrieves the transfer policy and enforces it.

It is considered that the network condition in the UE location could directly impact the transfer policy for future background data transfer provided to the ASP in particular:

- UE Moving Trajectory;

- Network performance information such as NF available capacity of 5G NF which serves the UE.

It is obvious that NWDAF could derive the UE Moving Trajectory by collecting the UE level information such as location information from AMF, Network performance information is collected from OAM.

6.X.1.1 Information to support Future Background Data Transfer

The UE level information related to background data transfer per UE is defined in Table 6.X.1.1-1.

Table 6.X.1.1-1: NWDAF input data

| Information | Presence | Source | Description |
| --- | --- | --- | --- |
| UE ID | M | AMF | Could be e.g. SUPI, which is used by NWDAF to correlate the information from AMF/OAM. |
| Location info |  |  |  |
| >Timestamp | O | AMF | The timing for the UE |
| >Location Info | O | AMF | The location info for the UE e.g. Cell ID or TA ID |
| UE Mobility pattern | O | AMF | UE mobility pattern |
| Network performance info |  |  |  |

Editor´ note: Network performance information is required to be able to a BDT policy, we need to ask and cooperate with SA5 which NW performance indicators are available, e.g. NF available capacity.

6.X.1.2 Procedure for Future Background Data Transfer

Prerequisite: The PCF subscribes to or periodically request for a network condition list for an ASP or for specific UEs have a subscription with each ASP, where each network condition include the network performance analytics and Mobility Patterns(s) analytics corresponding to a UE group belongs to the ASP or for specific UEs have a subscription with each ASP.

Table 6.X.1.2-1. EventId to provide analytics to enhance BDT functionality

| Event ID | Event Filter | Description |
| --- | --- | --- |
| Network Condition list | List of UE identities, Time, Date, Network area | As defined in Table 6.X.1.2-2 |

Table 6.X.1.2-2. Content of Network Condition List

| IE/Group Name | Presence | Range | Semantics description |
| --- | --- | --- | --- |
| Network Condition list |  | 1 |  |
| > Network Condition IEs |  | 1 to M |  |
| >>list of UE IDs | M |  |  |
| >> Expected Moving Trajectory | M | 1 to N | UE´s expected geographical movement (e.g. as described in TS 23.502 [3] clause 14.5.6.3). |
| >>Network Performance Info |  |  | Performance information on the NF where the UE is expected to be according to the expected moving trajectory |

Figure 6.X.1.2-1: Procedure for future BDT enhanced with Analytics

The existing procedure listed in 4.16.7.2 in TS 23.502 applies, except

- In step1 AF request shall contain optionally UE IDs List, besides an ASP identifier, the volume of data to be transferred per UE, the expected amount of UEs, the desired time window and optionally, network area information (e.g. list of TAs/RAs). When the UE ID list is provided and the network area information is not provided, the NWDAF derives the expected location of the UE(s).

- Steps 5 that needs to be updated to explain that a Policy decision PCF takes analytics into account to generate a transfer policy. The analytics information is either subscribed to or periodically requested by the PCF for an ASP or for a number of UE and therefore have it available at the time the ASP requests for a transfer policy.

6.X.2	Impacts on Existing Nodes and Functionality

Editor's note:	Capture impacts on existing 3GPP nodes and functional elements.

6.X.3	Solution Evaluation

Editor's note:	Use this section for evaluation at solution level.

* * * End of Change * * *