SA WG2 Meeting #128-bis	S2-188453

August 20-24, 2018, Sophia Antipolis, France	(revision of S2-188152)

Source:	NICT

Title:	FS_Vertical_LAN: Update to Key Issue #1: Network discovery, selection and access control for type-a and type-b networks

Document for:	Approval

Agenda Item:	6.15: 5GS Enhanced support of Vertical and LAN Services

Work Item / Release:	FS_Vertical_LAN/Rel-16

Abstract of the contribution:

This document proposes an update to Key Issue #1: Network discovery, selection and access control for type-a and type-b networks.

# Discussion

In the current description of Key Issue #1 it looks like an UE is provided with an identifier of type-a or type-b network and then network selection is based solely on this information and/or some preconfigured information in the UE.

While it could be sufficient for type-b networks, some additional information could be beneficial for network selection in case of type-a networks.

An example of such additional information is mentioned in merged potential service requirements Nsq.cmd.1, Nsq.cmd.2, Nsq.cmd.5 in TS 22.804, which are part of 8.2.3 Service QoS monitoring, and reporting requirements:

The 5G system shall provide a mechanism for supporting real time E2E QoS monitoring within a system.

The 5G system shall be able to provide real time QoS parameters and events to an authorised application / network entity.

The 5G system shall provide information on the current availability of a specific communication service in a particular area (e.g. cell id) upon request of an authorised user.

It is proposed to update Key Issue #1: Network discovery, selection and access control for type-a and type-b networks to study two additional aspects:

- Whether there is a need to provide additional information about type-a network (in addition to network identifier of a type-a network) for the purpose of automatic or manual network selection

- If yes, how such information could be provided.

# Proposal

* * * * Start of Change * * * *

## 5.1	Key Issue #1: Network discovery, selection and access control for type-a and type-b networks

### 5.1.1	General description

This key issue aims at studying network discovery, selection and access control for type-a and type-b networks. Solutions to this key issue are expected to address the following aspects:

-	Type-a and type-b network subscriptions

-	How is information identifying a type-a or type-b network provided to the UE for network discovery and selection?

-	Whether and how additional information about type-a network is provided to the UE for network selection?

-	Which criteria are used by the UE for automatic selection of type-a or type-b networks

-	How to support manual selection of type-a and type-b networks

-	How to prevent UEs not authorized for a given type-a or type-b network from attempting to automatically select and register in that type-a or type-b network?

How to enable the network to verify whether a UE is authorized to access a type-a or type-b network?

-	Which network entities perform access control for type-a and type-b networks?

-	Access barring aspects for type-a and type-b networks

-	Where access restrictions are configured (e.g. subscription or configuration)?

-	How to enable UEs to access type-b networks but prevent the same UEs from accessing public PLMNs?

-	How to prevent UEs not supporting type-a and type-b networks from attempting to access type-a and type-b networks?

-	How to prevent NG-RAN from handing over a UE to a type-a network if the UE is not permitted to access the type-a network?

* * * End of Change * * * *