SA WG2 Meeting #128bis	S2-188476

20 - 24 August 2018, Sophia Antipolis, France	(revision of S2-188029)

Source:	China Mobile, Alibaba, Tencent, China Unicom, Huawei, HiSilicon, ZTE, CAICT, NEC

Title:	Use case NWDA-Assisted slice SLA guarantee and related Key Issue update

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA

Abstract of the contribution: This contribution proposes a new use case ”NWDA-Assisted slice SLA guarantee” and update Key Issue 4 to assist the guarantee of slice level quality.

1 Discussion

According to the joint discussion paper mentioned in S2-186666/S2-186972 last SA2 meeting, S2-187263 is approved to update Key Issue 4 to make NWDAF data analytics available to OAM, allowing that SA2 cooperate with OAM to accurately adjust slice resource for slice SLA guarantee.

However, the S2-187263 did not update key issue 4 completely and missed the use case and key issue part about NWDAF data analytics for guaranteeing slice level quality based on the discussion paper S2-188028 “Discussion about network resource scheduling for operating slices”.

This contribution proposes to add the missing use case as mentioned in S2-186666/S2-186972 and the missing key issue part about NWDAF-assisted the guarantee of slice level quality.

* * * * Start of Change * * * *

5.1.14	Use Case x: <How to ensure that slice SLA is guaranteed>

5.1.14.1	Description

Operators can provide a network slice for slice customers to meet their slices SLA. Considering the limited network resource(e.g., RAN/Transport/ CN resource) and a changing condition(e.g., Signal to Noise Ratio, more and more slices competing shared network resource), there is a risk for operators to guarantee the slice SLA at any time, e.g., how many percent of UEs’ Service Experience (i.e., UE QoE or UE Service MoS ) is satisfied in a slice.

It is also not clear for operators how to decide whether slice SLA can be satisfied, and if not, what SLA can probably be promised to be met for each slice based on the overall consideration of meeting slice SLA of many slices. For example, as follows:

-	When the SLA requirement(s) of some existing slice(s) changes, whether operators can meet the new SLA requirements of the existing slices, if not, what the slice SLA can be supported for operators.

-	After several slices are already created and maintained, whether operators can meet the SLA of the existing slices and the new slice when receiving a new slice instantiation request from a slice customer. If not, what the slice SLA can be supported for operators.

The case that different group of UEs in one slice may access the slice in different place and are faced with different network situation (e.g., possibly different transport path from RAN to TN to CN) can be considered.

The use case describes scenarios in which analytics information that are used to assist the guarantee of slice level SLA, which includes:

-	How/what types of data analytics information that could be provided to OAM  to assist the guarantee of slice SLA. Whether there is any need to provide analytics information to NFs-

-  How/what types of network information that are neededfor analysis from OAM or from NFs.

The analytics result can be used for operators to assist the adjustment of network resource scheduling to maximize the slices SLA from several aspects, for example, as follows:

-	Whether the SLA of a slice can be satisfied?

-  What distribution of UEs’ Service Experience per slice (e.g., how many percent of UE’s Service MoS is between 3 and 4 and how many percent of UE’s Service MoS is between 4 and 5) can be met?

5.2.14	Key Issue 14: How to ensure that slice SLA is fguaranteed

5.2.14.1	Description

Based on use case #14,

, following issues need to be studied:

-	Which  network slice level KPIs defined in OAM (e.g., TS 28.554) can be used for analysis to guarantee slice level SLA?

-	Whether OAM may need any information from NF or NWDAF  the guarantee of slice level SLA?

-  What and how analytic data is provided to OAM or NFsfor the purpose of guaranteeing slice level quality?

The fllowing are examples of how analytics result can be used:

-	Whether slice(s) SLA of a slice can be satisfied?

-  What distribution of UEs’ Service Experience per slice (e.g., how many percent of UE’s Service MoS is between 3 and 4 and how many percent of UE’s Service MoS is between 4 and 5) can be met?

NOTE:  The feature supporting this use case should be defined in Rel16. And it requires SA5 to start the related work , and more coordination with SA5. The solution investigation in SA2 is suggested to wait for the response of SA5 on it.

5.2.14.2	Requirements

Analytic information should be provided to help OAM and/or NFs to derive proper slice level KPI, and proper RAN/CN performance information or configuration or distribution of UEs’ Service Experience per slice to assist the guarantee of slice level quality. Solutions should investigate:

- which analytics information can be provided by to help to fulfill SLA.

- Which input data does OAM needs from 5G NFs as input as input for analytcs.

* * * End of Change * * *