3GPP TSG-SA WG2 Meeting #128bis			S2-188634

Sophia Antipolis, France, 20 - 24 August, 2018

Title:	[DRAFT] Reply LS on 5G service request procedure

Response to:	Reply LS on 5G service request procedure (S2-187632 / C1-182803)

Release:	Release 15

Work Item:	5GS_Ph1

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Myungjune Youn

E-mail Address:	m.youn@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-18XXXX, S2-18XXXX

1. Overall Description:

SA2 thanks CT1 for their LS S2-187632 / C1-182803 on 5G service request procedure.

SA2 has discussed the AMF behaviour when there is pending downlink signalling for non-3GPP PDU Session and UE is CM-IDLE in non-3GPP access. SA2 agreed to enable the delivery of pending downlink signalling to the UE independently of whether the corresponding PDU session can be moved to the 3GPP access, and has updated Service Request procedure considering the following cases:

- In case of downlink NAS SM signalling only:

The AMF sends the received NAS SM signalling to the UE independently of whether the corresponding PDU session can be moved to the 3GPP access and without moving the PDU Session to the 3GPP access.

- In case of downlink data only:

If the corresponding PDU session can be moved to the 3GPP access, the AMF discard the received N2 SM information and notifies the SMF that the PDU Session can be moved to 3GPP access. The SMF sends N2 SM information to the AMF. The UE, AMF and SMF update associated access of the PDU Session when user plane is reactivated over 3GPP access.

- In case of NAS signalling and downlink data:

If the corresponding PDU session can be moved to the 3GPP access, the AMF discard the received NAS signalling and N2 SM information, and notifies the SMF that the PDU Session can be moved to 3GPP access. The SMF sends NAS signalling and N2 SM information to the AMF. The AMF sends NAS signalling after user plane is reactivated over 3GPP access. The UE, AMF and SMF update associated access of the PDU Session when user plane is reactivated over 3GPP access.

The attached CRs captures above agreement.

2. Actions:

To CT1 group.

ACTION: 	SA2 kindly asks CT1 take this information into account for their further work.

3. Date of Next TSG-SA2 Meetings:

TSG-SA WG2 Meeting #129	15 – 19 October 2018		Dongguan, China

TSG-SA WG2 Meeting #129BIS	26 – 30 November 2018	West Palm Beach, Florida, USA