3GPP TSG-SA WG2 Meeting #128bis	S2-188906

20 - 24, August, 2018, Sophia Antipolis, France

Source:	CATT, Huawei, OPPO

Title:	Solution for KI3: Session continuity enhancement based on the coordination between AF and 5GC

Document for:	Approval

Agenda Item:	6.20

Work Item / Release:	FS_5G_URLLC / Rel-16

Abstract of the contribution: This paper would like to propose a solution for to Key Issue 3: Enhancement of Session Continuity during UE Mobility.

1.	Discussion

This document proposes to a new solution to address the Key Issue 3: Enhancement of Session Continuity during UE Mobility identified in the TR 23.725.

* * * Start of Changes * * *

6.x	Solution for Key Issue #3: Session continuity enhancement based on the coordination between AF and 5GC

6.x.1	General Description

This solution addresses the problem in the key issue 3 regarding “How to enable runtime coordination between AF and 5GC for supporting application relocation without breaking upper layer session and service continuity.”

In order to enable low latency service in the 5GC, a UE may establish a SSC mode 3 PDU session, or a PDU session with UL CL or branch point. In this case, if the PDU sessions anchor (PSA) needs to be relocated due to UE mobility, the SMF needs to select a new PSA.

In order to avoid or minimize the service interruption during the PSA (PDU session anchor) relocation, the solution proposes that, after the SMF prepares the data forwarding path towards the new PSA, the SMF doesn’t activate the data forwarding path to new PSA immediately, instead, it sends a notification to the application function. The notification includes UP management events as described in the TS 23.502, such as “PSA change” or “DNAI change”. The AF receiving the notification can determine whether the service relocation is required. If not, the AF can send a response back immediately; otherwise, the AF sends the response after the service relocation is completed. Once the 5GC receives AF response indicating application layer is ready, the 5GC activates the new data forwarding path towards the new PSA.

6.x.2	Procedures

Figure 6.x.2-1 illustrates the procedure of using the coordination between AF and 5GC to enhance the session continuity.

Figure 6.x.2-1 Session continuity enhancement based on the coordination between AF and 5GC

1-2. The SMF determines to select a new target UPF as the new PSA based on the UE location or DNAI, e.g. after the handover.

3.	In the case of early notification, the SMF send a notification indicating PSA change or DNAI change to the AF. The AF determines whether the application layer service needs to be relocated based on the info of new PSA or DNAI, if not, the AF can send back a response immediately, and then step 8 is not executed. If the AF send a negative response, the SMF may decide to cancel the following PSA relocation.

Editor’s Note:	The AF behaviours on handling service relocation are out of scope of this study, but any impact due to possible AF relocation is FFS.

4-5. Based on the new selected PSA, if UL CL or BP is used, the SMF may also determine to select a new UPF to act as UL CL or BP, and then selects a new UL CL or BP further. The SMF establishes CN tunnel between the new UL CL or BP and the new PSA if UL CL or BP is used,.

6.	If BP is used, the SMF also sends CN tunnel info of the new BP to the RAN node to establish the CN tunnel between the RAN and the BP.

7. 	In the case of late notification, the SMF send notification indicating PSA change or DNAI change to the AF. The AF determines whether service relocation is required based on the info of new PSA or DNAI, if not, the AF can send back a response immediately and step 8 is not executed.

8.	(Optional) After the completion of the service relocation, the AF responds the SMF with an ACK which includes N6 traffic routing information.

9.	Once the SMF receives the response in step 8 from the AF, the SMF updates the new PSA using the receiving N6 traffic routing information, and activates the new data forwarding path towards the PSA2.

10.	If UL CL is used in the PDU session, the SMF activates the new data forwarding path by sending N3 CN tunnel info of new UL CL to the RAN node.

11. If BP is used in the PDU session, the SMF activates the new data forwarding path by providing new allocated IPv6 prefix to the UE.

12-13. The SMF release the old data forwarding path towards the old PSA.

6.x.2	Impacts on Existing Nodes and Functionality

Editor's note:	Capture impacts on existing 3GPP nodes and functional elements.

6.x.3	Solution Evaluation

Editor's note:	Use this section for evaluation at solution level.

* * * Start of Changes * * *