3GPP TSG-SA WG2 Meeting #128bis	S2-189018

Sophia Antipolis, France, 20 - 24 August, 2018	(revision of S2-188663)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0694 | rev | 3 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Default IPsec tunnel for a PDU Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-08-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In 3GPP access, the RAN performs mapping between radio bearers and QoS Flows. The mapping information is sent to the UE via RRC signaling but the RAN may not provide mapping of all QoS Flows. When the UE needs to send a QoS Flow but does not have a mapping information, the UE uses default radio bearer. For example, assume that the SMF established two non-GBR QoS Flows (A,B) and one GBR QoS Flow (C). If the RAN decided to use default radio bearer for two non-GBR QoS Flows (A,B) and use separate radio bearer for GBR QoS Flow (C), the RAN only provides mapping information of GBR QoS Flow (C). This enables reduce signalig between UE and RAN. This CR proposes to use the same mechansim for non-3GPP access to reduce signaling beteen UE and N3IWF. For example, the N3IWF may use single Child SA for a PDU Session. In this case, all QoS flows are mapped to the same Child SA. When the SMF creates new QoS flow, the N3IWF need to update QoS Flow to Child SA mapping information. If the defaut Child SA is established, the N3IWF need not send signaling because the UE uses default Child SA if the UE does not have corresponding mapping information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The N3IWF may include default Child SA inidcation during the establishment of Child SA. The default Child SA is used to send QoS Flows does not have mapping information to a specific Child SA. Clarify that the N3IWF may update QoS Flow to Child SA mapping information during the PDU Session Modification procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE and N3IWF need to exchange IKE signaling to update IPsec SA – QoS Flow mapping information whenever new QoS Flow is established. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.12.5, 4.12.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.12.5	UE Requested PDU Session Establishment via Untrusted non-3GPP Access

Clause 4.12.5 specifies how a UE can establish a PDU Session via an untrusted non-3GPP access network as well as to hand over an existing PDU Session between 3GPP access and non-3GPP access. The procedure applies in non-roaming, roaming with LBO as well as in home-routed roaming scenarios.

For non-roaming and LBO scenarios, if the UE is simultaneously registered to a 3GPP access in a PLMN different from the PLMN of the N3IWF, the functional entities in the following procedures are located in the PLMN of the N3IWF. For home-routed roaming scenarios, the AMF, V-SMF and associated UPF in VPLMN in the following procedure is located in the PLMN of the N3IWF.

The procedure below is based on the PDU Session Establishment procedure specified in clause 4.3.2.2.1 (for non-roaming and roaming with LBO) and the PDU Session Establishment procedure specified in clause 4.3.2.2.2 (for home-routed roaming).

Figure 4.12.5-1: PDU Session establishment via untrusted non-3GPP access

1.	The UE shall send a PDU Session Establishment Request message to AMF as specified in step 1 of clause 4.3.2.2.1. This message shall be sent to N3IWF via the IPsec SA for NAS signalling (established as specified in clause 4.12.2) and the N3IWF shall transparently forward it to AMF in the 5GC.

2a.	In case of non-roaming or roaming with Local Breakout, steps 2-11 specified in clause 4.3.2.2.1are executed according to the PDU Session Establishment procedure over 3GPP access. In case of home-routed roaming, steps 2-14 specified in clause 4.3.2.2.2 are executed according to the PDU Session Establishment procedure over 3GPP access.

2b.	As described in step 12 of clause 4.3.2.2.1, the AMF shall send a N2 PDU Session Request message to N3IWF to establish the access resources for this PDU Session.

3.	Based on its own policies and configuration, and based on the QoS profiles received in the previous step, the N3IWF shall determine the number of IPsec child SAs to establish and the QoS profiles associated with each IPsec child SA. For example, the N3IWF may decide to establish one IPsec child SA and associate all QoS profiles with this IPsec child SA. In this case, all QoS Flows of the PDU Session would be transferred over one IPsec child SA.

4a.	The N3IWF shall send to UE an IKE Create_Child_SA request according to the IKEv2 specification in RFC 7296 [3] to establish the first IPsec child SA for the PDU Session. This request shall include a 3GPP-specific Notify payload which contains (a) the QFI(s) associated with the child SA and (b) the identity of the PDU Session associated with this child SA and (c) optionally a DSCP value associated with the child SA and (d) optionally a default Child SA indication. If a DSCP value is included, then the UE and the N3IWF shall mark all IP packets sent over this child SA with this DSCP value. There shall be one and only one default Child SA per PDU session. The UE shall send all QoS flows to this Child SA for which there is no mapping information to a specific Child SA. The IKE Create_Child_SA request also contains other information (according to RFC 7296 [3]) such as the SA payload, the Traffic Selectors (TS) for the N3IWF and the UE, etc. The IKE Create_Child_SA request indicates that the requested IPsec child SA shall operate in transport mode.

4b.	If the UE accepts the new IPsec child SA, the UE shall send an IKE Create_Child_SA response according to the IKEv2 specification in RFC 7296 [3]. During the IPsec child SA establishment the UE shall not be assigned an IP address.

4c-4d.	If in step 3 the N3IWF determined to establish multiple IPsec child SAs for the PDU Session, then additional IPsec child SAs shall be established, each one associated with one or more QoS profiles.

5.	After all IPsec child SAs are established, the N3IWF shall forward to UE via the IPsec SA for NAS signalling the PDU Session Establishment Accept message received in step 2b.

6.	The N3IWF shall send to AMF an N2 PDU Session Request Ack.

7.	In case of non-roaming or roaming with Local Breakout, all steps specified in clause 4.3.2.2.1 after step 14 are executed according to the PDU Session Establishment procedure over 3GPP access. In case of home-routed roaming, all steps specified in clause 4.3.2.2.2 after step 18 are executed according to the PDU Session Establishment procedure over 3GPP access.

8.	On the user-plane:

-	When the UE has to transmit an UL PDU, the UE shall determine the QFI associated with the UL PDU (by using the QoS rules of the PDU Session), it shall encapsulate the UL PDU inside a GRE packet and shall forward the GRE packet to N3IWF via the IPsec child SA associated with this QFI. The header of the GRE packet carries the QFI associated with the UL PDU.

-	When the N3IWF receives a DL PDU via N3, the N3IWF uses the QFI and the identity of the PDU Session in order to determine the IPsec child SA to use for sending the DL PDU over NWu. The N3IWK encapsulates the DL PDU inside a GRE packet and copies the QFI in the header of the GRE packet. The N3IWF may include also in the GRE header a Reflective QoS Indicator (RQI), which shall be used by the UE to enable reflective QoS.

* * * * Start of 2nd Change * * * *

4.12.6	UE or Network Requested PDU Session Modification via Untrusted non-3GPP access

The UE or network requested PDU Session Modification procedure via untrusted non-3GPP access is depicted in figure 4.12.6-1. The procedure applies in non-roaming, roaming with LBO as well as in home-routed roaming scenarios.

For non-roaming and LBO scenarios, the functional entities in the following procedures are located in the PLMN of the N3IWF.

The procedure below is based on the PDU Session Modification procedure specified in clause 4.3.3.2 (for non-roaming and roaming with LBO) and on the PDU Session Modification procedure specified in clause 4.3.3.3 (for home-routed roaming).

Figure 4.12.6-1: UE or Network Requested PDU Session Modification via untrusted non-3GPP access

1.	If the PDU Session Modification procedure is initiated by the UE, the UE shall send a PDU Session Modification Request message to AMF as specified in step 1 of clause 4.3.2.2. The message shall be sent to N3IWF via the established IPSec SA for NAS signalling. The N3IWF shall transparently forward the PDU Session Modification Request to AMF/SMF.

2.	In case of non-roaming or LBO, the steps 1a (from AMF) to 1e and steps 2-3 as per the PDU Session Modification procedure in clause 4.3.3.2 are executed.

In case of home-routed, the steps 1a (from AMF) to 1d and steps 2-3 as per the PDU Session Modification procedure in clause 4.3.3.3 are executed.

3.	The AMF sends N2 PDU Session Request (N2 SM information received from SMF, NAS message) message to theN3IWF. This step is the same as step 4 in clause 4.3.3.2 (for non-roaming and roaming with Local Breakout) and step 5 in clause 4.3.3.3 (for home-routed roaming).

4.	The N3IWF may issue IKEv2 signalling exchange with the UE that is related with the information received from SMF according to the IKEv2 specification in RFC 7296 [3]. Based on the N2 SM information received from the SMF, the N3IWF may perform following:

-	4a.	[Conditional] The N3IWF may decide to create a new Child SA for the new QoS flow(s). In this case, the N3IWF establishes a new Child SA by sending an IKE_CREATE_CHILD_SA request message, which includes the SA, the PDU Session ID and the QFI(s).

-	4b. [Conditional] The N3IWF may decide to add or remove QoS flow(s) to/from an existing Child SA. In this case, the N3WIF updates the QoS Flow and Child SA mapping information by sending an INFORMATIONAL request message, which includes the QFI(s) associated with the Child SA.

-	4c. [Conditional] The N3IWF may decide to delecte an existing Child SA, e.g. when there is no QoS flow mapped to this Child SA. In this case, the N3IWF deletes the existing Child SA by sending INFORMATION request message, which includes a Delete payload.

NOTE:	If the N3IWF has included the default Child SA indication during the establishment of one of the Child SAs of the PDU Session, the N3IWF may not update the mapping between QoS flows Child SAs.

5.	The N3IWF acknowledges N2 PDU Session Request by sending a N2 PDU Session Response Message to the AMF to acknowledge the success or failure of the request.

6.	In case of non-roaming or LBO, step 7 as per the PDU Session Modification procedure in clause 4.3.3.2 is executed. In case of home-routed, the steps 8-10 as per the PDU Session Modification procedure in clause 4.3.3.3 are executed.

7.	The N3IWF sends the PDU Session Modification Command to UE (if received in step 3) and receives the response message from UE.

Steps 4a/4c and step 7 may happen consecutively. Steps 7b map happen before step 4b/4d.

8.	The N3IWF forwards the NAS message to the AMF.

9.	For non-roaming and roaming with LBO, all the steps after step 10 in clause 4.3.3.2 are executed according to the general PDU Session Modification procedure. For home-routed roaming, all steps after step 13 in clause 4.3.3.3 are executed according to the general PDU Session Modification procedure.

* * * * End of Changes * * * *