3GPP TSG-SA WG2 Meeting #128	S2-186719

Vilnius, Lithuania, 02 July - 06 July, 2018	(revision of S2-18xxxx)

Source:	LG Electronics

Title:	Update Solution 8 - resolving editor's notes

Document for:	Approval

Agenda Item:	6.8 Study on Access Traffic Steering, Switch and Splitting support in the 5G system architecture

Work Item / Release:	FS_ATSSS / Rel-16

Abstract of the contribution: This contribution is proposed to resolve some editor's notes in solution 8 (ATSSS rules for supporting 3GPP RAT differentiation).

Discussion

This paper proposes to resolve following editor's notes in Solution 8: ATSSS rules for supporting 3GPP RAT differentiation.

1. Editor's note on dual connectivity

Editor's note:	It is FFS whether UE applies or ignores ATSSS rule when Access Technology is configured to specific 3GPP RAT (e.g. NR, E-UTRA) and the UE is using more than two access technologies at the same time (e.g. Dual Connectivity).

In downlink direction, the PCF may generate PCC rules based on access type and RAT type if applicable. Because the RAN does not report whether a UE is using dual connectivity, the PCF does not have any knowledge about dual connectivity and generates PCC rules without considering dual connectivity. Considering that, the SMF generates ATSSS rules based on the PCC rules, ATSSS rule should be applied regardless of whether dual connectivity is applied or not. So we propose that the UE applies ATSSS rule based on RAT type of Master RAN node.

2. Editor's note on unlicensed spectrum

Editor's note:	It is FFS whether NR/E-UTRA in unlicensed spectrum is needed.

Considering the characteristics of unlicensed spectrum, NR and E-UTRA over unlicensed spectrum may have different data rate and/or coverage compared to NR and E-UTRA over licensed spectrum. Therefore, operators may have preferred spectrum for specific services and may use different charging policy. Therefore, we propose to differentiate RATs over unlicensed spectrum and removes corresponding editor's note.

Proposal

* * * * Start of 1st Change * * * *

6.8	Solution 8: ATSSS rules for supporting 3GPP RAT differentiation

6.8.1	General

This solution applies to Key Issue #6: Key issue on differentiation of 3GPP RATs in ATSSS rule.

An ATSSS Rule contains the following parameters:

-	ATSSS Rule Identifier.

-	ATSSS Rule Priority.

-	Packet Filter Set.

-	Access Technology.

Editor's note:	Any other parameters are FFS.

The ATSSS Rule Identifier uniquely identifies an ATSSS Rule in the UE.

For the purpose of matching data flows against ATSSS Rules, the Packet Filter Set is applied in the order of the ATSSS Rule Priority.

There are two types of Packet Filter Set, i.e. IP Packet Filter Set, and Ethernet Packet Filter Set, corresponding to those PDU Session Types. The Packet Filter Set may contain multiple packet filters. The Packet Filter Set is defined in TS 23.501 [5] clause 5.7.6.

The Access Technology indicates a prioritized access technology for steering data flows, and possible values for the Access Technology are described in Table 6.8-1. If a 3GPP access technology corresponding to the value configured for the Access Technology is not selected by the UE then the UE shall ignore this configuration until the 3GPP access technology corresponding to the value configured for the Access Technology becomes selected.

NOTE	: When a UE is using more than two access technologies at the same time (e.g. Dual Connectivity), the UE uses Master RAN node's RAT to find matching ATSSS rule.

Table 6.8-1: Possible values for the Access Technology

| Value | Description |
| --- | --- |
| 0 | Reserved |
| 1 | Any 3GPP access technology e.g. NR, E-UTRA |
| 2 | WLAN |
| 3 | NR |
| 4 | E-UTRA |
| 5 | NR in Unlicensed |
| 6 | E-UTRA in Unlicensed |
| 7-255 | Reserved |

The SMF sends ATSSS Rule to the UE via NAS signalling. When the SMF sends ATSSS Rule, the SMF includes a default ATSSS Rule, which has a match all Packet Filter Set with the highest ATSSS Rule Priority value. The UE uses the received ATSSS Rule for uplink data flow steering. The UE evaluates UL packets against the Packet Filter Set in the ATSSS rule based on the ATSSS Rule Priority in increasing order until a matching ATSSS rule is found. If there is matching ATSSS rule the UE sends uplink data flow to the target access (i.e. access using the matched Access Technology). If there is no matching ATSSS rule, the UE follows default ATSSS rule.

NOTE:	The UE does not use ATSSS Rule to determine which access or RAT to connect to.

* * * End of Changes * * * *