SA WG2 Meeting #130 	S2-1900286

21 - 25 January 2019, Kochi, India	 (revision of S2-190xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	update of interim conclusions

Document for:	Approval

Agenda Item:	6.28

Work Item / Release:	FS_RACS/Rel-16

Abstract of the contribution:

# Discussion

This paper updates the interim conclusions.

It clarifies that >1 UE capability Id can be assigned by the PLMN to a UE.

It clarifies that this UE capability ID can be related to a UE capability Filter, and if so this relationship must be stored in the system (UE and network entities). If this is not done the UE and the network cannot understand whether a different UE capability ID or fresh capabilities needs to be signalled/retrieved when the filter changes.

Consequently, it is clarified that the association to the filter must be shared among nodes in the system when the mapping of a UE capability ID to a set of UE radio capabilities is provided.

# Proposal

It is proposed to update TS 23.743 as follows.

FIRST CHANGE

# 8	Interim Conclusions

Editor's note:	This clause will capture conclusions from the study.

Editor's note:	These are interim conclusions and will be reviewed after response from RAN WGs.

For key issue #1 (How UE radio capabilities are identified?) the following principles are agreed:

-	The UE capability ID is a short pointer (few octets, the exact size is to be determined by RAN WG2) that is used to uniquely identify a set of UE Radio Capabilities;

-	The UE capability ID is assigned either by the serving PLMN or by the UE manufacturer, as follows:

-	Manufacturer-specific: The UE Capability ID may be assigned by the UE manufacturer in which case it is accompanied with the UE manufacturer information (e.g.TAC field in the PEI). In this case, the UE Capability ID uniquely identifies a set of UE Radio Capabilities for this manufacturer, and together with this UE manufacturer information uniquely identify this set of UE Radio Capabilities in any PLMN;

-	PLMN-specific: If a manufacturer-assigned UE Capability ID is not used by the UE or the serving network, or it is not recognised by the serving network, the serving core network may allocate  UE Capability IDs for the UE corresponding to different sets of UE Radio capabilities. In this case, the UE Capability IDs the UE receives are applicable to the serving PLMN and uniquely identify correspondingset of UE Radio Capabilities in this PLMN;

NOTE 1:	Which one of Manufacturer-specific or PLMN-specific and associated procedures for assignment are mandatory or optional in the UE, will be decided in normative phase of the work.

-	When the UE Capability ID is allocated by the serving PLMN the same PLMN-specific UE Capability ID can be provided to multiple different UEs with the same radio capabilities or different PLMN-specific UE Capability ID can be provided to multiple different UEs with the same radio capabilities. The strategies for allocating PLMN-specific UE Capability ID are based on local policy;

NOTE 2:	When the UE Capability ID is allocated by the serving PLMN, the method that the core network detects that the same UE Radio Capabilities are signalled by mutliple different UEs or the same "model" in order to allocate the same PLMN-specific UE Capability ID from SA2 point of view is left up to implementation or can be decided by RAN.

-	The UE stores the PLMN-specific UE Capability ID in non-volatile memory when in RM-DEREGISTERED state and can use it again when it registers in the same PLMN.

NOTE 3:	The number of PLMN-specific UE Capability IDs that UE stores in non-volatile memory is left up to UE implementation.

-	It shall be possible for a UE to change the set of UE Radio Capabilities in time and signal the associated UE capability ID, if available.

-	if a UE capability ID is assigned by a PLMN when a UE capability filter is used, then the UE capability ID is used only when the PLMN applies the same filter (the UE may otherwise signal a wider set of capabilities and use or obtain a different  UE capability ID).

-	The network or the Manufacturer shall be able to change the UE Capability ID associated with a device, e.g., due to a SW upgrade enabling new UE Radio Capabilities on the device side (for the manufacturer assigned UE Capability ID) or for remapping or OAM procedures in the network side;-	At any given instant the UE has only one UE capability ID that is indicated to the network.

-	Solution #9 (UE Capability ID with delta set of capabilities) is recommended for normative work, subject to feasibility being confirmed by RAN WG2.

-	The mapping between a specific capability ID and a corresponding set of capabilities does not change once set.

For key issue #2 the following principles are agreed:

-	Owing to the need to support UE Radio Access Capabilities > 65 536 bytes (i.e. > 524 288 bits), and, the need to support fast, reliable, low processing complexity mechanisms for frequently used procedures (at least Service Request, RRC Connection Resume, X2&Xn handover, secondary gNB addition), the full UE Radio Access Capabilities shall not normally be transferred as part of those procedures. This requires that the serving and target RAN stores a local copy of the mapping between the UE Capability IDs and the full UE Radio Access Capabilities for the UEs that frequently use that RAN node.

-	if a UE capability ID assigned by PLMN is the result of the UE signalling a set of capabilities related to a UE capability filter provided by the network, the UE capability ID is stored always alongside (a reference to) the filter which was used.

-	AMF that supports the RACS feature is mandated to have access to full set of UEs radio capabilities and the mapping between UE Capability ID and corresponding UE radio capabilities for at least the UEs registered in this AMF;

-	NG-RAN that supports RACS, is mandatory to be able to maintain local storage of UE radio capabilities and have access to the mapping between the UE Capability ID and the full set of UEs radio capabilities;

-	A specific NG-RAN node that does not have the mapping between a specific UE Capability ID and the corresponding UE radio capabilities, shall be able to retrieve the mapping from CN.

For key issue #3, the following principles are agreed:

-	RACS procedures will apply to 5GS. If there is interest similar procedures may apply to EPS but will be decided based on the objectives of the related work item in normative phase;

-	From SA WG2 point of view, for UEs that support the RACS feature, for UEs that are already assigned with an applicable UE Capability ID, it is mandatory to signal the UE Capability ID in Initial Registration;

NOTE 4:	Whether UE indicates the UE capability ID via NAS or via RRC connection establishement+N2 signalling will be determined in coordination with RAN2 and SA WG3.

-	For backwards compatibility between nodes that support the feature and nodes that do not support the feature, if a peer node is not supporting RACS, the source node attempts to send to the peer node the UE capabilities that map to the UE capability ID.

-	When a UE capability ID is associated to a UE capability filter, the association to this filter is conveyed over the signalling interfaces when a mapping between the UE capability ID and the UE capabilities is provided.

NEXT CHANGE

END OF CHANGES