SA WG2 Meeting #130	S2-1900066

21 – 25 January, Kochi, India	(revision of S2-19xxxx)

Source:	Vodafone

Title:	Addressing a duplicated pargraph

Document for:	Approval

Agenda Item:	6.29

Work Item / Release:	FS_UDICOM / Rel-16

Abstract of the contribution: Addressing a duplicated paragraph

# 1 Proposal

A duplicated paragraph is deleted and relevant text from S2-1812636 is added.

******************First change***********************

##### 6.5.3.5.3	Terminating domain selection information for IMS voice

According to TS 23.501 [6], clause 5.16.3.6, the UDM/HSS shall be able to query the serving AMF for T-ADS related information, and AMF shall respond to the query with the relevant information unless the UE is detached:

-	whether or not IMS voice over PS Session is supported in the registration area(s) where the UE is currently registered;

-	whether or not IMS voice over PS Session Supported Indication over non-3GPP access is supported in the WLAN where the UE is currently registered;

-	the time of the last radio contact with the UE; and

-	the current Access Type and RAT type.

Retrieval of T-ADS information is initiated by the IMS AS.

To retrieve the T-ADS information from either the UDM or the HSS FE, the SLF functionality is invoked in order to perform a user identity to HSS FE identity or UDM identity resolution.

The SLF will return the list of different servers (UDMs and HSS FEs) that may be in charge of the UE at that point in time, for the IMS AS to request the information for access domain selection. This procedure is according to TS 29.328 [11] clause 6.5.

If the UE is attached to IMS via 5GS, the T-ADS information request to the IMS HSS/SLF will end up in a Sh request (UDR/UDA) to UDM, which in turn will invoke the AMF service Namf_MT_ ProvideDomainSelectionInfo as specified in TS 23.501 [6] clause 5.16.3.6 and TS 23.228 [7] Annex Y.

To translate IMPU to SUPI, for the cases in which the Sh request is based on IMPU, the 5G UDR can incorporate a new data key (=IMPU) for the Subscription Data data set to be used for the Nudr_DM_Query operation, in order to request identifier translation from IMPU to SUPI.:

The query to the 5G UDR is simply converted by the Access Data Layer into a Ud request to the IMS data repository, in order to retrieve the IMSI that is associated with the IMPU received in the Sh request.

The request from IMS AS, when the UE is attached to the EPS, should follow the same procedures as defined in TS 23.228 [7] clause 4.2.4a.

NOTE 1:	It is assumed that IMS AS is aware of the IMS registration status of the UE prior to request T-ADS information.

Both UDM and HSS FE will check the registration status in the corresponding network in order to either execute the procedure (according to TS 23.228 [7]) or reject it.

In case of dual registration, and according to annex D of TS 29.238 [11], the IMS AS will expect the T-ADS information containing 0 or 1 RAT types, depending on whether IMS Voice over PS is supported or not. So, in order to avoid impacts on the IMS AS, this solution proposes that UDM and HSS FE are configured with indication regarding being the default system for IMS voice. Both will inspect this default indication, in order to execute the T-ADS information retrieval procedure, or reject it.

******************End of changes***********************