SA WG2 Meeting #130	S2-1900303

January 21 - 25, 2019, Kochi, India

Source:	Nokia, Nokia Shanghai Bell

Title:	evaluation of solutions 3.

Document for:	Approval

Agenda Item:	6.29

Work Item / Release:	FS_UDICoM/Rel-16

Abstract of the contribution:

# 1.	Proposal

This document provides some views on solution 3 in TR 23.732.

For the case of T-ADS where the UE can be registered on EPS on 3G and also on 5GS, the problem exists that the no support of Voice over PS may arrive from HSS to the UDM and unless a timestamp is indicated it is not possible to know whether the Support of Voice over PS from 5GS the UDM retrieves from an AMF was related to a registration that happened before or after the registration on 3G. so it needs to be clarified in the documentation that this timestamp is required, and this impacts the Sh specification.

It is proposed that the following text changes and proposals are added to TR 23.732.

***********************PROPOSED TEXT********************************

#### 6.3.3.3	T-ADS for IMS voice

The procedure of the interaction between UDM and HSS FE is shown in Figure 6.3.3.3-1.

Figure 6.3.3.3-1: T-ADS Procedure between UDM and HSS FE

1.	SCC AS initiates T-ADS query towards UDM by sending a User-Data-Request message via Sh interface.

2.	UDM forwards this User-Data-Request message to get T-ADS information related to EPS and UMTS.

3.	HSS FE may query MME to get more T-ADS information as specified in TS 23.401 [16].

4.	HSS FE may query SGSN to get more T-ADS information as specified in TS 23.060 [15].

5.	HSS FE returns T-ADS information based on the information it derives by sending a User-Data-Answer message. The HSS includes a timestamp of the time when the UE registered on 3G or 4G.

6.	after receiving T-ADS information from HSS FE, UDM may query AMF for more T-ADS information as specified in TS 23.501 [6].

7.	UDM determines T-ADS result based on T-ADS information received from HSS FE and AMF and responds to SCC AS with the T-ADS information by sending a User-Data-Answer message.

NOTE:	the other signalling interactions not related to the solution are not present in this figure for simplification.

#### ***********************PROPOSED TEXT********************************

### 6.3.4	Impacts on existing services and interfaces

#### 6.3.4.1	Authentication impacts

UDM supports:

-	generating Diameter S6a Authentication-Information-Request message based on Nudm_UEAuthentication_Get service operation;

-	forwarding Diameter S6a Authentication-Information-Answer message with authentication vectors to MME;

-	generating the respond to Nudm_UEAuthentication_Get service operation invocation with authentication vectors received in Authentication-Information-Answer, and sends the respond to AMF;

-	generating and sending Location Update Request message to perform fake 4G registration in HSS FE when the UE is registered in 5G;

-	de-registering the UE in 5G when it receives Cancel Location request with dedicated MME Identity from the HSS FE.

HSS FE: No impact.

5GS UDR: No impact.

EPS UDR: No impact.

#### 6.3.4.2	IP address continuity impacts

UDM supports:

-	forwarding Diameter S6a Update Location Request message to HSS FE;

-	retrieving PGW-C+SMF FQDN from 5GS UDR after receiving Update Location Ack from HSS FE.

-	generating new Update Location Ack to add PGW-C+SMF FQDN into the message and forwarding the message to MME.

HSS FE: No impact.

5GS UDR: No impact.

EPS UDR: No impact.

#### 6.3.4.3	T-ADS for voice impacts

UDM supports:

-	forwarding User-Data-Answer message to HSS FE;

-	determine T-ADS result based on T-ADS information received from HSS FE and AMF.

-	Significant part of the HSS FE functionality needs to be supported in UDM

HSS FE: Storing time stamp of registration in UDR and provide this for T-ADS.

5GS UDR: Storing time stamp of registration.

EPS UDR: Storing time stamp of registration.

#### 6.3.4.4	Supporting IMS related procedures

UDM supports:

-	forwarding requests for IMS procedures received from IMS to HSS FE and further forwarding responses from HSS FE to IMS;

-	forwarding requests for IMS procedures received from HSS FE to IMS and further forwarding responses from IMS to HSS FE;

HSS FE: No impact.

5GS UDR: No impact.

EPS UDR: No impact.

### 6.3.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

There is impact on Sh interface specification to support the transfer of Registration time stamps for T-ADS. So, this means the solution is impacting specifications that are not related to interaction between HSS and UDM only.

The same applies for S6a specification where the 5G authentication vectors need to be carried and also 5G vectors need to be requested.

***********************END of PROPOSED TEXT***************************