SA WG2 Meeting #130	S2-1900351

January 21-25, 2019, Kochi, India	(revision of S2-19xxxx)

Source:	Cisco Systems

Title:	Solution for selection of Rx vs N5 by P-CSCF

Document for:	Agreement

Agenda Item:	X.X

Work Item / Release:	FS_eIMS5G/ Release 16

Abstract of the contribution: Proposes solution for selection or Rx vs N5 by P-CSCF

# 1. Discussion

In this contribution we look at solution to the one of the sub key issues of key issue #7:

-	how IMS utilizes services provided by Npcf to support the interaction with PCF.

-	what information is used by P-CSCF and how P-CSCF derives the information to determine which PCC interface is used for a specific IMS session establishment, when P-CSCF supports N5 and Rx simultaneously.

-	what input and output parameters are needed in Npcf service operations to add support existing IMS procedures over 5GS.

The reason why it is important for the P-CSCF to know this is such that the P-CSCF will not need to perform trial-and-error to determine if it should use Rx or N5, since this would also increase call-setup times.

Pictorially this is depicted in the figure below:

UE when it attaches to 5GC, can connect in one of the following two ways:

A.	Using NR or eLTE access, or

B. Using LTE access but then anchored on SMF+UPF.

The main issue is for the P-CSCF to know whether the UE is anchored at the SMF+UPF (in which case the P-CSCF will use the N5 interface, or the UE is anchored on PGW (in which case the P-CSCF will use the Rx interface).

When the UE is attached via NR or eLTE access (#A) and performs IMS registration, the P-CSCF (and I-CSCF and S-CSCF) knows that the UE is anchored on SMF+UPF, since the UE provides in the P-Access-Network information, the following according to 24.229,

A.1 NR: access-type = "3GPP-NR-FDD" or "3GPP-NR-TDD"

A.2 eLTE: access-type = 3GPP-E-UTRAN-FDD" or "3GPP-E-UTRAN-TDD" and "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), TAC Tracking Area Code  to 6 hex digits. (4 hexadecimal digits when accessing to EPC and 6 hexadecimal digits when accessing to 5GCN)

However, if the UE is attached via LTE then it is not possible for the P-CSCF to differentiate between the following two cases:

B.1 LTE to PGW: access-type = 3GPP-E-UTRAN-FDD" or "3GPP-E-UTRAN-TDD" and "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), TAC Tracking Area Code  to 4 hex digits. (4 hexadecimal digits when accessing to EPC and 6 hexadecimal digits when accessing to 5GCN)

B.2 LTE to SMF+UPF: access-type = 3GPP-E-UTRAN-FDD" or "3GPP-E-UTRAN-TDD" and "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value),TAC Tracking Area Code  to 4 hex digits. (4 hexadecimal digits when accessing to EPC and 6 hexadecimal digits when accessing to 5GCN)

There is no information in the SIP header of IMS registration that would enable the P-CSCF to differentiate between case B.1 and B.2.  For B.1 the P-CSCF should use the Rx interface (Diameter) and for B.2 the P-CSCF should use N5 interface.

Also, the UE's NAS stack will not be able to differentiate if the UE is anchored on PGW or SMF+UPF. There can be secondary indication that the UE can use to determine if it is anchored on PGW or SMF+UPF (eg. if N26 interworking is supported, the UE will get mapped 5G QoS parameters), but this is not a clean and always available indication. For eg no 5G QoS parameters are provided if N26 interworking is not supported in the network.

Hence, network based solutions are needed to solve this ambiguity.

Two solutions in the TR are captured to solve this ambiguity:

-	Solution #20: Using some PGW-U/UPF node information (MAC address, etc).

-	Solution #21: Having different IP address pools for PGW-U and for UPF. The P-CSCF determines whether to use Rx or N5 based on UE's contact IP address.

Here we propose another solution to solve this problem.

-	The P-CSCF has two different FQDNs which map to two different IP@, FQDN-1 ~ IP@1 and FQDN-2 ~ IP@2.

-	FQDN-1/IP@1 is used for IMS registrations when UE is anchored on PGW

-	FQDN-2/IP@2 is used for IMS registrations when UE is anchored on SMF+UPF.

When the UE creates PDN connection anchored at the PGW, the FQDN-1/IP@1 of the P-CSCF is provided to the UE either as part of attach accept or as part of DHCP configuration. When the UE creates PDN connection which is anchored at SMF+UPF, FQDN-2/IP@2 is provided to the UE.

This procedure will not work if the UE is configured with P-CSCF FQDNs and uses those, since it is not possible to inform the UE if the IMS PDN connection is anchored at the PGW or SMF+UFP. However, the PGW/SMF+UPF can always provides FQDN/IP@ of P-CSCF that overrides the configuration in the UE.

## 2. Proposal

Update to 23.794

## 6.x	Solution x: Selection of Nx or N5 based on two different P-CSCF FQDN/IP@

### 6.x.1	Description

This is a solution to part of Key issue #7 – specifically the question of what information is used by P-CSCF and how the P-CSCF derives the information to determine which PCF interface is used for a specific IMS session establishment, when P-CSCF supports N5 and Rx simultaneously"

The solution is as follows.

-	The P-CSCF has two different FQDNs which map to two different IP@, FQDN-1 ~ IP@1 and FQDN-2 ~ IP@2.

-	FQDN-1/IP@1 is used for IMS registrations when UE is anchored on PGW

-	FQDN-2/IP@2 is used for IMS registrations when UE is anchored on SMF+UPF.

### When the UE creates PDN connection anchored at the PGW, the FQDN-1/IP@1 of the P-CSCF is provided to the UE either as part of attach accept or as part of DHCP configuration. When the UE creates PDN connection which is anchored at SMF+UPF, FQDN-2/IP@2 is provided to the UE.

### 6.x.2	Impacts to Existing Nodes and Functions

P-GW

-	Configured to provide FQDN-1/IP@1 of the P-CSCF. Also if DHCP is being used, the DHCP-proxy at PGW only provides FQDN-1 (and not FQDN-2) of the P-CSCF.

SMF+PGW-C

-	Configured to provide FQDN-2/IP@2 of the P-CSCF. Also if DHCP is being used, the DHCP-proxy at PGW only provides FQDN-2 (and not FQDN-1) of the P-CSCF.

P-CSCF

-	Needs to support two different IP@s for IMS registration of UEs, one for UEs whose IMS PDN connections are anchored at PGW and the other for UEs whose IMS PDU sessions are anchored at the SMF+UPD.

### 6.x.3	Evaluation

Editor's note:	This clause provides an evaluation of the solution

This solution requires the PGW/SMF+PGW-C to provide P-CSCF FQDN/IP@ to over-ride what is in UE's configuration.

This solution is based on configuration only and does not require updates to specifications.