3GPP TSG-SA WG2 Meeting #130 	S2-1900360

# Kochi, India, 21st January - 25th January

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0820 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Restriction of use of Enhanced Coverage in 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of restriction of Enhanced Coverage for 5GC as part of conclusion of Key Issue 6 in the TR 23.724. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new section is added to introduce Restriction of Enhanced Coverage in 5GC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Management of Enhanced coverage will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Start of Changes *****

## 5.x.y Restriction of use of Enhanced Coverage

Support of UEs in Enhanced Coverage is specified in TS 36.300 [5].

The usage of Enhanced Coverage requires use of extensive resources (e.g. radio and signalling resources). Specific subscribers can be restricted to use Enhanced Coverage feature through Enhanced Coverage Restricted information that is stored in the UDM as part of subscription data and specifies per PLMN whether the Enhanced Coverage functionality is restricted or not for the UE. Enhanced Coverage Restricted Information indicates whether CE mode B is restricted for the UE, or both CE mode A and CE mode B are  restricted for the UE, or both CE mode A and CE mode B are  not restricted for the UE.

The AMF receives Enhanced Coverage Restricted information from the UDM during the Registration procedure.The AMF based on local configuration, UE Usage setting, UE subscription information and network policies, or any combination of them, determines whether Enhanced Coverage (i.e. CE mode B or both CE mode B & CE mode A) is restricted for the UE and stores updated Enhanced Coverage restriction information in the UE context in the AMF. If the UE usage setting indicated that UE is "voice centric", then the AMF shall set CE mode B restricted for the UE in Enhanced Coverage restriction information.

If the UE includes the support for restriction of use of Enhanced Coverage, the AMF sends Enhanced Coverage Restricted Information to the UE in the Registration Accept message. The UE shall use the value of Enhanced Coverage Restricted Information to determine if enhanced coverage feature is restricted or not. The AMF provides an Enhanced Coverage Restricted Information to the RAN via N2 signalling whenever the UE context is established in the RAN, e.g. during N2 Paging procedure, Service Request procedure, initial Registration and periodic Registration procedure.

For roaming UEs, if the UDM doesn't provide any Enhanced Coverage Restricted information or the provided Enhanced Coverage Restricted information is in conflict with the roaming agreement, the AMF uses default Enhanced Coverage Restricted information locally configured in the VPLMN based on the roaming agreement with the subscriber's HPLMN.

The UE indicates its capability of support for restriction of use of Enhanced Coverage to the AMF in the Registration procedure for the RAT it is camping on. The UE that indicates its support Enhanced Coverage over N1 interface shall also support restriction of the Enhanced Coverage over N1 interface.

The UE shall assume that restriction for use of Enhanced Coverage is thesame in the equivalent PLMNs.

***** End Of Changes *****