3GPP TSG-SA2 Meeting #130	S2-1900560

# Kochi, India, 21 – 25 January, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0872 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Description of solution 7 in 23.725 as replication framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_URLLC |  |  |  |  |  | Date: |  |  | 2019-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Impement the conclusion of the FS_URLLC for user plane replication |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the possibility to support replication framework within the 5GS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusion of FS_URLLC not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change ( all new text)* * * *

5.x.y Redundancy framework in the 5G System

Replicator functionality can be supported within the 5GS without making any assumption on support for IEEE FRER in the application layer (DN only) at the same time it can be introduced by isolating it to the replicator framework. Replicator functionality within 3GPP system helps avoid blind replication irrespective of the type of application. If the application does not support replication of traffic stream but requires lowest latency and high availability, replicator within 5GS can perform replication of the traffic stream. If the application supports replication of traffic stream in order to support lowest possible latency, replicator within 5GS can avoid further replication of the traffic stream.

The main principles are as follows:

1.	Replicator functionality is able to detect multiple related flows, and whether they are utilized for redundant packets, of incoming IP/Ethernet flows at the transmitter side. Control plane aspects necessary for the replicator functionality resides in the SMF and the User plane aspects necessary for the replicator functionality resides in the UPF and the NG-RAN.

2.	The replicator guides the lower layers to ensure their corresponding latency/availability/reliability requirements are fulfilled.

a.	Case 1:  Based on the application requirement to support lowest latency and the need to support replication within 5G System (within the domain of 3GPP System), the replicator framework replicates the packets towards the lower layers.

b.	Case 2: Based on the application functionality to support redundancy by default, replication framework within 5G System (within the domain of 3GPP System), replicator framework can eliminate further replication rather it can transmit the packet stream just once towards the lower layers.

3.	For case 1, to ensure the two N3 tunnels can be transferred via disjointed transport layer paths, the NG-RAN node, SMF or UPF will provide different routing information in the tunnel information (e.g. different IP addresses or different Network Instances), and the routing information will be mapped to disjoint transport layer paths according to network deployment configuration.

* * * * End of Change * * * *