SA WG2 Meeting #130 	S2-1900569

# 21 - 25 January 2019, Kochi, India											 (revision of S2-190xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0961 | rev | - | Current version: | 15.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Selection of a combo PGW+SMF at EPC attachment via WLAN access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2019-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a 5G NAS capable UE attaches to EPC via 3GPP access, it indicates its support for 5G NAS protocol. The MME uses that information to select a combo PGW+SMF, which allows for a handover to 5GS. However, when the UE attaches to EPC via untrusted WLAN, there is only a sentence in the handover from EPC-ePDG to 5GS: "The UE has indicated its 5G NAS capability in order for the ePDG to select a combined PGW+SMF, and has provided the PDU Session ID to the combined PGW/SMF." But - it does not describe when and how the UE sends its 5G NAS capability and PDU Session ID. - it does not either describe how the ePDG gets subscription information (i.e. UE's subscription from HSS that includes UE's mobility restriction parameters related to 5GS and/or indication of support for interworking with 5GS for this APN) over SWm and SWx. - It does not either describe how the PGW+SMF is aware of the 5GS Interworking and of the PDU Session ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new subclause 4.11.2.4.x is added to the clause related to impacts to EPS procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When a 5G NAS capable UE attaches to EPC via untrusted WLAN, the ePDG would not always select a combo PGW-SMF and that a handover to 5GS would fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 4.11.2.4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.11.2.4	Impacts to EPS Procedures

##### 4.11.2.4.x	Initial Attach on S2b

Impacts on TS 23.402 [26], clause 7.2.1 are the following ones:

-	step 2:

If the UE supports 5GC NAS procedures (see clause 5.17.2 in TS 23.501 [2]), the UE shall indicate its support of 5G NAS in a NAS indicator to the ePDG during IKEv2 message exchanges. When the UE supports 5G NAS, it shall also provide the PDU Session ID during IKEv2 message exchanges.

UE's mobility restriction parameters related to 5GS and/or indication of support for interworking with 5GS for this APN are obtained by the ePDG as part of the reply from the 3GPP AAA Server to the ePDG. Together with the 5G NAS support indicator, it enables the ePDG to determine if PGW-C+SMF or a standalone PGW-C should be selected.

If the UE supports 5G NAS and the PDN connection is not restricted to interworking with 5GS by user subscription, the ePDG shall send the 5GS Interworking Indication and the PDU Session ID to the PWG+SMF in step 3.

END OF CHANGES