SA WG2 Meeting #130	S2-1900605

21 - 25 Jan 2019, Kochi, India	(revision of S2-19xxxx)

Source:	NTT DOCOMO, Sprint, AT&T

Title:	NF Service Instance Set, definition

Document for:	Discussion

Agenda Item:	6.19

Work Item / Release:	5G_eSBA/Rel-16

Abstract of the contribution: This document proposes a definition of NF Service Instance set.

# 1. 	Discussion

TR 23.742 V2.0.0 provides the following definition:

NF Service Instance Set is the group of NF Service Instances in the same NF instance that are of the same service type and are interchangeable. The NF Service Instance Set is identified by NF Service Instance SET ID.

This definition makes clear that a NF service instance set cannot span across multiple NF instances. But it is still not clear whether there can be multiple NF service instance sets for the same service type (identified by the “Service Name”) within the same NF instance. A decision on this topic is necessary also to clarify whether it is necessary to have a specific identifier for the NF Service instance set.

As there is no need to have multiple NF Service Instance Sets for the same service type within an NF instance, it is proposed to clarify this aspect in the definition.

Proposal 1: In an NF instance there is only one NF Service Instance Set for each Service Name. The NF Service Instance Set is identified by the combination of the NF instance ID and Service Name.

# 2. Conclusion

It is proposed to include in TS 23.501 the definition of Service instance Set provided by TR 23.742 with the clarification that there cannot be multiple NF Service instance Sets for the same service type within a Network Function.

Proposal 1: include the following definition in TS 23.501 (changes on top of the definition in the conclusions in TR 23.742)

NF Service Instance Set is the group of NF Service Instances in the same NF instance that are of the same service type. The NF Service Instances in the NF Service Instance Set are interchangeable (e.g. they can be used for subsequent service invocations concerning the same resource because they access to the same data context). In an NF instance there is only one NF Service Instance Set for each Service Name. The NF Service Instance Set is identified by the combination of the NF instance ID and Service Name.

Location suggested for inserting the text: TS 23.501, new clause 5.21.x under 5.21 (Architectural support of virtualized deployment).