3GPP TSG-SA2 Meeting #130	S2-1900607

# Kochi, India, 21 Jan 2019 - 25 Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.740 | CR | 0001 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Documenting the conclusion for FS_eNS KI#1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS |  |  |  |  |  | Date: |  |  | 15/01/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was decided in SA2#129bis to give more time to conclude KI#1, whether to not pursue the KI, or to select one of the two approaches described in section 8.1. S2-1900606 discusses in detail the differences between the two approaches and concludes that approach 2, with no normative impacts, is sufficient. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates the conclusion to document that approach 2 is selected for KI#1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no description for deployment hints to fulfil the scenarios in 23.501 Rel-16. This does not mean that the scenarios cannot be implemented already in Rel-15. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

# 8	Conclusions

## 8.1	Conclusions for key issue #1 - Mutually exclusive access to Network Slices

### 8.1.1	Support for mutually exclusive access to network slices

| Key attributes | Approach 1 (Soln 3,4,7) | Approach 2+3 (Soln 2+5) |
| --- | --- | --- |
| Handling of UE requesting incompatible slices | New cause code is needed for rel-16 to signal that some S-NSSAIs are rejected due to incompatibility | The AMF uses existing rel-15 procedures. |
| How is the UE aware of compatibility among S-NSSAIs so it forms a correct Requested NSSAI from the outset. | The CN configures the UE with information that the UE uses to assess whether a S-NSSAI is compatible with other S-NSSAIs on the configured NSSAI. The UE shall have the same per S-NSSAI compatibility information as the CN, so it can locally (e.g. by local MMI interaction outside the scope of 3GPP) form a compatible and acceptable by the user set of requested S-NSSAIs before it is sent to the network. | The UE is preconfigured with method outside the scope of 3GPP if needed. Otherwise, can sort S-NSSAIs in priority order. |
| How does the Serving PLMN understand the requested S-NSSAIs are compatible | Identify compatibility as a subscription attribute of the S-NSSAI so roaming partners and a PLMN can define which S-NSSAIs are compatible and map these to the Allowed NSSAI provided by NSSF to AMF or by an AMF, even if the AMF supports more S-NSSAIs the UE also subscribes to. In other words, the incompatibility among slices should not impose AMF Set deployment constraints. | AMF sets are deployed to match each and every compatibility groups. |
| Compatibility between UE and network | The UE signals to the network whether it support Mutually incompatible slices handling. If the UE does not support incompatible slices handling the UE is configured only with compatible Slices. The subscription information shall include a basic set of compatible Network Slices S-NSSAIs to be used if the UE the subscriber uses does not signal support of mutually incompatible slices. | It is assuming using rel-15 compliant UE and Rel-15 compliant network |
| Support in roaming to PLMNs that are not upgraded to support handling of incompatible slices | The subscription information sent to PLMNs of roaming partners which do not support incompatible slices handling shall only contain mutually compatible Network Slices S-NSSAIs. | As per Rel-15 The deployment of the VPLMN has to match requirements from HPLMN as per an SLA (i.e. the VPLMN deploys AMF Sets that satisfy the SLA for compatibility Groups of the inbound roamers) |

Approach 2 (documented in solution 2 in clause 6.1.2) is selected for the Rel-16 work item specification work (only informative impacts to the specifications).

### 8.1.2	Enhancements for change of a set of Network Slices

It is recommended to, during normative phase, address the issue of how the UE performs a change of S-NSSAIs in the Requested NSSAI e.g. whether the UE does it always from CM-IDLE or also from CM-CONNECTED.

End of changes