3GPP TSG-SA2 Meeting #130	S2-1900611

# Kochi, India, 21st Jan 2019 - 25th Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0888 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the concepts of NF Service instance Set and Service Zone. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Sprint, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce the concepts of NF Service instance Set and Service Zone, with the related identifiers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause under under 5.21 (Architectural support of virtualized deployment). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The concepts of NF Service instance Set and Service Zone, agreed in the conclusions of TR 23.742, remained undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

### 5.21.x	Service Zone

Service Zone can be used to describe the location of the NF/NF service instance (e.g. geographic location, data center) When a single NF/NF Service instance set expands across multiple locations within an operator network, the Service Zone ID can be used to refer to NF/NF Service instances within a Set in particular location.

The Service Zone ID can be used as an attribute of the NF service instance and NF instance. When the NF instance expands across multiple Service Zones, a list of corresponding Service Zone IDs is defined as attribute for the NF instance.

NOTE: The Service Zone ID corresponds to the “locality” information in stage 3.

### 5.21.y	NF Service Instance Set

NF Service Instance Set is the group of NF Service Instances in the same NF instance that are of the same service type. The NF Service Instances in the NF Service Instance Set are interchangeable (e.g. they can be used for subsequent service invocations concerning the same resource because they can access to the same data context). In an NF instance there is only one NF Service Instance Set for each Service Name. The NF Service Instance Set is identified by the combination of the NF instance ID and Service Name.

* * * End of Change 1 * * *