3GPP TSG-SA2 Meeting #130	S2-1900671

# Kochi, India, 21st Jan 2019 - 25th Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0986 | rev | - | Current version: | 15.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to 4.3.6, based on Solutions #6 & #13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_URLLC |  |  |  |  |  | Date: |  |  | 2019-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | URLLC study item studied different solutions and agreed to solution # 6 and solution #13, related to KI # 3 Enhancement of Session Continuity during UE Mobility. Both solutions has certain common aspects related to AF influence on traffic routing. Solution #13 requires that AF replies, either positive or negative, to received SMF/NEF early or late notification, along with required information such as any changed N6 routing details and Target Application Function resulting due to possible change in DN. In solution #6, steps 9 and 10 are related to SMF interaction with AF, and these needs to be reflected in the procedures corresponding to Application Function influence on traffic routing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional text/steps related to Handling AF influence on traffic routing, to enable solutions #6 and solution #13 as agreed in URLLC TR 23.725 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | URLLC TR 23.725 conclusions realted to Sol #6 & #13 not captured in TS 23.502 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** 1st Change *****

#### 4.3.6.3	Notification of User Plane Management Events

The SMF may send a notification to the AF if the AF had subscribed to user plane management event notifications as described in clause 4.3.6.2 and in TS 23.501 [2] clause 5.6.7. The following are the examples of such events:

-	A PDU Session Anchor identified in the AF subscription request has been established or released.

-	A DNAI has changed.

-	The SMF has received a request for AF notification and the on-going PDU Session meets the conditions to notify the AF.

The SMF uses notification reporting information received from PCF to issue the notification either via an NEF (2a, 2b and 4a, 4b) or directly to the AF (2c and 4c).

The following flow depicts the sequence of events:

Figure 4.3.6.3-1: Notification of user plane management event

1.	A condition for an AF notification has been met as described above. The SMF sends notification to the NF that is subscribed for SMF notifications. Further processing of the SMF notification depends on the receiving NF, as shown in steps 2a and 2c.

2a.	If early notification via NEF is requested by the AF, the SMF notifies the NEF of the target DNAI of the PDU Session by invoking Nsmf_EventExposure_Notify service operation.

2b.	When the NEF receives Nsmf_EventExposure_Notify, the NEF performs information mapping (e.g. AF Transaction Internal ID provided in Notification Correlation ID to AF Transaction ID, SUPI to GPSI, etc.) as applicable according to TS 23.501 [2], clause 5.6.7, and triggers the appropriate Nnef_TrafficInfluence_Notify message. In this case, step 2c is not applicable.

2c.	If early direct notification is requested by the AF, the SMF notifies the AF of the target DNAI of the PDU Session by invoking Nsmf_EventExposure_Notify service operation.

2d. AF replies to Nnef_TrafficInfluence_Notify either immediately or after any required application relocation in the new DN is completed. AF includes N6 traffic routing details corresponding to new PSA, Target-AF address, any preference on maintaining UE IP address. AF may reply in negative e.g. if the AF determines that the application relocation cannot be completed on time.

2e. AF replies to Nsmf_EventExposure_Notify either immediately or after any required application relocation in the new DN is completed. AF includes N6 traffic routing details corresponding to new PSA, incase Target AF identifier, any preference on maintaining UE IP address.  AF may reply in negative e.g. if the AF determines that the application relocation cannot be completed on time.

3.	The SMF enforces the change of DNAI or addition, change, or removal of a UPF. The SMF sends notification to the NF that is subscribed for SMF notifications. Further processing of the SMF notification depends on the receiving NF, as shown in steps 4a and 4c.

4a.	If late notification via NEF is requested by the AF, the SMF notifies the NEF of the selected target DNAI of the PDU Session by invoking Nsmf_EventExposure_Notify service operation.

4b.	When the NEF receives Nsmf_EventExposure_Notify, the NEF performs information mapping (e.g. AF Transaction Internal ID provided in Notification Correlation ID to AF Transaction ID, SUPI to GPSI, etc.) as applicable according to TS 23.501 [2], clause 5.6.7, and triggers the appropriate Nnef_EventExposure_Notify message. In this case, step 4c is not applicable.

4c.	If late direct notification is requested by the AF, the SMF notifies the AF of the selected target DNAI of the PDU Session by invoking Nsmf_EventExposure_Notify service operation.

4d. AF replies to Nnef_TrafficInfluence_Notify either immediately or after any required application relocation in the new DN is completed. AF includes N6 traffic routing details corresponding to new PSA, Target-AF address, any preference on maintaining UE IP address. AF may reply in negative e.g. if the AF determines that the application relocation cannot be completed on time.

4e. AF replies to Nsmf_EventExposure_Notify either immediately or after any required application relocation in the new DN is completed. AF includes N6 traffic routing details corresponding to new PSA, incase Target AF identifier, any preference on maintaining UE IP address.  AF may reply in negative e.g. if the AF determines that the application relocation cannot be completed on time.

***** End of Changes *****