SA WG2 Meeting #130	S2-1900810

21 – 25 January, Kochi, India	(revision of S2-1900059)

Source:	Vodafone

Title:	FS_UDICOM - Conclusion for KI#2

Document for:	Approval

Agenda Item:	6.29

Work Item / Release:	FS_UDICOM / Rel-16

Abstract of the contribution: This pCR proposes conclusion for FS_UDICOM KI#2

# 1 Proposal

As closure of UDICOM study approaches, evaluations and conclusions are deemed necessary. The proposal is to include conclusions for the study.

******************First change***********************

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.y Conclusion for Key Issue #2: Interaction between UDM and HSS with common repository

### 8.y.1 For EPS and 5GS interworking required for mobility cases from EPS to 5GS and from 5GS to EPS, including the impacts on SMS handling

Solution #2 is considered to progress as normative optional feature both at stage 2 and stage 3. A number of open issues listed below need to be addressed during normative phase:

- The solution needs to address cases in which the applications front end logic is split

- The solution needs to address conversion of Nudr to Ud is required when the common repository is based on EPS UDR.

For cases in which operator deployments do not consider feasible to upgrade legacy systems to SBA, it is recommended to capture in the normative stage 2 specifications the following implementation option:

- Use of an adaptation layer between front ends and EPS UDR repository (as in solution #4) making use of notifications to trigger the relevant procedures in 5GS or EPS. .

NOTE:	The use of an adaptation layer will always be needed when EPS UDR is used as the common repository, to translate Nudr requests and their associated data model with Ud requests, that have no standard data model. This conclusion does not imply any stage 3 work.

These conclusions require a new stage 2 spec to clarify the interactions between HSS and UDM. CT4 is expected to take responsibility over the new spec while SA2 will add relevant references in affected 5G specifications (TS 23.501, TS 23.502).

### 8.y.2 For IMS and 5GS interworking required for T-ADS, P-CSCF restoration and NPLI retrieval

Three different scenarios are addressed by the proposed solutions:

- HSS as single point of contact for IMS (diameter based)

- UDM as single point of contact for IMS (diameter based)

- IMS HSS/SLF as single point of contact for IMS (diameter based)

Any solution for IMS interactions needs to serve the plethora of different deployments and not preclude any conclusion taken by the ongoing study for evolving IMS to SBA (see ref [14]).

To facilitate this, the same conclusion as for key issue #1 are reached with the exception of combo UDM/HSS, and this is repeated here for the sake of clarity:

1.-	Progress normative work for new service(s) with new service Request-Response operation(s) produced by UDM: for T-ADS (i.e. IMS over PS support) and NPLI (i.e. time stamp and RAT type), and for P-CSCF restoration. The decision about the number of operations is left for normative work.

The operations can be consumed by HSS (as in solution #2) for scenarios in which HSS is the single point of contact for IMS (diameter based) interactions and adapted to SBA.

The operations can be consumed by an IMS HSS when in stand-alone mode and acting as a single point of contact for a diameter-based IMS. This case would correspond to solution #2 when the HSS referred acts as IMS HSS and is adapted to SBA.

NOTE 1: 	These new operations require coordination with the normative work derived from FS_eIMS5G [14].

2.-	Specify a mechanism to retrieve the UE SUPI, when IMS requests are based on IMPU, prior to triggering any retrieval of T-ADS/NPLI to AMF or carry out any P-CSCF restoration procedure. Where to store the IMPU-SUPI relation needs to be determined during normative phase.

NOTE 2: 	These conclusions require modification of TS 23.228 to clarify the interactions between HSS, UDM and IMS; the responsibility for this modification will be coordinated with any normative work arising from FS_eIMS5G [14].

******************End of changes***********************