3GPP TSG-SA WG2 Meeting #130	S2-1900854

# Kochi, India, 21 – 25 January 2019 													(revision of S2-1900789)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0744 | rev | 2 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SUPI and SUCI for wireline access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is a need to describe how SUPI and SUCI are used with wireline acces. Two cases need to be distinguished: 5G-RG and FN-RG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description of SUPI and SUCI for 5G-RG and FN-RG is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.9.2, 5.9.2a, 5.9.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS); Stage 2".

[6]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3".

[7]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[8]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[9]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[10]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[11]	ITU-T Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[12]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[13]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS): Stage 3".

[14]	IETF RFC 3736: "Stateless DHCP Service for IPv6".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 22.173: "IMS Multimedia Telephony Service and supplementary services; Stage 1".

[17]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[18]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[19]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[20]	IETF RFC 7542: "The Network Access Identifier".

[21]	3GPP TS 23.002: "Network Architecture".

[22]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[23]	3GPP TS 23.221: "Architectural requirements".

[24]	3GPP TS 22.153: "Multimedia priority service".

[25]	3GPP TS 22.011: "Service Accessibility".

[26]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[27]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[28]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[29]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[32]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[33]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[35]	3GPP TS 33.126: "Lawful Interception Requirements".

[36]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[37]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[38]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[39]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[40]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[41]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[42]	3GPP TS 38.401: "NG-RAN Architecture description".

[43]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[44]	IETF RFC 4960: "Stream Control Transmission Protocol".

[45]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[46]	3GPP TS 23.041: "Public Warning System".

[47]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[48]	3GPP TS 24.502: "Access to the 5G System (5GS) via non-3GPP access networks; Stage 3".

[49]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[50]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[51]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[52]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[53]	IETF RFC 1027: "Using ARP to Implement Transparent Subnet Gateways".

[54]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[55]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[56]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[57]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[58]	3GPP TS 29.510: "5G System: Network function repository services; Stage 3".

[59]	3GPP TS 29.502: "5G System: Session Management Services: Stage 3".

[60]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2) ".

[61]	3GPP TS 23.380: "IMS Restoration Procedures".

[62]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[63]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[64]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs".

[65]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[66]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[67]	3GPP TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[68]	3GPP TS 32.255: "5G Data connectivity domain charging; Stage 2".

[69]	TS 38.306: "NR; User Equipment -UE) radio access capabilities".

[70]	TS 36.306: "Evolved Universal Terrestrial Radio Access -E-UTRA); User Equipment -UE) radio access capabilities".

[71]	3GPP TS 29.274: "5G System; Access and Mobility Management Services; Stage 3".

[72]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[X]	BBF TR-101 issue 2 "Migration to Ethernet-Based Broadband Aggregation".

[Y] 	BBF TR-177 Issue 1 Corrigendum 1 "IPv6 in the context of TR-101".

[Z] 	IETF RFC 6788: "The Line-Identification Option".

[W] 	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

**** Next Change ****

### 5.9.2	Subscription Permanent Identifier

A globally unique 5G Subscription Permanent Identifier (SUPI) shall be allocated to each subscriber in the 5G System and provisioned in the UDM/UDR. The SUPI is used only inside 3GPP system, and its privacy is specified in TS 33.501 [29].

The SUPI may contain:

-	an IMSI as defined in TS 23.003 [19], or

-	a network-specific identifier, used for private networks as defined in TS 22.261 [2], or

-	a Line Id and an operator identifier of the operator administrating the Line ID value, used for supporting FN-BRGs, as further described in clause 5.9.X, or

- 	a HFC_Identifier and an operator identifier of the operator administrating the HFC_Identifier value, used for supporting FN-CRGs and 5G-CRG, as further described in clause 5.9.X.

It is possible for a SUPI to take the form of a Network Access Identifier (NAI) using the NAI RFC 7542 [20] based user identification as defined in TS 23.003 [19], for either IMSI based or non-IMSI based (e.g. when used over a non-3GPP Access Technology or for private networks) NAI.

In order to enable roaming scenarios, the SUPI shall contain the address of the home network (e.g. the MCC and MNC in the case of an IMSI based SUPI).

For interworking with the EPC, the SUPI allocated to the 3GPP UE shall always be based on an IMSI to enable the UE to present an IMSI to the EPC.

The usage of SUPI for W-5GAN is further specified clause 5.9.x.

**** Next Change ****

### 5.9.2a	Subscription Concealed Identifier

The Subscription Concealed Identifier (SUCI) is a privacy preserving identifier containing the concealed SUPI. It is specified in TS 33.501 [29].

For wireline access the SUCI may contain a Line Id or a HFC_Identifier, as further described in clause 5.9.X.

**** Next Change ****

The below changes are to be placed in new TS

### 5.9.X 	SUPI and SUCI for RG support

#### 5.9.X.1 	SUPI and SUCI for 5G-BRG support

The SUPI for an 5G-BRG shall contain an IMSI, as described in clause 5.9.2.

The SUCI provided by the 5G-BRG to the network contains the concealed SUPI, as described in TS 33.501 [29].

#### 5.9.X.2 	SUPI and SUCI for 5G-CRG support

The SUPI for a 5G-CRG subscription shall contain either an IMSI, as described in clause 5.9.2, or an HFC_Identifier. If the SUPI contains an HFC_Identifier, and since the HFC_Identifier is not globally unique, the SUPI also needs to contain an operator identifier of the operator administrating the HFC_Identifier value.

Editor’s note: The use of non-IMSI based SUPI and associated credentials and authentication method needs to be specified by SA3.

The SUCI provided by the 5G-CRG to the network contains the concealed SUPI, as described in TS 33.501 [29].

The format of the HFC_Identifier is an octet string and may contain a cable modem MAC address or an overall HFC account identifier, as defined by CableLabs in [W].

#### 5.9.X.3 	SUPI and SUCI for FN-BRG support

The SUPI for an FN-BRG subscription shall contain either an IMSI or a Line Id, based on operator configuration. If the SUPI contains a Line Id, and since the Line Id is not globally unique, the SUPI also needs to contain an operator identifier of the operator administrating the Line ID value.

The SUCI provided by the W-AGF to the 5GC for FN-BRG support always contains a Line Id together with an operator identifier of the operator administrating the Line ID value. If the SUPI associated with the subscription for FN-BRG contains an IMSI, the SUCI acts as pseudonym of the SUPI and UDM performs a mapping to a SUPI containing an IMSI. If the SUPI associated with the subscription for FN-BRG contains a Line Id, there is no need for a mapping in UDM.

As described in TS 23.003 [19], the SUCI also contains an identifier of the Home network.

Editor’s note: It is FFS whether a specific description of the Home ID for BBF scenarios is needed.

The format of the Line ID is generic, allowing different operators and access networks to use different encoding of the content. The Line ID can have the following formats:

-	DHCPv4 option 82 exchange (TR-101i2 [X] Annex B, original specification 2006)

-	PPPoE Circuit and Remote ID AVP insertion (TR-101i2 [X] Annex A/8.3, original specification 2006)

-	DHCPv6 use of option 18 via LDRA functionality in the access node (TR-177 corr 1 [Y], original specification 2010)

-	Use of the Line ID Option (LIO) in RS messaging (TR-177 corr1 [Y] , original specification 2010). Also documented in RFC 6788 [Z], which differs slightly from the BBF encodings.

Editor’s note: Additional details regarding SUCI, including whether or not a Line ID in SUCI is concealed, is to be deteremined by SA3. The text above about SUCI for wireline access may need to be updated based on SA3 outcome.

#### 5.9.X.4 	SUPI and SUCI for FN-CRG support

The SUPI for an FN-CRG subscription shall contain either an IMSI or a HFC_Identifier, based on operator configuration. If the SUPI contains a HFC_Identifier, and since the HFC_Identifier is not globally unique, the SUPI also needs to contain an operator identifier of the operator administrating the HFC_Identifier value.

The SUCI provided to the network for FN-CRG support always contains a HFC_Identifier together with an operator identifier of the operator administrating the HFC_Identifier value. If the SUPI associated with the subscription for FN-CRG contains an IMSI, the HFC_Identifier included in the SUCI acts as pseudonym of the SUPI and UDM performs a mapping to a SUPI containing an IMSI. If the SUPI associated with the subscription for FN-CRG contains a HFC_Identifier, there is no need for a mapping in UDM.

As described in TS 23.003 [19], the SUCI also contains an identifier of the Home network.

Editor’s note: It is FFS whether a specific description of the Home ID for Cable scenarios is needed.

The format of the HFC_Identifier is an octet string and may may contain a cable modem MAC address or an overall HFC account identifier, as defined by CableLabs in [W].

Editor’s note: Additional details regarding SUCI, including whether or not a HFC_Identifier in SUCI is concealed, is to be deteremined by SA3. The text above about SUCI for wireline access may need to be updated based on SA3 outcome.

**** End of Changes ****