3GPP TSG-SA WG2 Meeting #130	S2-1901026

21 – 25 January 2019, Kochi, India                                                       was S2-1900950/S2-1900533

Source:	China Mobile, Huawei, Ericsson, Nokia?

Title:	TS 23.288 NWADF-assisted QoS profile provision

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes to add the new feature about NWADF-assisted QoS profile provision into TS 23.288, related information and description are also given in this TS.

- Discussion

As discussed and studied in TR 23.791, Solution 3 is selected as the concluded solution for KI 5:NWDAF-Assisted QoS Profile Provisioning. This PCR proposes to add the new feature about NWADF-assisted QoS profile provision into TS 23.288 as an implementation.

- Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

2		References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[xx]	3GPP TS 23.502: "Procedures for the 5G System".

[yy]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[zz]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module ".

* * * Next change * * * *

7.X       Observed Service experience related network data analytics

7.X.1	General

NWDAF subscribes the network data from 5GC NF(s) defined in Table x. and the service data from AF, or via NEF, defined in Table y to offline training a Service MOS Model for the given application and then provides the observed service experience for an application, which is based on historical information for the application.

7.X.2	Input Data

The service data from the AF and the network data from 5GC NFs for QoS profile Provisioning are defined in Table 6.X.2-1 and Table 6.X.2-2 here below.

Table 6.X.2-1: Service Data from AF related to the observed service experience

| Information | Source | Description |
| --- | --- | --- |
| Correlation ID | AF | Could be e.g. IP address 5-tuple or newly allocated temporary ID by 5GC, which is used by NWDAF to correlate the service data from AF and the network data from 5GC NF for the service. (NOTE 1) |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service) |
| Observed Service Experience | AF | E.g. Service MOS, MOS or video MOS as specified in ITU-T P.1203.3 [zz], to characterize the experience for the service. (The observed level of service by AF) |
| Timestamp | AF | A time stamp associated to the observed level of Service Experience provided by the AF, mandatory if the observed Service Experience is provided by the ASP. |

NWDAF subscribes the service data from AF in the table 6.X.2-1 by invoking Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = Application ID, Target of subscription = Any UE) as defined TS 23.502 [xx].

Editor´s note: whether the correlation ID is IP address 5-tuple or newly allocated temporary ID by 5GC and how to correlate the Service data and Network data is FFS.

Table 6.X.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Correlation ID | PCF | Could be e.g. IP address 5-tuple or newly allocated temporary ID by 5GC , which is used by NWDAF to correlate the service data and the network data for the service |
| Timestamp |  | The timing for the service. |
| Location Info | AMF | The UE location information when the service is delivered. |
| DNN |  | To identify the DNN for the PDU Session which contains the QoS flow |
| S-NSSAI |  | To identify the S-NSSAI for the PDU Session which contains the QoS flow |
| Application ID |  | Provided by the AF, which is used by NWDAF to identify the application service provider and application for the QoS flow |
| IP filter information |  | Provided by the AF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow |
| QoS flow Bit Rate |  | The observed bit rate |
| QoS flow Packet Delay |  | The observed Packet delay |
| QoS flow Packet Error Rate |  | The observed packet error rate |

NWDAF subscribes the network data from 5GC NF(s) in the table 6.X.2-2 by invoking Nnf_EventExposure_Subscribe service (Event ID = 5QI Statistics, Event Filter information = Application ID, Target of subscription = Any UE) as defined in TS 23.502 [xx].

Editor´s note: The NWDAF needs information on the observed QoE at the UPF, how this information is provided to NWDAF needs to be studied.

7.X.3	Output Analytic

The NWDAF services as defined in the clause 7.2.2 and 7.2.3 are used to exposure the observed service experience (i.e. average observed Service MoS) from the NWDAF to the consumer NF (e.g. PCF), indicating e.g. how well the used QoS Parameters satisfy the Service MoS agreed between the MNO and the end user or between the MNO and the external ASP. A new Analytic ID named “Service Experience” is defined with one or multiple combinations of the following  analytic filters:

- 	Application ID

-	Network area

- 	Time window

-	Media/application bandwidth

-	S-NSSAI

-	DNN

7.X.4	Procedures

7.X.4.1	NWDAF assisted  QoS profile provisioning and adjustment

Figure 6.X.4.1-1: Procedure to NWDAF assisted QoS profile provisioning and adjustment

For an application Id, a set of initial QoS parameter combinations per service MOS window (e.g. one is for 3<Service MOS<4 and the other is for 4<Service MOS<5) is defined in PCF (e.g. by configuration of operator policies).

1.	PCF send an Analytics request/subscribe (Event ID = Service Experience statistics, Event Filter information = (Application ID, time window, S-NSSAI, DNN)) to NWDAF by invoking a Nnwdaf_EventSubscription_Subscribe.

2a. NWDAF subscribes the service data from AF in the Table 6.X.2-1 by invoking Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = Application ID, Target of subscription = Any UE) as defined TS 23.502 [xx].

2b.	NWDAF subscribes the network data from 5GC NF(s) in the Table 6.X.2-2 by invoking Nnf_EventExposure_Subscribe service (Event ID = 5QI Statistics, Event Filter information = Application ID, Target of subscription = Any UE) as defined in TS 23.502 [xx].

2c. With these data, the NWDAF offline trains a Service MOS Model for the given application, which will be used to determine the Service MOS for an application later on.

NOTE 1:	The data collection call flow only shows a subscription-notification model for the simplicity instead of both request-response model and subscription-notification model.

NOTE 2:	The explicit list of input data is not finalised and whether NWDAF needs to collect management data from OAM, such as PM measurements, is to be resolved in normative phase and some discussion with SA5 is needed to see what measurements are available and needed.

NOTE 3:	QoE measurements from the applications are based on outcome of the ongoing SA5 Rel-16 WID  "Management of QoE measurement collection" which addresses how to collect the QoE measurements from the applications in the UE.

3.	The NWDAF provides the data analytics, i.e. the observed Service Experience (maybe a range) to the PCF by means of Nnwdaf_EventSubscription_Notify, indicating how well the used QoS Parameters satisfy the Service MoS agreed between the MNO and the end user or between the MNO and the external ASP:

a)	the Service Experience statistics e.g. average Service MOS.

b)	Spatial validity condition, when the estimated Service Experience applies.

c)	Time validity condition, when the estimated Service Experience applies.

d)	NWDAF transaction id.

NOTE 4:	The call flow only shows a request-response model for the simplicity instead of both request-response model and subscription-notification model.

4.	Based on the observed Service Experience (this can be a range) provided by NWDAF, and the operator policies that takes into account the SLA (including the required Service Experience), the PCF takes into account to determine the used QoS parameters to be applied for the service if PCF determines that application SLA is not satisfied, which is defined in clause 6.1.1.3 and clause 6.2.1.2, TS 23.503 [zz].

NOTE 5:	The non-real time data information from AF includes the overserved service experience data (see Table 6.X.2-1), which indicates the service quality during the service lifetime.

Editor’s Note:	The corresponding clause 6.1.1.3 and clause 6.2.1.2, TS 23.503 will be updated in the future meeting.

* * * End of changes * * * *