SA WG2 Meeting #130 	S2-1901384

21 - 25 January 2019, Kochi, India	 (revision of S2-1901135)

Title:	LS on configured NSSAI handlingLS on Configured NSSAI handling

Release:	REl-15

Work Item:	5GS_Ph1

Source:	SA2

To:	CT1

Contact Person:

#### Name:	Alessio Casati

E-mail Address:	alessio.casati@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-1901252

1. Overall Description:

SA2 has discussed the case when a roaming UE with Home routed PDN connections/PDU sessions moves from EPS to 5GS or between 5GS PLMNs and what Requested NSSAI should be indicated when the UE has no Configured NSSAI nor allowed NSSAI in the 5GS/target PLMN but it has some PDN connections/PDU sessions established in EPS/Source PLMN which are kept as the UE moves to 5GS/the target PLMN.

SA2 has agreed the attached CR which requires the UE to include in the Requested NSSAI mapping information the S-NSSAIs of the HPLMN associated to the established PDU sessions. However, the corresponding S-NSSAI value for the 5GS/ target PLMN is not known as the UE has no configured NSSAI nor allowed NSSAI for the 5GS/Target PLMN.

SA2 has chosen this design so that the mapping information is clearly detected to be incorrect (as it contains an invalid S-NSSAI value for the VPLMN), so this trigger the AMF to provide the UE with a Configured NSSAI.

2. Actions:

To CT1 group.

ACTION: 	SA2 would politely request CT1 to consider the attached CR and to provide feedback on whether SA2 should reserve SST=0 for the purpose of indicating an “unknown S-NSSAI” value.

3. Date of Next TSG-SA WG2 Meetings:

TSG-SA WG2 Meeting #131	25 Feb - 1 Mar 2019	Santa Cruz - Tenerife, ES

TSG-SA WG2 Meeting #132 	08 - 12 Apr 2019		RP China