SA WG2 Meeting #S2-131	S2-1901420

25 February - 1 March, 2019, Santa Cruz - Tenerife	(revision of S2-1900680)

SA WG2 Meeting #S2-130	S2-1900680

21 - 25 January, 2019, Kochi, India

# Liaison Statement

| Liaison Statement Title: | LS to 3GPP SA2 on MME Functionality to Control Emergency Call Bearer Support Indicator |
| --- | --- |
| Security Classification: | Non-confidential |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| GSMA NG Packet #100 | 6th November 2018 | Tokyo, Japan |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Authors: |
| PACKET Doc 100 202 | 16th October 2018 | Azumi Matsusako |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| GSMA NG Packet |  | azumi.matsusako.nb@s1.nttdocomo.com |

| Action Required by Recipient |  |
| --- | --- |
| Internal Recipients: | NG-RILTE |
| External Recipients: | 3GPP SA2 |

# Background

As VoLTE advances and more operators are launching VoLTE roaming, becomes evident the need for requirements on emergency call such as the willing for VPMN to use IMS emergency call for their subscribers and for specific inbound roaming partners, as well as CSFB for other inbound roaming partners.

GSMA NG Packet has agreed that 3GPP specifications allow multiple solutions to cover the above-mentioned requirements. However, operators observed, through testing, that many of those solutions (as outlined in the attached document) are not feasible, and GSMA NG Packet would like to see a unified solution that would work with all VoLTE devices.

Only feasible solution seems to be the one which relies on EMC BS Indicator in MME, however, its usage appears to be implementation specific.

# Request to support selective emission of EMC BS flag by the MME based on Home network

In order to control the domain of emergency call per roaming partner, GSMA NG Packet kindly requests 3GPP SA2 to support and implement the “Emergency bearer service indicator” (EMC BS) by the MME per Home network (roaming partners) (from the earliest possible 3GPP Release).

The following table describes different EMC BS configuration options in order to support different emergency call cases for both VPMN’s own users and for inbound roamers.

| Cases supported by VPMN |  | Value of EMC BS |  |
| --- | --- | --- | --- |
| For VPMN’s own users | For in-bound roaming users | For VPMN’s own users | For in-bound roaming users |
| IMS emergency call | IMS emergency call | 1(supported) | 1(supported) |
| CS fall back emergency call | CS fall back emergency call | 0(not supported) | 0(not supported) |
| IMS emergency call | CS fall back emergency call | 1(supported) | 0(not supported) |
| CS fall back emergency call | IMS emergency call | 0(not supported) | 1(supported) |

# Next Meetings for PACKET

5th Feb 2019

2nd April 2019

# Attachment

Packet Doc 100_106 PPT-EMC BS.pptx