SA WG2 Meeting #131 	S2-1901538

# 25 February – 1 March 2019, Santa Cruz - Tenerife, Spain		(revision of S2-19xxxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0931 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Access Network Performance Measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR introduces the Access Performance Measurements related aspects for ATSSS. It includes general principles about measurement Assistance Information and the list of measurements that apply for this release of the specification. It also includes - details on the RTT measurements and on the Access Availability/Unavailability reporting; - the protocol stack used to convey measurement messages and reporting messages between UE and UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause Access Performance Measurements is added to 23.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance Access Measurements would not be specified and ATSSS feature cannot be implemented for switching and smallest delay steering modes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 5.x.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.x.x	Access Performance Measurements

#### 5.x.x.1	General principles

When the UE requests an MA-PDU Session Establishment and indicates it is capable to support ATSSS, the UE may receive Measurement Assistance Information, during the establishment of an MA-PDU session. This information assists the UE in determining which measurements shall be performed over both accesses, as well as whether and when measurement reports need to be sent to the network.

Measurement Assistance Information shall include:

-	the IP address and the port of a Performance Measurement Function (PMF) in the UPF, with which the UE can initiate measurement sessions;

-	the list of measurements to be performed and, for each measurement type, the conditions under which they are performed:

-	the list of steering modes related to the measurement (e.g. RTT for Smallest Delay);

-	if the measurement needs to be reported to the network, the conditions of reporting (thresholds, reporting frequencies, the QoS used to convey the measurement reports);

In this release of the specification, the following measurement are supported:

-	Round Trip Time (RTT) measurement over both accesses on UE and UPF;

-	Access availability/unavailability sent by the UE to the UPF.

#### 5.x.x.2	Round Trip Time measurement

If required by the network in ATSSS rules at the MA-PDU session level, RTT is measured at each side (UE and UPF) independently. There is no reporting from one side to the other.

The estimation of the RTT by the UE or by the UPF is based on Echo Request / Echo Response mechanism: the entity (UE or UPF) sends an Echo Request to the destination address/port of the PMF function in the peer entity, which replies with an Echo Response to the source address/port of the PMF function in the originating entity. The originating entity does this several times (e.g. one message every S seconds for N times) and is able to derive an estimation of the RTT based on the average of the delays obtained between Echo Request and Echo Reponse. Periodicity P of such measurement is defined in the ATSSS rule.

The address/port of the PMF function in the UE is provided to the UPF during the MA-PDU Session Establishment.

Editor's Note:	Whether the 5QI value used to convey the Echo Request / Echo Response messages is determined by an ATSSS rule or is pre-configured in both UE and UPF is FFS.

#### 5.x.x.3	Access Availability/Unavailability

If required by the network in ATSSS rules at the MA-PDU session level, the detection of the unavailability and of the availability of an access, as well as and the decision of reporting it, can be performed by the network or the UE, based on implementation. When the UE has decided to report the unavailability or the availability of an access, it shall:

-	build a measurement report containing the access type, the indication of availability or of unavailability, the UE source address, the UPF Performance Management target address;

-	send the measurement report to the UPF via the user plane.

#### 5.x.x.4	Protocol stack for user plane measurements and measurement reports

Figure 5.x.x.4-1: UE-PMF / UPF-PMF control plane for 3GPP access

Figure 5.x.x.4-2: UE-PMF / UPF-PMF control plane for non-3GPP access

END OF CHANGES