3GPP TSG-SA WG2 Meeting #131	S2-1901829

# Santa Cruz-Tenerife, Spain, 25 Feb – 1 March, 2019  	 (builds from S2-1900876)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0966 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architecture for FN-RG, UE behind RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CableLabs, Charter Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  |  |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As noted in p-CR S2-1900876 as agreed upon in SA2 #130, the architectures for FN-RG and UE behind RG needs to be added to 23.501. This contribution adds the FN-RG and UE behind RG, building upon S2-1900876. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following revisions are proposed: - addition of the architecture for the FN-RG - addition of the architecture for a UE behind RG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of support of FN-RG and UEs behind RGs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.8.4 (New, as agreed upon in SA2 #130)) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | It is expected this CR will be merged to the final form of CR based off the endorsed S2-1900876. The delta from S2-1900876 is highlighted in green. |  |  |  |  |  |  |  |  |

******************************* start  1st changes *********************

#### 4.2.8.4 Architecture Reference Model for Wireline Access

Figure 4.2.8.4-1: Non- roaming architecture for 5G Core Network for 5G-RG with Wireline 5G Access network and NG RAN

The 5G-RG can be connected to 5GC  via W-5GAN, NG RAN or via both accesses.

NOTE 1:	The reference architecture in figure 4.2.8.4-1 only shows the architecture and the network functions directly connected to Wireline 5G Access Network, and other parts of the architecture are the same as defined in clause 4.2.

NOTE 2:	The reference architecture in figure 4.2.8.4-1 supports service based interfaces for AMF, SMF and other NFs not represented in the figure.

NOTE 3:	The two N2 instances in Figure 4.2.8.4-1 apply to a single AMF for a 5G-RG which is simultaneously connected to the same 5G Core Network over 3GPP access and Wireline 5G Access Network.

NOTE 4	The two N3 instances in Figure 4.2.8. 4-1 may apply to different UPFs when different PDU Sessions are established over 3GPP access and Wireline 5G Access Network.

NOTE 5	Roaming for wireline with the W-5GCAN is not specified in this release.

Editor’s note: architecture for UE behind RG to be added

Editor’s note: Roaming architecture for wireline with W-5GBAN  is FFS

Figure 4.2.8.4-2: Architecture for supporting the FN-RG

The FN-RG can be connected to the 5GC via W-5GAN. The wireline core within the W-5GAN is shown for information. Interfaces between the wirelines core and the 5GC are outside the scope of this specification. The interface between the FN-RG and the wireline AN is defined by the BBF in the case of an FN-BRG and CableLabs in the case of FN-CRG. The wireline AN and wireline core, and associated interfaces, are defined by the BBF in the case of an FN-BRG and CableLabs in the case of FN-CRG.

The W-AGF provides the N1 interface to the AMF on behalf of the FN-RG.

Figure 4.2.8.4-3: Architecture for the 5G-capable UE behind a 5G-RG using Broadband Wireline access

The IWF and associated non-underlined N1, N2, N3 and N6 reference points support the 5G capable UE behind the 5G-RG. The underlined reference points in Figure 4.2.8.4-3 support the 5G-RG.

Figure 4.2.8.4-4: Architecture for the 5GC-capable UE behind a FN-RG using Broadband Wireline access

The IWF and associated non-underlined N1, N2, N3 and N6 reference points support the 5G capable UE behind the 5G-RG. The underlined reference points in Figure 4.2.8.4-4 support the FN-RG.

******************************* end  1st changes *********************