3GPP TSG-SA2 Meeting #131	S2-1902018

# February 25 – March 1, 2019, Santa Cruz, Tenerife, Spain

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1004 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing QoS differentiation for access to PLMN services via non-public networks and vice versa |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Charter Communications, Orange, China Telecom, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-02-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is still FFS for support of QoS differentiation for access to PLMN services via non-public networks and vice versa. In order to support "Accessing PLMN services via SNPN networks and vice versa", the N3IWF based architecture was reused. The N3IWF is used for N3GPP before, the QoS is not guaranteed by N3GPP, but QoS can be guaranteed by the SNPN. The UE’s PLMN PDU session and PLMN NAS signalling are carried over the IPsec SA over Nwu interface. And the related IPsec SA data will be transmitted over the SNPN PDU session.The SNPN will be transparent to the PLMN’s NAS signalling and user plane data and QoS differentiation within the SNPN for PLMN’s services (e.g. IMS voice) is unspecified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing QoS differentiation for access to PLMN services via non-public networks and vice versa |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of QoS differentiation for access to PLMN services via non-public networks and vice versa |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.2.7, 5.X.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes >>>

5.X	Support for non-public networks

5.X.2.7	Access to PLMN services via stand-alone non-public networks

A UE in SNPN mode of operation may access PLMN services following the same architectural principles as specified in clause 4.2.8 and the SNPN taking the role of “Untrusted non-3GPP access”.

When a UE accesses PLMN services via a SNPN,

- Upon reception of the signalling IPsec SA setup (i.e, for NAS signalling) from the N3IWF of the PLMN, the UE shall request the SNPN to setup a QoS flow solely used for the signalling IPsec SA. The UE shall provide to the SMF of the SNPN the SPI of the signalling IPsec SA as the packet filter of the requested QoS flow and the priority in the requested QoS flow description as the highest priority in the PDU session.

- Upon reception of the IPsec child SA setup (i.e, for user plane data) from the N3IWF of the PLMN, the UE shall request SNPN to setup the QoS flow for each IPsec child SA. The UE shall provide to the SMF of the SNPN the requested QoS flow description based on the QoS flow description (e.g, QFI(s)) associated with the IPsec child SA and the SPI of the IPsec child SA as the packet filter of the requested QoS flow.

5.X.2.8	Access to stand-alone non-public network services via PLMN

A UE may access SNPN services following the same architectural principles as specified clause 4.2.8 and the PLMN taking the role of “Untrusted non-3GPP access”.

When a UE accesses SNPN services via a PLMN,

- Upon reception of the signalling IPsec SA setup (i.e, for NAS signalling) from the N3IWF of the SNPN, the UE shall request the PLMN to setup a QoS flow solely used for the signalling IPsec SA. The UE shall provide to the SMF of the PLMN the SPI of the signalling IPsec SA as the packet filter of the requested QoS flow and the priority in the requested QoS flow description as the highest priority in the PDU session.

- Upon reception of the IPsec child SA setup (i.e, for user plane data) from the N3IWF of the SNPN, the UE shall request PLMN to setup the QoS flow for each IPsec child SA. The UE shall provide to the SMF of the PLMN the requested QoS flow description based on the QoS flow description (e.g, QFI(s)) associated with the IPsec child SA and the SPI of the IPsec child SA as the packet filter of the requested QoS flow.

<<< End of changes >>>