SA WG2 Meeting #131	S2-1902027

25 February - 01 March 2019, Tenerife, Spain

Source:	Samsung

Title:	Corrections for Solution #4

Document for:	Agreement

Agenda Item:	6.28

Work Item / Release:	FS_RACS / Rel-16

Abstract of the contribution:

# Discussion

Some editorial changes are proposed, and an editor’s note is removed. In addition some text is proposed to be added to the evaluation section to say that at step 4/5 of the procedure described in 6.12.3.2 the AF (if owned by the operator) could allocate a PLMN-specific UE capability ID if the ID provided by the UE is not known, or if no UE capability ID is provided by the UE. This would allow the AF to act as a centralised NF for PLMN-specific UE capability ID management and storage.

# Proposal

The following changes are proposed to TR 23.743.

<<<< Start of changes >>>>

## 6.12	Solution #12: Bulk Provisioning of UE Radio Capability from AF

### 6.12.1	Introduction

The solution addresses key issue #2. It proposes that the AF provisions to the AMF via NEF the representation of UE radio capability (e.g., UE Radio Capability ID) and corresponding UE radio capabilities. In this solution, UE Radio Capability Information is composed of the UE Radio Capability and corresponding UE Radio Capability ID. The NEF provisions the UE Radio Capability Information to the AMFs in the PLMN by using bulk provisioning method. The AMF can retrieve the UE Radio Capability information from AF via NEF if AMF has no UE Radio Capability Information stored.

### 6.12.2	Functional Description

This solution has the following characteristics:

-	The AF is integrated with the data base for UE Radio Capability Information, in which the information can be provided by UE manufacture or operator. The AF is able to provision UE Radio Capability Information (UE Radio Capability ID and corresponding UE Radio Capabilities) to the AMF(s). The AMF can trigger the API for UE Radio Capability provisioning if the UE Radio Capability information is updated.

-	The NEF exposes API for UE Radio Capability provisioning to the AF. The NEF authorizes that the AF provisions the UE Radio Capability Information.

-	The NEF retrieves AMF address from the NRF.

-	The NEF performs bulk provisioning to all AMFs in the PLMN.

-	The AMF exhibits service operation for UE Radio Capability Provisioning. Consumer of this service operation is NEF.

-	The AMF can retrieve the UE Radio Capability information from AF via NEF if AMF has no UE Radio Capability Information stored.

### 6.12.3	Procedures

#### 6.12.3.1	Bulk provisioning of UE Radio Capability

In this clause, the procedure for bulk provisioning of UE Radio Capability is described.

Figure 6.12.3-1: Bulk Provisioning of UE Radio Capability information to AMFs

1.	The AF triggers Nnef_RadioCapability_Provisioning Request service operation to the NEF. The AF includes UE Radio Capability Information in this message. The UE Radio Capability information is composed of UE Radio Capabilities and the corresponding identifier (e.g., UE Radio Capability ID). The AF can include multiple sets of UE Radio Capability Information.

If CAPIF is not supported, the AF is locally configured with the API termination points for this service operation. If CAPIF is supported, the AF obtains the service API information from the CAPIF core function as specified in TS 23.222 [4].

2.	The NEF authorized the AF request based on the operator policy.

3.	The NEF acknowledges the execution of Nnef_RadioCapability_Provisioning_request. The NEF stores the received UE Radio Capability Information in order to provision to AMFs in the PLMN after invoking NF discovery service operation. The NEF may cache the UE Radio Capability Information provisioned by the AF.

4.	The NEF invokes Nnrf_NFDiscovery_Request to the NRF. In this message, the NEF indicates NF Type as AMF. The NEF includes the expected NF service name for the provisioning request (i.e., Namf_Provisioning service operation).

5.	The NRF authorizes the Nnrf_NFDiscovery_Request. If allowed, the NRF provides the set of address of discovered AMFs to the NEF via Nnrf_NFDiscovery_Request Response message. The NEF stores the set of address of AMFs.

The NEF can register with the NRF for any newly registered AMF by invoking Nnrf_NFStatus_Subscribe. If NEF gets notified from the NRF for the newly registered AMF in the PLMN, the NEF can trigger Namf_Provisioning_Request service operation in order to provide the cached UE Radio Capability Information to the AMF.

6.	Based on the information of the discovered AMFs in the PLMN from step 5, the NEF invokes bulk Namf_Provisioning_Request to discovered AMFs in order to provision UE Radio Capability Information. In this message, one or set of UE Radio Capability Information received from the step 3 are included.

NOTE:	Namf_Provisioning service operation is not per UE operation.

7.	The AMF stores the received UE Radio Capability Information. The AMF acknowledges to the NEF by sending Namf_Provisioning_Response. The AMF uses the UE Radio Capability Information to identify UE Radio Capability when UE requests registration with UE Radio Capability ID.

8.	The NEF may notify to the AF that the UE Radio Capability Information is successfully provisioned to the all AMF in the PLMN, based on the local policy.

If the AF is trusted in the PLMN and allowed to invoke Namf_Provisioning service operation, the AF can directly use Nnrf service operation to discover AMFs and can directly use Namf service operation to provision the UE Radio Capability Information.

#### 6.12.3.2	On-demand UE Radio Capability Information Retrieval from AF

This section applies in the case that the AMF has no UE Radio Capability Information or cannot derive UE capabilities according to the capability ID received from the UE (e.g. the database is not up to date). The AMF acquires UE Radio Capability Information from the AF via NEF. This procedure ensures the AMF always has up to date UE Radio Capability information.

Figure 6.12.3-2: On-demand UE Radio Capability from AF

1.	UE provides UE capability ID and AF information e.g. manufacture information or PLMN information to the AMF.

2.	AMF receives the UE capability ID and the AF information, e.g. manufacture information, and obtains UE capabilities according to the stored UE Radio Capability Information for the received UE Capability ID for this AF.

3.	If AMF has no UE Radio Capability Information for the AF information or cannot derive UE capabilities according to the capability ID received from the UE, the AMF acquires UE Radio Capability information from the AF via NEF. The AMF requests from the NEF UE Radio Capability Information, and if the NEF has the UE Radio Capability Information, then NEF responds to the AMF.

Regardless of step 1 and step 2, if AMF determines that the stored UE Radio Capability Information is not up to date then based on local configuration, the AMF can trigger step 3 to retrieve the up to date UE Radio Capability Information from the AF.

4-5.	If the NEF does not have the UE Radio Capability Information, the NEF requests the AF to provide the UE Radio Capability Information, and the AF provides the UE Radio Capability Information based on the request.

NOTE:	The NEF is expected to support CAPIF functions for external exposure if the AF is the third party.

6.	The NEF stores the newly provisioned UE Radio Capability Information and responses to the AMF.

7.	AMF based on the newly provisioned UE Radio Capability Information, derives UE Radio Capability and transfers to the RAN if applicable.

### 6.12.4	Impacts on existing entities and interfaces

NEF: new API exposure for UE Radio Capability Information provisioning. The NEF can cache the UE Radio Capability Information received by the AF to provision to the AMF. The NEF can store the address of AMFs in the PLMN after querying to the NRF.

AMF: new Namf service to allow for requesting and provisioning of UE Radio Capability Information. AMF can trigger service operation to retrieve UE Radio Capability Information from the AF.

### 6.12.5	Evaluation

The AF provides the UE Radio Capability Information to the related AMFs via bulk provisioning method when the AF generates or updates the UE Radio Capability Information. This method decreases the O&M configuration cost for the operator. And on-demand retrieval from AF can provide the AMF up to date UE Radio Capability Information if the AMF has no such information. Otherwise, the AMF will retrieve UE explicit radio capabilities from the UE, which will cost extra Uu and N2 interface signalling.

Step 4/5 of the procedure described in 6.12.3.2 that, the AF (if owned by the operator) could allocate a PLMN-specific UE capability ID if the ID provided by the UE is not known, or if no UE capability ID is provided by the UE. This provides a centralised way for the operator to allocate PLMN-specific IE capability IDs that are unique.

The database of UE Radio Capability Information is part of the AF in this solution, Use of the AF for this could reduce the amount of signalling in the network as it enables to provision the information to all AMFs proactively.

<<<< End of changes >>>>