3GPP TSG-SA WG2 Meeting #131	S2-1902731

# Santa Cruz - Tenerife, Spain, 25 Feb - 1 Mar 2019	   was_S2-1902717_was S2-191687

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0942 | rev | 2 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transport Level Packet Marking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Perspecta Labs, OEC, AT&T, T-Mobile USA, Sprint, Ericsson, Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2019-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In EPC, transport level marking is performed on a per EPS bearer basis in the SGW and PGW. Transport level marking refers to the process of marking traffic with a DSCP value based on the locally configured mapping from the QCI and optionally ARP. For 5GC, transport level marking is defined to be done by the SMF and provided to the UPF for marking the DSCP value for the DL packets. However, the existing specification also suggests that the UPF can determine the transport level packet mapping on its own; but the UPF does not have the means to determine the DSCP value. In addition, unlike EPC, where QCI Priority Level has a one-to-one mapping with the QCI value, in 5G, when Priority Level is explicitly signalled or when dynamic 5QI is used, the Priority Level does not have a one-to-one relationship with 5QI. Therefore, the Priority Level should also be considered when determining the transport level packet marking. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified that the UPF uses the DSCP value provided by the SMF to mark the transport level packet marking in the DL on a QoS Flow basis. Removed the option for the UPF to determine on its own the DSCP value to be used. Clarified that the SMF uses Priority Level in addition to 5QI and ARP to determine the DSCP value to be provided to the UPF. Similarly, added that when the RAN determines the DSCP value for packet marking in the UL, it uses the Priority Level in addition to 5QI and ARP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect determination of DSCP value. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1.6, 5.7.1.7, 5.8.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 5.7.1.6	DL traffic

The following characteristics apply for processing of DL traffic:

-	UPF maps User Plane traffic to QoS Flows based on the PDRs.

-	UPF performs Session-AMBR enforcement and counting of packets for charging.

-	UPF transmits the PDUs of the PDU Session in a single tunnel between 5GC and (R)AN, the UPF includes the QFI in the encapsulation header. In addition, UPF may include an indication for Reflective QoS activation in the encapsulation header.

-	UPF performs transport level packet marking in DL on a per QoS Flow basis. The UPF uses the transport level packet marking value provided by the SMF (as described in clause 5.8.2.7).

-	(R)AN maps PDUs from QoS Flows to access-specific resources based on the QFI and the associated 5G QoS profile, also taking into account the N3 tunnel associated with the DL packet.

NOTE:	Packet Filters are not used for the mapping of QoS Flows onto access-specific resources in (R)AN.

-	If Reflective QoS applies, the UE creates a new derived QoS rule as defined in clause 5.7.5.2.

#### 5.7.1.7	UL Traffic

Following characteristics apply for processing of UL traffic:

-	UE uses the stored QoS rules to determine mapping between UL User Plane traffic and QoS Flows. UE marks the UL PDU with the QFI of the QoS rule containing the matching Packet Filter and transmits the UL PDUs using the corresponding access specific resource for the QoS Flow based on the mapping provided by (R)AN. For NG-RAN, the UL behaviour is specified in TS 38.300 [27] clause 10.5.2.

-	(R)AN transmits the PDUs over N3 tunnel towards UPF. When passing an UL packet from (R)AN to CN, the (R)AN includes the QFI value, in the encapsulation header of the UL PDU, and selects the N3 tunnel.

-	(R)AN performs transport level packet marking in the UL on a per QoS Flow basis with a transport level packet marking value that is determined  based on the 5QI, the Priority Level (if explicitly signalled) and the ARP priority level of the associated QoS Flow. When applicable, (R)AN uses the transport level packet marking value (e.g. the DSCP value) if it is provided by the SMF during the PDU Session Establishment/Modification.

-	UPF verifies whether QFIs in the UL PDUs are aligned with the QoS Rules provided to the UE or implicitly derived by the UE in the case of Reflective QoS).

-	UPF and UE perform Session-AMBR enforcement and the UPF performs counting of packets for charging.

* * * Second Change * * *

#### 5.8.2.7	PDU Session and QoS Flow Policing

ARP is used for admission control (i.e. retention and pre-emption of the new QoS Flow). The value of ARP is not required to be provided to the UPF.

For every QoS Flow, the SMF shall determine the transport level packet marking value (e.g. the DSCP in the outer IP header) based on the 5QI, the Priority Level (if explicitly signalled) and optionally, the ARP priority level, and provide the transport level packet marking value to the UPF.

The SMF shall provide the Session-AMBR value of the PDU Session to the UE to the UPF so that the UPF and the UE can enforce the Session-AMBR of the PDU Session across all Non-GBR QoS Flows of the PDU Session.

SMF shall provide the GFBR and MFBR value for each GBR QoS Flow of the PDU Session to the UPF. SMF may also provide the Averaging window to the UPF, if Averaging window is not configured at the UPF or if it is different from the default value configured at the UPF.

* * * End Of Changes * * *