SA WG2 Meeting #S2-131	S2-1902916

25 February - 1 March, 2019, Santa Cruz - Tenerife	(e-mail revision 5 of S2-1902558)

Source:	Ericsson, KDDI, Convida Wireless

Title:	Network Performance Analytics

Document for:	Approval

Agenda Item:	 6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes to add the update the procedure to request Network Performance Analytics from NWDAF and the functional description.

# Discussion

There is a description on how to request Network Performance Analytics by a NF, however the following aspects were not so clear last SA2 meeting and then it was agreed to have more details on this as follows:

- A description of Network Performance Analytics is included to help the reader to decide when to use it and what input and output data is needed.

- Load information is retrieved from OAM, but the PM counters are not listed yet, so an Editor´s note is added.

- Clarifications on the Network Performance Analytics that aims to align with the Nnwdaf service description.

- The procedure is completed to retrieve input data and to provide analytics. A subscription based procedure is proposed, since the one time request/response is not studied so far and may not request retrieval of input data by NWDAF.

# Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

## 6.6	Network Performance Analytics

### 6.6.1	General

Network Performance Analytics provides either statistics or predictions on the load in an Area of Interest; in addition it may provide statistics or predictions on the number of UEs that are located in that Area of Interest.

The consumer of these analytics may indicate in the request:

-	AnalyticsId set to “Network Performance”.

-	The Area of Interest where the load at the time indicated in the Observation period is requested.

-	An Internal Group Identifier that refers to the UEs that belong to the Internal Group that are located in the Area of Interest at the time indicated in the Observation period.

-	An Observation period indicates the time when the statistics or prediction are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.6.3.

### 6.6.2	Input Data

The NWDAF collects Load information in an Area of Interest from the sources listed in Table 6.6.2-1 and location information of each of the UEs within an Internal Group Identifier from the sources listed in Table 6.10.2-1.

NOTE:	The table below shows the information provided to the NWDAF, in other words the information provided by NWDAF is not shown here but in the procedures in clause 6.6.4.

Table 6.6.2-1:  Load information collected by NWDAF

| Load information | Source | Description |
| --- | --- | --- |
| Area of interest | OAM | A list of TAi or Cell IDs |
| Load information | OAM | Statistics on load per TAI or per CellId |
| Load information | NRF | Load per NF |

### 6.6.3	Output Analytics

The NWDAF provides Network Performance Analytics to a consumer that includes the load in the area of interest and the number of UEs located in the area of interest or both at the time requested by the consumer in the Observation Period:

-	Analytics ID set to “Network Performance”.

-	Notification Target Address including the address of the consumer.

-	Notification Correlation Id, for the consumer to correlate notifications from NWDAF if subscription applies.

-	Analytic specific parameters including the load in the Area of Interest and the number of UEs in the Area of Interest, at the time indicated in the Observation period.

### 6.6.4	Procedures

Figure 6.6.4-1: Procedure for subscription to network performance analytics

- The NF sends Nnwdaf_AnalyticsSubscription_Subscribe Request (AnalyticsId= ”Network Performance”, AnalyticsFilter=”Area of interest” or “Area of interest” and “Internal Group Identifier”, Observation Period(s)) to the NWDAF.

2.	The NWDAF subscribes to NF load information from NRF.

3.	The NWDAF subscribe to OAM to get the load information on the Area of Interest  and OAM provide back the first report for requested information in the response.

Editor's note:	The exact SA5 service and counters that are used are FFS.

4.	The NWDAF derives the number of UEs located in the Area of Interest at the time indicated in the Observation period, input data from AMF is requested using Namf_EventExposure service including the Internal Group Identifier.

5.	The NWDAF sends Nnwdaf_AnalyticsSubscribe_Response (one or more instances of the n-tuple (Load in Area of Interest and number of UEs, timestamp) SubscriptionCorrelationId, Probability of assertion)

6.	A change of network performance information, i.e. change in the load in the area of interest at the observed period is detected by OAM, or a change in the NF load information is reported by NRF, and is notified to NWDAF.

7.	The NWDAF derives new analytics.

The NWDAF provides a notification using Nnwdaf_AnalyticsNotify_Response (one or more instances of the n-tuple (Load in Area of Interest and number of UEs, timestamp) SubscriptionCorrelationId, Probability of assertion

* * * End of changes * * * *