SA WG2 Meeting #S2-131	S2-1902930

25 February - 1 March, 2019, Santa Cruz - Tenerife	(e-mail revision 2 of S2-1902899)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0873 | rev | 8 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eSBA communication schema co-existence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBA |  |  |  |  |  |  | Date: |  | 2019-01-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | FS_eSBA |  |  |  |  |  |  |  |  |
| Summary of change: |  | eSBA communication modes coexistence Rev 3/4 updates: - Terminology updates – NF/NF Service - Clarification of the address (i.e. URI) used.. |
| Consequences if not approved: |  | Coexistence and backward compatibility fails |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Implement as subsection of 6.3.1 |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 6.3.1.x	NF Discovery and Selection aspects relevant with indirect communication

For indirect communication shown in Annex X, the SCP performs the following functionalities regarding Network Function and Network Function Service discovery and selection:

-	If the end point address of a request sent by a NF service consumer points to a specific target NF service producer instance (e.g. the request URI points to a specific target NF service producer instance, the SCP resolves the URI to a specific target instance), the SCP shall route the service request to the requested target unless the target is not available (e.g. in the case of instance failure, operator configuration).

-	If the end point address of a request sent by a NF service consumer points to an NF Set or NF Service Set (e.g. the request URI points to the Set), the SCP shall select an NF service producer instance from the Set pointed to by the end point address and route the service request to the selected target NF service producer instance but within the set.

Editor's note:	This text of this bullet needs to be updated depending on the outcome of the NF set discussion.

-	Otherwise, if the end point address sent by a NF service consumer points to a specific target NF service type of a NF type (i.e. delegated discovery is employed), the SCP shall discover and select a specific NF service producer instance matching the specific target.

*** END CHANGES ***