3GPP TSG-SA WG2 Meeting #132	S2-1902983

8 - 12 Apr 2019, Xi'an, China

Source:	ORANGE, Huawei

Title:	global or UE specific Mobility analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes alternative procedures to compute mobility analytics.It is based on the global mobility matrix of all UEs. Two techniques a proposed to build such matrix. For the second alternative, a new “mobility matrix” exposed event should also be defined in 23.501 and 23.502.

# Discussion

NWDAF supporting individual UE mobility statistics or predictions at stated in §6.10 shall be able to collect individual UE mobility related information from 5GC, OAM and/or AFs, and to perform data analytics to provide UE mobility statistics or predictions.

Such technique results in high accuracy of prediction, but may result into high signalling load if an important number of UEs are monitored.

On the other hand, lower accuracy may be sufficient in many cases, based on the general behaviour of all UEs (e.g. traffic jam, peak hours, social event etc).

Therefore an alternate proposal is put forward which estimates the location of a given UE with lower accuracy, basing on the location matrix estimation of all UEs. This location matrix estimation is obtained with lower signalling, based either on:

- A) Collection of mobility events from a sample of UEs

- B) Direct upload of a mobility matrix from the AMF

As this analytics may be of general interest, it is proposed to define it appart in a specific chapter 6.X. 
In a second step, the chapter §6.10 makes a reference to the 6.X data collection techniques A/B as possible options in complement to the existing procedures.

# Proposal

The following changes are proposed in TS 23.288.

* * * First change (all new text) * * * *

## 6.X Global mobility analytics

### 6.X.1	General

NWDAF supporting UE mobility statistics or predictions shall be able to collect global UE mobility related information from 5GC, and to perform data analytics to provide global UE mobility statistics or predictions.

The consumer of these analytics may indicate in the request:

-	AnalyticsId set to "UE global mobility matrix".

-	The Area of Interest for the source Area

-	The Target of Analytic Reporting can be any UE This target refers to the UEs that are presumed to be located in the Area of Interest at the time indicated in the Observation period.

-	An Observation period indicates the time when the statistics or prediction are requested.

### 6.X.2	Input Data

The NWDAF supporting data analytics on UE mobility shall be able to collect UE mobility information from AMF.

Two following two procedures may be used:

collection of timestamped mobility related events based on Event Exposure for a representative subset of UE for each AMF. The subset shall be requested using the sampling ratio parameter of the Event Reporting Information as stated in TS 23.502) targeting “any UE”. The table 6.X-2-1 describes which information elements are collected from the AMF.

collection of periodical timestamped mobility matrix based on Event Exposure for any UE of each AMF instance.  The table 6.X-2-2 describes which information elements are collected from the AMF.

Table 6.X.2-1: Individual location report information collected from AMF for a sample of UEs

| Information | Source | Description |
| --- | --- | --- |
| UE IDs | AMF | SUPI |
| UE location | AMF | UE positions |
| >UE location |  | Updated location (TAI, Cell-ID for 3GPP access, most recent N3IWF node) |
| >Timestamp |  | A time stamp when the AMF detects the UE enters this location |

Table 6.X.2-2: Mobility matrix collected from AMF

| Information | Source | Description |
| --- | --- | --- |
| Mobility entry (1..n) | AMF | List of mobility entries in the mobility matrix |
| >TAI or Cell source |  | Source location |
| >TAI or Cell destination |  | Destination location |
| >Number of UEs |  | Number of UEs moving from source to destination |

Editor's note:	Such mobility matrix requires extension of the Exposure framework for the AMF with a new eventID.

### 6.X.3	Output Analytics

The NWDAF supporting data analytics on UE global mobility shall be able to provide UE global mobility analytics to consumer NFs or AFs. The analytic results provided by the NWDAF are UE global mobility statistics or predictions at the time indicated in the Observation period:

-	UE mobility statistics is defined in the Table 6.X.3-1.

Table 6.X.3-1: UE mobility statistics

| Information | Description |
| --- | --- |
| Mobility entry (1..n) | List of mobility entries in the mobility matrix |
| >TAI or Cell source | Source location |
| >TAI or Cell destination | Destination location |
| >Number of UEs | Number of UEs moving from source to destination measured during the Observation period |

-	UE mobility predictions is defined in the Table 6.X.3-2.

Table 6.10.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| Mobility entry (1..n) | List of mobility entries in the mobility matrix |
| >TAI or Cell source | Source location |
| > TAI or Cell destination | Destination location |
| > Number of UEs | Number of UEs moving from source to destination forecasted during the Observation period |
| Confidence | Confidence of this prediction |

The source location is a subset of the Area of Interest provided as parameter in the request or subscription.

### 6.X.4	Procedures

2c-Nafm_EventExposure_Subscribe
(UE mobility matrix)

NF

NWDAF

1-Nnwdaf_AnalyticsInfo_Request

/ Nnwdaf_AnalyticsInfo_Subscribe

AMF

2a-Nafm_EventExposure_Subscribe 
(locations of UEs (sample))

2b-Namf_EventExposure_Notify

4-Nnwdaf_AnalyticsInfo_Response

/ Nnwdaf_AnalyticsInfo_Notify

3- NWDAF derives analytics

6- Nnwdaf_AnalyticsInfo_Notify

5- NWDAF derives new analytics

2d-Namf_EventExposure_Notify

Figure 6.X.4-1: Procedure for subscription to global mobility analytics

1.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe Request (AnalyticsId="Global mobility Analytics", AnalyticsFilter="Area of interest", Observation Period(s)) to the NWDAF.

2ab.	the NWDAF may subscribe to the location of a subset of UEs from AMFs using Namf_EventExposure service, including a sampling ratio provided as an input parameter. This step can be performed at the initiative of the NWDAF as a permanent data collection.

2cd.	the NWDAF may subscribe to the mobility matrix from AMFs using Namf_EventExposure service. This step can be performed at the initiative of the NWDAF as a permanent data collection.

4.	The NWDAF sends Nnwdaf_AnalyticsSubscribe_Response containing the mobility matrix.

5.	The NWDAF derives new analytics based on the most recent data collected.

6.	The NWDAF provides a notification using Nnwdaf_AnalyticsNotify_Response.

* * * Second change * * * *

## 6.10 UE mobility analytics

### 6.10.1	General

NWDAF supporting UE mobility statistics or predictions shall be able to collect UE mobility related information from 5GC, OAM and/or AFs, and to perform data analytics to provide UE mobility statistics or predictions.

The consumer of these analytics may indicate in the request:

-	The Target of Analytic Reporting which is a single UE.

-	An Observation period indicates the time when the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics (low/high)

### 6.10.2	Input Data

The NWDAF supporting data analytics on UE mobility shall be able to collect UE mobility information from OAM, 5GC and AFs. The detailed information collected by the NWDAF could be MDT data from OAM, network data from 5GC and/or service data from AFs:

-	UE mobility information from OAM is UE location carried in MDT data;

-	Network data related to target UE mobility from 5GC is UE individual location information as defined in the Table 6.10.2-1;

Table 6.10.2-1: UE location information collected from 5GC

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| UE locations (1..max) | AMF | UE positions |
| >UE location |  | TA or cells that the UE enters |
| >Timestamp |  | A time stamp when the AMF detects the UE enters this location |

-	Network data related to global UE mobility, as stated in §6.X UE global mobility analytics.

Such procedure may be used  at the initiative of the NWDAF for predictions as a complement to other collected data, or if low level accuracy is tolerable or in case of high signalling load. In such cases, the prediction of UE future locations shall be based partly of totally on the global mobility prediction.

- 	Service data related to UE mobility provided by AFs is defined in the Table 6.10.2-2;

Table 6.10.2-2: Service Data from AF related to UE mobility

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AF | Could be GPSI or external UE ID |
| Application ID | AF | Identifying the application providing this information |
| UE trajectory (1..max) | AF | Timestamped UE positions |

Editor's note:	The exact information carried in the UE trajectory is FFS.

### 6.10.3	Output Analytics

The NWDAF supporting data analytics on UE mobility shall be able to provide UE mobility analytics to consumer NFs or AFs. The analytic results provided by the NWDAF could be UE mobility statistics, UE mobility predictions:

-	UE mobility statistics is defined in the Table 6.10.3-1.

Table 6.10.3-1: UE mobility statistics

| Information | Description |
| --- | --- |
| UE ID | Internal UE ID |
| UE location (1..max) |  |
| >UE location | TA or cells which the UE stays |
| >Time slot | Time slot that the UE moves to this location |
| >Average Duration | Average time period that the UE stays in this location |
| >Frequency | Number of times that the UE moves to this location measured during the Observation period |

-	UE mobility predictions is defined in the Table 6.10.3-2.

Table 6.10.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| UE ID | Internal UE ID |
| Predicted UE location (1..max) |  |
| >UE location | TA or cells where the UE may move into |
| >Time slot | Time slot within which the UE may move to this location during the Observation period |
| >Duration | Time period that the UE may stay in this location during the Observation period |
| >Confidence | Confidence of this prediction |

* * * End of changes * * * *