SA WG2 Meeting #S2-132	S2-190xxxx

May 13-17 2019 Reno, Nevada	(revision of S2-1903007)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1128 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of RACS: UCMF services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Samsung, Nokia, Nokia Shanghai Bell, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2019-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of RACS: UCMF services |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Extends the NEF services with a new service for UCMF provisioning via the NEF. 2) Defines UCMF services for: - Provisioning of Manufacturer-assigned UE Radio Capability ID from the AF (either directly or via the NEF) - Resolving UE Radio Capability ID into UE radio access capability. - Storage of UE radio access capability in the UMCF and assignment of PLMN-assigned UE Radio Capability ID. - Subscription for UCMF dictionary updates. - Notification of creation or deletion of UCMF dictionary entries. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RACS feature will remain unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.1; 5.2.6.x (new); 5.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR1037 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*********** Start change **************

### 5.2.6	NEF Services

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | SMF |
|  | Trigger | Trigger | AF |
| Nnef_NIDD | Delivery | Request/Response | AF, SMF |
| Nnef_SMContext | Create | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request / Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |

*********** Next change **************

#### 5.2.6.x	Nnef_UCMFProvisioning service

##### 5.2.6.x.1	General

This service is for allowing external party to provision the UCMF with UCMF dictionary entries for Manufacturer-assigned UE Radio Capability IDs.

##### 5.2.6.x.2	Nnef_UCMFProvisioning_Create operation

Service operation name: Nnef_UCMFProvisioning_Create

Description: The consumer creates a UCMF dictionary entry for a Manufacturer-assigned UE Radio Capability ID via the NEF.

Inputs (required): UE Radio Capability ID, UE radio access capability.

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

##### 5.2.6.x.3	Nnef_UCMFProvisioning_Delete operation

Service operation name: Nnef_UCMFProvisioning_Delete

Description: The consumer deletes a UCMF dictionary entry for a Manufacturer-assigned UE Radio Capability ID via the NEF.

Inputs (required): UE Radio Capability ID.

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

*********** Next change **************

### 5.2.x	UCMF Services

#### 5.2.x.1	General

The following table illustrates the UCMF Services.

Table 5.2.x.1-1: NF services provided by UCMF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Nucmf_Provisioning | Create | Request/Response | AF, NEF |
|  | Delete | Request/Response | AF, NEF |
| Nucmf_UECapabilityManagement | Resolve | Request/Response | AMF |
|  | Store | Request/Response | AMF |
|  | Subscribe | Subscribe/Notify | AMF |
|  | Unsubscribe | Subscribe/Notify | AMF |
|  | Notify | Subscribe/Notify | AMF |

#### 5.2.x.2	Nucmf_Provisioning service

##### 5.2.x.2.1	Nucmf_Provisioning_Create service operation

Service operation name: Nucmf_Provisioning_Create.

Description: The consumer creates a UCMF dictionary entry for a Manufacturer-assigned UE Radio Capability ID.

Inputs (required): UE Radio Capability ID, UE radio access capability.

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

##### 5.2.x.2.2	Nucmf_Provisioning_Delete service operation

Service operation name: Nucmf_Provisioning_Delete.

Description: The consumer deletes a UCMF dictionary entry for a Manufacturer-assigned UE Radio Capability ID.

Inputs (required): UE Radio Capability ID.

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

#### 5.2.x.3	Nucmf_UECapabilityManagement Service

##### 5.2.x.3.1	Nucmf_UECapabilityManagement Resolve service operation

Service Operation name: Nucmf_UECapabilityManagement_Resolve

Description: Consumer NF gets the UE radio access capability corresponding to a specific UE Radio Capability ID (either Manufacturer-assigned or PLMN-assigned).

Inputs, Required: NF ID, UE Radio Capability ID.

Inputs, Optional: None.

Outputs, Required: UE radio access capability, UCMF Restart Counter.

Outputs, Optional: None.

##### 5.2.x.3.2	Nucmf_UECapabilityManagement_Store service operation

Service or service operation name: Nucmf_UECapabilityManagement_Store

Description: The NF consumer stores the UE radio access capability in the UCMF and obtains a PLMN-assigned UE Radio Capability ID in return.

Inputs, Required: UE radio access capability, TAC.

Inputs, Optional: None.

Outputs, Required: UE Radio Capability ID, UCMF Restart Counter. .

Outputs, Optional: None.

##### 5.2.x.3.3	Nucmf_UECapabilityManagement_Subscribe service operation

Service operation name: Nucmf_UECapabilityManagement_Subscribe

Description: The NF consumer subscribes for updates to UCMF dictionary entries. The UCMF shall check the requested consumer is authorized to subscribe to requested updates.

Inputs, Required: None.

Inputs, Optional: None.

Outputs, Required: UCMF Restart Counter, List of TACS s for which UE manufacturer-assigned UE Radio Capability ID is used.

Outputs, Optional: None.

##### 5.2.x.3.4	Nucmf_UECapabilityManagement_Unsubscribe service operation

Service operation name: Nucmf_UECapabilityManagement_Unsubscribe

Description: The NF consumer unsubscribes from updates to UCMF dictionary entries.

Inputs, Required: None.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

##### 5.2.x.3.5	Nucmf_UECapabilityManagement_Notify service operation

Service Operation name: Nucmf_UECapabilityManagement_Notify

Description: Producer NF provides notifications about creation or deletion of UCMF dictionary entries to subscribed consumer NF.

Inputs, Required: One or more UCMF dictionary entries, each UCMF dictionary entry consisting of a UE Radio Capability ID and the corresponding UE radio access capability.

Inputs, Optional: List of TACs for which UE manufacturer-assigned UE Radio-capability ID is used, List of TACs for which UE manufacturer-assigned UE Radio-capability ID shall no longer be used, UCMF new restart counter.

Outputs, Required: None.

Outputs, Optional: None.

*********** End change **************