3GPP TSG-SA2 Meeting #133	S2-1905017

# Reno, Nevada, United States, 13th May 2019 - 17th May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1319 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to downlink data delivery status event to support PCC and HR roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA2 has received LS S2-1904878 from CT3. This LS asks with respect to the "Information flow for downlink data delivery status" in subclause 4.15.3.2.5 of TS 23.502 how the SMF detects the downlink delivery status. There are several related issues with the present information flow: - Packet filters related with downlink data delivery status imply UPF interactions (N4 impacts) to configure the UPF to buffer packets or forward them to the SMF and to provide notifications about the downlink data delivery status when the first packet was transmitted or is being buffered and subsequently discarded. Related procedural updates are provided in TS 23.501, see S2-1905016. - Traffic filters in the UPF have priorities that need to be coordinated with other installed traffic filters. This is controlled by the PCF via PCC rules. The subscription for downlink data delivery status thus also needs to be integrated in PCC rules. The subscription to the Nsmf_EventExposure notification is thus indirect via the Npcf_SMPolicyControl service. - For HR roaming, the SMF that is aware of data being buffered is the V-SMF and the notifications corresponding to data being buffered originate from a V-UPF in the VPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Interactions of the SMF with the UPF for subscribing to downlink data delivery status are added to subclause 4.15.3.2.5. - The subscription to the Nsmf_EventExposure notification is indirect via PCC rule information of the Npcf_SMPolicyControl service. - For the home routed roaming case, the V-SMFF and an attached UPF detect downlink data delivery status events and the V-SMF sends related notifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The detection of downlink data delivery status event does not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.501 CR 1295 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 4.15.3.2.5	Information flow for downlink data delivery status

The procedure is used by the Application Function to subscribe to data delivery status notifications and to explicitly cancel a previous subscription. Cancelling is done by sending Nnef_EventExposure_UnSubscribe request identifying Subscription Correlation ID. The notification steps 8 to 10 are not applicable in cancellation case.

Figure 4.15.3.2.5-1: Information flow for downlink data delivery status

1.	AF sends Nnef_EventExposure_Subscribe Request to NEF requesting data delivery status for a UE or group of UEs. Traffic filter information is included in the message to identify the source of the downlink IP or Ethernet traffic. The Downlink delivery status events include:

-	Downlink Packet(s) in extended buffering event.

-	This event is triggered every time a new downlink data packet is buffered with extended buffering matching the traffic filter information.

-	Downlink Packet(s) discarded.

-	This event occurs when Extended Buffering time, as determined by the SMF, expires or the amount of downlink data to be buffered is exceeded.

-	Downlink Packet(s) transmitted.

-	This event occurs when the UE of the PDU Session becomes ACTIVE, and buffered data can be delivered to UE as per clause 4.2.3.3.

2.	NEF sends the Npcf_EventExposure_Subscribe Request to PCF. Identifier of the UE or group of UEs, traffic filter information, monitoring event received AF at step 1, and NEF notification endpoint are included in the message.

3.	PCF sends the PCC rule information including the received traffic filter and a request for notifications about Downlink delivery status via the Npcf_SMPolicyControl_Update  message to the SMF(s) which serve the UE(s) identified in step 2  and includes the notification endpoint of NEF. (implicit subscription to Notifications of the Nsmf_EventExposure service)

4a	(not depicted) In the case of home-routed roaming, the H-SMF creates a QoS Flow corresponding to the PCC rule information received from the PCF and exchanges related signalling with the V-SMF.

4.	The SMF (in the no-roaming case the H-SMF. in the roaming case the V-SMF) configures the relevant UPF as per clause 5.8.3.2 in TS 23.501 [2] to buffer packets or forward them to the SMF and to provide notifications about the downlink data delivery status when the first packet was transmitted or buffered and then subsequently discarded.

5.	The SMF sends the Npcf_SMPolicyControl_Update Response message to PCF.

6.	PCF send sends the Npcf_EventExposure_Subscribe response message to NEF.

7.	NEF sends the Nsmf_EventExposure_Subscribe response to AF.

8.	The SMF or UPF detects a change in Downlink Delivery Status event as described in subclause 5.8.3.2 in TS 23.501 [2] . If the UPF detects the change, the UPF repoprts that to the SMF.

9.	The SMF sends the Nsmf_EventExposure_Notify with Downlink Delivery Status event message to NEF.

10.	The NEF sends Nnef_EventExposure_Notify with Downlink Delivery Status event message to AF.