SA WG2 Meeting #133 	S2-1905096

# May 13 – 17, 2019 ; Reno, Nevada, USA	              (revision of S2-1905096)

Source:	Nokia, Nokia Shanghai Bell

Title:	ETSUN: list of FFS / open issues

Document for:	Endorsement

Agenda Item:	6.12

Work Item / Release:	ETSUN / Rel-16

Abstract of the contribution: ETSUN: list of FFS / open issues

# List of ETSUN FFS / open issues

References: 23.501 § 5.34 + 23.502 § 4.23

- Addition PDU Session Anchor and Branching Point or ULCL controlled by I-SMF: how many Local UPF(s) does the SMF consider?

  - 23.501 § 5.34.6 (e.g. Nokia CR 1179 was S2-1904312)

- agree SMF does not know how many local UPF are actually introduced by the I-SMF. The debate is on how many Local UPF(s) does the SMF consider?

- Spec update about this aspect only within 23.501 in 5.34.6.x

- Use of a Create or Update operation (e.g. Nokia 23.502 CR 1142)

  - the context fetch is used just to fetch the context, not to create a forwarding data path. An Update operation is used for the purpose of to create a forwarding data path.

In the following the New I-SMF could be a new VSMF (in a new VPLMN)

  - SR or N2 HO or Xn HO / I-SMF insertion (going from N11 to N16a):

    - AMF to ISMF = Create /

    - ISMF to SMF = Create /

  - SR or N2 HO or Xn HO / I-SMF change:

    - AMF to new ISMF = Create

    - New ISMF to SMF = Update

  - SR or N2 HO or Xn HO / I-SMF removal (going from N16a to N11):

    - AMF to SMF = Create

  - I-SMF insertion or relocation during Registration procedures (DCM Tdoc): correcting the Note is needed (check potential CR clash). But otherwise avoid spreading ETSUN related updates in message flows not within §4.23: just consider changes in 4.23 (dedicated ETSUN clause)

  - Xn HO / I-SMF removal is missing; (HW CR)

  - Context fetch from VSMF to VSMF in a different PLMN/ any issue?

- 23.502 CR to §5.2.7 needed? : impacts on NRF services of “the AMF gets from NRF information about the Service Area of SMF(s)”; (DCM CR)

- 23.501 §5.34.3: Clarify which parameters are (not) applicable for I-SMF selection taking into account feature interaction

- 23.502 § 4.23.3 (registration) reads “If the AMF does not have the service area of the SMF(s) received from old AMF, the AMF queries NRF to get service area information of the received SMF(s)”: the description of the service area of SMF(s) may be a bit bulky: do we really want to forward it over N14 AMF-AMF interface at each UE mobility?

inter AMF HO looks OK wrt this point

- 23.502 § 4.23.4.2 “UE Triggered Service Request without I-SMF change/removal”: Step 5a, the I-SMF notifies the Access type and UE location information to SMF. This does not apply to N16, and may induce useless signalling if not controlled (one of the usage of I-SMF is to shield SMF from frequent local events such as Access type change and IDLE-CONNECTED transitions)

At least not notify when Access Type not changed. Possible SMF can control what I-SMF is to report (e.g. based on PCRT conditions)

- Inter PLMN HO with VSMF change?

R15 supports inter PLMN inter AMF mobility and ETSUN may bring the complement

VPLMN to VPLMN only supported?

HPLMN – VPLMN needed?

- I-SMF can change /remove UL CL /BP (HW contribution planned)

- Solve EN how to release ISMF resources (align with CT4) (HW contribution planned)

If all points above complete ETSUN can go to maintenance mode.

# Proposal

It is proposed to endorse that ETSUN is complete at ~ 90% and can go to maintenance mode with following SA2 meeting TU requirement

- June: 0.25 TU

- October: 0.5 TU

- November: 0.25 TU