3GPP TSG-SA WG2 Meeting #133	S2-1905360

# Reno, USA, 13th May – 17th May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1379 | rev |  | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for access to PLMN services via SNPN and SNPN services via PLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is concluded that to access PLMN services, a UE in SNPN access mode that has successfully registered with an SNPN may perform another registration via the SNPN with a PLMN (using the credentials of that PLMN) with the SNPN taking the role of un-trusted non-3GPP access and vice versa for access to stand-alone Non Public Network services. It is not obvious from the normative text the architecture represented by the two networks and access to services supported on target network when connected on a source network. The annex proposed here clarifies the inter-network architecture when registered on one network and access services over another network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Representation of architecture between stand-alone Non-Public Network and PLMN is included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in the specification for access to PLMN services via SNPN and SNPN services via PLMN could lead to different implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.7,5.30.2.8,Annex D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Start of Changes *****

#### 5.30.2.7	Access to PLMN services via stand-alone non-public networks

To access PLMN services, a UE in SNPN access mode that has successfully registered with an SNPN may perform another registration via the SNPN user plane with a PLMN (using the credentials of that PLMN) following the same architectural principles as specified in clause 4.2.8 and the SNPN taking the role of "Untrusted non-3GPP access". Annex D.X provides additional details.

Editor's note:	The need to support QoS differentiation for access to PLMN services via non-public networks is FFS.

#### 5.30.2.8	Access to stand-alone non-public network services via PLMN

To access SNPN services, a UE that has successfully registered with a PLMN may perform another registration via the PLMN user plane with an SNPN (using the credentials of that SNPN) following the same architectural principles as specified in clause 4.2.8 and the PLMN taking the role of "Untrusted non-3GPP access".Annex D.X provides additional details.

Editor's note:	The need to support QoS differentiation for access to SNPN services via PLMN is FFS.

****Next Change****

Annex D (informative):
5GS support for Non-Public Network deployment options

# D.1	Introduction

This annex provides guidance on how 5GS features and capabilities can be used to support various Non-Public Network deployment options.

# D.2	Support of Non-Public Network as a network slice of a PLMN

The PLMN operator can provide access to an NPN by using network slicing mechanisms. The following are some considerations in such a case:

1.	The UE has subscription and credentials for the PLMN;

2.	The PLMN and NPN service provider have an agreement of where the NPN Network Slice is to be deployed (i.e. in which TAs of the PLMN and optionally including support for roaming PLMNs);

3.	The PLMN subscription includes support for Subscribed S-NSSAI to be used for the NPN (see clause 5.15.3);

4.	The PLMN operator can offer possibilities for the NPN service provider to manage the NPN Network Slice according to TS 28.533 [79].

5.	When the UE registers the first time to the PLMN, the PLMN can configure the UE with URSP including NSSP associating Applications to the NPN S-NSSAI (in case the UE also is able to access other PLMN services);

6.	The PLMN can configure the UE with Configured NSSAI for the Serving PLMN (see clause 5.15.4);

7.	The PLMN and NPN can perform a Network Slice specific authentication and authorization using additional NPN credentials;

8.	The UE follows the logic as defined for Network Slicing, see clause 5.15;

9.	The network selection logic, access control etc are following the principles for PLMN selection; and

10.	The PLMN may indicate to the UE that the NPN S-NSSAI is rejected for the RA when the UE moves out of the coverage of the NPN Network Slice. However, limiting the availability of the NPN S-NSSAI would imply that the NPN is not available outside of the area agreed for the NPN S-NSSAI, e.g. resulting in the NPN PDU Sessions being terminated when the UE moves out of the coverage of the NPN Network Slice. Similarly access to NPN DNNs would not be available via non-NPN cells.

# D.X Support for access to PLMN services via Stand-alone Non-Public Network and Stand-alone Non Public Network services via PLMN

Figure D.3-1: Access to PLMN services via Non-Public Network

NOTE1: The reference architecture in Figure D.3-1 and Figure D.3-2 only shows the network functions directly connected to the UPF or N3IWF and other parts of the architecture are same as defined in TS 23.501 clause 4.2.

The UE performs a PDU Session(s) handover to achieve service continuity between stand-alone Non-Public Network and PLMN as described in clause 4.9.2 TS 23.502 [3].For access to PLMN services via a stand-alone Non-Public Network, the UE obtains IP connectivity via the Non-Public Network to discovers and establish connectivity to an N3IWF provided by the PLMN.

In the Figure D.3-1, the N1NPN represents the reference point between UE and the AMF in stand-alone Non-Public Network.The NWuPLMN represents the reference point between the UE and the N3IWF in the PLMN for establishing secure tunnel between UE and the N3IWF over the stand-alone Non-Public Network. N1PLMN represents the reference point between UE and the AMF in PLMN.

Figure D.3-2: Access to Non-Public Network services via PLMN

For access to stand-alone Non-Public Network services via PLMN, the UE obtains IP connectivity via the PLMN to discovers and establish connectivity to an N3IWF provided by the stand-alone Non-Public Network.

In Figure D.3-2, the N1NPN represents the reference point between UE and the AMF in stand-alone Non-Public Network.The NWuPLMN represents the reference point between the UE and the N3IWF in the stand-alone Non-Public Netowkr for establishing secure tunnel between UE and the N3IWF over the PLMN. N1PLMN represents the reference point between UE and the AMF in PLMN.

***** End Of Changes *****