3GPP TSG- SA WG2 Meeting #133	S2-1905399

# Reno, NV, USA, 13-17 May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1393 | rev | - | Current version: | 16.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Context Transfer Support in UDSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, Deutsche Telekom, Telecom Italia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2#132, Service Context Transfer procedures (S2-1904843) were apporved in TS23.502. UDSF shall support Service Context Transfer with Any NF/NF service. To specify this, we update the specification of UDSF. Service Context identifier is assigned by UDSF in order to identify Service Context. We also propose that Service Context identifier should be only unique in the same NF or NF service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add the new service in UDSF service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UDSF does not support Service Context Transfer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.14.1, 5.2.14.3(NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * First of change * * * *

5.2.14	UDSF Services

5.2.14.1	General

The following table illustrates the UDSF Services.

Table 5.2.14-1: NF Services provided by UDSF

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Unstructured | Query | Request/Response | Any NF |
| Data | Create | Request/Response | Any NF |
| Management | Delete | Request/Response | Any NF |
|  | Update | Request/Response | Any NF |
| Service | Query | Request/Response | Any NF |
| Context | Push | Request/Response | Any NF |
|  | Delete | Request/Response | Any NF |
|  | Update | Request/Response | Any NF |

NOTE:	whether Nudsf is an exception as compared to other service-based interface due to dynamic data access performance requirement is per stage 3 decision.

* * * * Second of change * * * *

5.2.14.3	Nudsf_ServiceContext service

5.2.14.3.1	General

The Nudsf_ServiceContext serivce provides Nudsf_ServiceContext_Query, Nudsf_ServiceContext_Push, Nudsf_ServiceContext_Delete, and Nudsf_ServiceContext_Update service operations.

5.2.14.3.3	Nudsf_ServiceContext_Query service operation

Service operation name: Nudsf_ ServiceContext _Query.

Description: NF service consumer intends to query Service Context from UDSF.

Inputs, Required: Type of NF or NF service, Service Context Identifier

Service Context Identifier uniquely identifies Service Context in the same NF/NF serivce to be retrieved from the UDSF

Inputs, Optional: None.

Outputs, Required: Requested Service Context (e.g., SM Context of identified PDU session and UE context).

See Table 5.2.8.2.10-1 for single SM Context that may be transferred, and Table 5.2.2.2.2-1 for single UE Context that may be transferred.

.

Outputs, Optional: None.

5.2.14.3.4	Nudsf_ ServiceContext _Push service operation

Service operation name: Nudsf_ ServiceContext _Push.

Description: NF service consumer intends to push Service Context.

Inputs, Required: Service Context Identifier, Service Context, Type of NF or NF service.

Inputs, Optional: None

Outputs, Required: Result indication.

Outputs, Optional: None.

5.2.14.3.5	Nudsf_ ServiceContext _Delete service operation

Service operation name: Nudsf_ ServiceContext _Delete.

Description: NF service consumer intends to delete Service Context stored in the UDSF, e.g. when AMF deletes the particular Service Context in the UDSF.

Inputs, Required: Type of NF or NF service, Service Context Identifier.

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

5.2.14.3.6	Nudsf_ ServiceContext _Update service operation

Service operation name: Nudsf_ ServiceContext _Update.

Description: NF service consumer intends to update Service Context in the UDSF.

Inputs, Required: Service Context Identifier, Service Context, Type of NF or NF service.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

* * * * End of Changes * * * *