3GPP TSG-SA WG2 Meeting #133	S2- 1905747

13 May – 17 May 2019, Reno, USA

Source:	Samsung

Title:	Adding metric and unit within analytics reporting information

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This contribution proposes to explicitly include the metric and unit of analytics information in NWDAF analytics exposure.

- Discussion

Analytic information can be represented in various form in terms of metric and units according to the way of implementation. For example, NF load can be presented in throughput, CPU/memory usage, link utilization, peak rate, and etc. Moreover, a metric represented in various units. For example, memory usage can be represented in percentage, current/maximum byte, current/maximum megabyte, and so on. It leads to introduce a coupling relationship between consumers and NWDAF. If consider multi-vendor and multi-operator environment, the metric and unit should be fully understandable between consumer and producer.

Proposal – explicitly include the metric and unit of analytic information as contents of analytic exposure.

2.	Proposal

It is proposed to include the added text into the TS 23.288. This contribution proposes to include metric and unit of analytic information in the content of analytics exposure.

* * * * First Change * * * *

6.1.3	Contents of Analytics Exposure

The NFs/AFs/ OAM subscribing to (using Nnwdaf_ AnalyticsSubscription) or requesting (using Nnwdaf_AnalyticsInfo) analytics to NWDAF may provide:

-	Analytics ID: one or multiple Analytics ID(s)

NOTE `1:	Analytics ID(s) are also used by AFs subscribing to analytics information via NEF. NEF controls the mapping of Analytics IDs allowed to be exposed to AFs. Details in clauses 6.1.1.2 and 6.1.2.2.

-	Analytics Filter Information. This set of parameter types and values enables to select which type of analytics information is requested (e.g. subset of all available analytics produced by NWDAF for the given Analytics ID value).

-	Target of Analytics Reporting: the object targeted by analytics. The target of analytics reporting indicates entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from NWDAF with this subscription.

-	Analytics Reporting Information with the following parameters:

-	Parameters defined in Table 4.15.1-1, TS 23.502 [3].

-	Observation period: time interval [start..end], either in the past or in the future, expressed as positive or negative offsets to the present. An interval in the past is a request for statistics. An interval in the future is a request for predictions. A default value means statistics in the recent past, with a time range defined by the NWDAF.

-	Preferred level of accuracy of the analytics (e.g. Low/High).

-	Time when analytics are needed (if applicable).

-    Metric and unit of analytics reporting information

NOTE 2:	The feasibility of the parameter "Time when analytics are needed" will be checked by stage 3.

The NWDAF provides analytics feedback to NFs/AFs/ OAM by notifying them (using Nnwdaf_AnalyticsSubscription service) or by responding to analytics requests (using Nnwdaf_AnalyticsInfo service) to analytics with following information:

-	The Notification Correlation Information provided in the subscription request.

-	The Analytics ID(s),

-	Related list of analytics on the requested observation period;

-	Optional additional information:

-	Timestamp of analytics generation, which allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period, which defines the time period for which the analytics information is valid.

-	Probability assertion: level of certainty, degree of confidence in statistics/prediction.

-    Metric and unit of analytics reporting information

* * * * End of change * * * *