3GPP TSG SA WG2 Meeting #133	S2-1905837

13th – 17th May 2019, Reno, Nevada, USA 	was 05781 merge 5088

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1450 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Capturing 23.402 impacts for 5GS interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Cisco Systems, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2019-04-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Moving over impacts to 23.402 for 5GS IWK to 23.502 Subclause 4.11.2.4.3 for initial attach on S2b is misplanced under clause for interworking without N26. ### 4.11.2 Interworking procedures without N26 interface #### 4.11.2.4 Impacts to EPS Procedures ##### 4.11.2.4.3 Initial Attach on S2b The referenced step 3 of clause 7.2.1 in TS23.402 is incorrect as it refers to PMIP message Proxy Binding Update. Step B.1 of clause 7.2.4 (Initial Attach with GTP on S2b) in TS23.402 should be referenced instead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moving interworking aspects with 5GS from 23.402 to 23.502 Remove 4.11.2.4.3; Move the text for Initial Attach on S2b to the new clause 4.11.4.x; Use the correct referenced step (i.e. step B.1 of clause 7.2.4 in 23.402) for ePDG including 5GS IWK indication; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent approach to documentation of interworking aspects between 23.401 and 23.402. This will also save effort in future to update specifications. Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.4.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 23.402... CR 2994 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 4.11.2	Interworking procedures without N26 interface

#### 4.11.2.4	Impacts to EPS Procedures

##### 4.11.2.4.3	Void

NEXT CHANGE

### 4.11.4	Handover procedures between EPC/ePDG and 5GS

#### 4.11.4.3	Impacts to EPC/ePDG Procedures

##### 4.11.4.3.1		General

This clause captures enhancements to procedures in TS 23.402 [26] to support interworking with 5GS. The architecture for interworking is shown in TS 23.501 [2] clause 4.3.4. with the ePDG connected to SMF+PGW-C and UPF+PGW-U using GTP based S2b.

##### 4.11.4.3.2	ePDG FQDN construction

##### TS 23.402 [26] clause 4.5.4.2 applies with the following modification:

##### -	Tracking/Location Area Identity FQDN: When the 5GC NAS capable UE uses the Tracking Area of the NG-RAN when the UE is registered with the 5GC when constructing the Tracking Area Identity FQDN.

##### 4.11.4.3.3	Initial Attach with GTP on S2b

##### The procedure in TS 23.402 [26] clause 7.2.4 applies with the following modifications:

-	In Step A.1 IKEv2 tunnel establishment procedure, the 5GC NAS capable UE shall indicate its support of 5GC NAS in IKEv2. The UE allocates a PDU Session ID and also includes this in IKEv2 to the ePDG.

-	In Step A.1, UE's mobility restriction parameters related to 5GS or indication of support for interworking with 5GS for this APN or both as defined for MME in clause 4.11.0a.3 apply to the ePDG and are obtained by the ePDG as part of the reply from the HSS via the 3GPP AAA Server. These parameters and the 5G NAS support indicator from the UE, may be used by the ePDG to determine if a combined PGW-C+SMF or a standalone PGW should be selected.

##### -	In Step B.1, if the UE supports 5G NAS and the PDN connection is not restricted to interworking with 5GS by user subscription, the ePDG shall send the 5GS Interworking Indication and the PDU Session ID to the PGW-C+SMF.

##### -	In Step D.1 (Create Session Response), the PGW-C+SMF assigns a S-NSSAI to be associated with the PDN connection as specified in TS 23.501 [2] clause 5.15.7.1. The PGW-C+SMF sends the S-NSSAI to the ePDG.

##### -	In the IKEv2 Authentication Response message, the ePDG sends S-NSSAI to the UE. The UE associates the received S-NSSAI with the PDN Connection.

##### 4.11.4.3.4	Interaction with PCC

When interworking with 5GS is supported and a PGW-C+SMF is selected by the ePDG, policy interactions between PDN GW and PCRF specified in TS 23.402 [26] are replaced by equivalent interactions between PGW-C+SMF and PCF as captured in clause 4.11.0a.2.

##### 4.11.4.3.5	UE initiated Connectivity to Additional PDN with GTP on S2b

##### This procedure in TS 23.402 [26] clause 7.6.3 references the initial attach procedure with GTP on S2b. Impacts to the initial attach procedure with GTP on S2b are captured in clause 4.11.4.3.3 above.

* * * End Of Change * * * *