3GPP TSG SA2 Meeting #133	S2-1905887

# Reno, USA, May 13 – 17 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 23.501 | CR | 1384 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding UDR NF Group ID association functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eIMS5G_SBA |  |  |  |  |  | Date: |  |  | 05/06/2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Curently 5GC uses private identifiers of the susbscriber (e.g. SUPI) to identfy the subscriber to which the service is being invoked IMS needs a way to NF derive NF Group ID’s from IMS public identifiers. However, IMS is not the only potential use case that may only have a subscribers public identifier at the start of a service request, and thus the new functionality to associate the subscriber identifiers with NF group IDs is is introduced to the UDR so that the capability may be re-used in future. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds UDR data set to associate subscriber identifiers with NF Group IDs, and updates NRF, SCP and UDR descriptions to accommodate the service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS CSCF will not be able to determine the HSS instance for a subscriber in response to a SIP request. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6, 6.2.11, 6.2.19, 6.3.1, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First Change ***

### 6.2.6	NRF

The Network Repository Function (NRF) supports the following functionality:

-	Supports service discovery function. Receive NF Discovery Request from NF instance or SCP, and provides the information of the discovered NF instances (be discovered) to the NF instance or SCP.

-	Maintains the NF profile of available NF instances and their supported services.

NF profile of NF instance maintained in an NRF includes the following information:

-	NF instance ID.

-	NF type.

-	PLMN ID.

-	Network Slice related Identifier(s) e.g. S-NSSAI, NSI ID.

-	FQDN or IP address of NF.

-	NF capacity information.

-	NF priority information.

NOTE 1:	This parameter is used for AMF selection, if applicable, as specified in the clause 6.3.5. See clause 6.1.6.2.2 of TS 29.510 [58] for its detailed use.

-	NF Set ID.

-	NF Service Set ID of the NF service instance.

Editor's note:	It is FFS whether the NF Service Set ID is unique within the NF instance or NF Set. If former, it is FFS how to correlate the NF Service Set ID values between the NF instances within the NF Set.

--	NF Specific Service authorization information.

-	if applicable, Names of supported services.

-	Endpoint Address(es) of instance(s) of each supported service.

-	Identification of stored data/information.

NOTE 2:	This is only applicable for a UDR profile. See applicable input parameters for Nnrf_NFManagement_NFRegister service operation in TS 23.502 [3] clause 5.2.7.2.2. This information applicability to other NF profiles is implementation specific.

-	Other service parameter, e.g., DNN, notification endpoint for each type of notification that the NF service is interested in receiving.

-	Location information for the NF instance.

NOTE 3:	This information is operator specific. Examples of such information can be geographical location, data center.

-	TAI(s).

-	Routing Indicator, for UDM and AUSF.

-	One or more GUAMI(s), in case of AMF.

-	SMF area identity(ies) in case of UPF.

-	UDM Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers for UDM.

-	UDR Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers for UDR.

-	AUSF Group ID, range(s) of SUPIs for AUSF.

-	HSS Group ID, range(s) of SUPIs, range(s) of IMPIs, range(s) of IMPU, for HSS.

-	Analytics ID, in case of NWDAF.

NOTE 4:	It is expected service authorization information is usually provided by OA&M system, and it can also be included in the NF profile in the case that e.g. an NF instance has an exceptional service authorization information.

NOTE 5:	It is also expected that the NRF stores (or retrieves from the UDR) a mapping between UDM Group ID and SUPI(s), UDR Group ID and SUPI(s), AUSF Group ID and SUPI(s), HSS Group ID and SUPI(s), IMPI (s), IMPU (s) to enable discovery of UDM, UDR, AUSF, HSS using SUPI, SUPI ranges, IMPI, IMPU as specified in clause 6.3.

In the context of Network Slicing, based on network implementation, multiple NRFs can be deployed at different levels (see clause 5.15.5):

-	PLMN level (the NRF is configured with information for the whole PLMN),

-	shared-slice level (the NRF is configured with information belonging to a set of Network Slices),

-	slice-specific level (the NRF is configured with information belonging to an S-NSSAI).

In the context of roaming, multiple NRFs may be deployed in the different networks (see clause 4.2.4):

-	the NRF(s) in the Visited PLMN (known as the vNRF) configured with information for the visited PLMN.

-	the NRF(s) in the Home PLMN (known as the hNRF) configured with information for the home PLMN, referenced by the vNRF via the N27 interface.

*** Second Change ***

### 6.2.11	UDR

The Unified Data Repository (UDR) supports the following functionality:

-	Storage and retrieval of subscription data by the UDM.

-	Storage and retrieval of policy data by the PCF.

-	Storage and retrieval of structured data for exposure.

-	Application data (including Packet Flow Descriptions (PFDs) for application detection, AF request information for multiple UEs, 5GLAN group information for 5GLAN management).

-	Storage and retreval of association between subscriber ID to NF Group ID.

NOTE 1:	For the subscriber ID to NF Group ID association functionality, implementations may need to use instances for this functionality that are separate and distinct from the instances used for other UDR subscription data functionality (e.g. so that the group ID UDR instance can be used to discover the associations used for the subscription data UDR instance).

The Unified Data Repository is located in the same PLMN as the NF service consumers storing in and retrieving data from it using Nudr. Nudr is an intra-PLMN interface.

NOTE 2:	Deployments can choose to collocate UDR with UDSF.

*** Third Change ***

### 6.2.19	SCP

The Service Communication Proxy (SCP) includes one or more of the following functionalities. Some or all of the SCP functionalities may be supported in a single instance of an SCP:

-	Indirect Communication (see clause 7.1.1 for details).

-	Delegated Discovery (see clause 7.1.1 for details).

-	Message forwarding and routing to destination NF/NF service.

-	Communication security (e.g. authorization of the NF Service Consumer to access the NF Service Producer API), load balancing, monitoring, overload control, etc.

-	Ability to discover and select one or more UDM(s), AUSF(s), UDR(s), PCF(s), HSS(s) with access to subscription data stored in the UDR based on UE's SUPI, SUCI or GPSI (or in the case of HSS IMPI or IMPU).

NOTE 1:	Communication security, e.g. authorization of the NF Service Consumer to access the NF Service Producer's API is specified in TS 33.501 [29].

NOTE 2:	Load balancing, monitoring, overload control functionality provided by the SCP is left up to implementation.

NOTE 3:	Discovery and selection of UDM, AUSF, UDR, HSS may use the UDR functionality associating subscriber IDs with NF Group IDs.

The SCP may be deployed in a distributed manner.

NOTE 4:	More than one SCP can be present in the communication path between NF Services.

*** Fourth Change ***

### 6.3.1	General

The NF discovery and NF service discovery enable Core Network entities (NFs or Service Communication Proxy (SCP)) to discover a set of NF instance(s) and NF service instance(s) for a specific NF service or an NF type. NF service discovery is enabled via the NF discovery procedure, as specified in TS 23.502 [3], clauses 4.17.4, 4.17.5, 4.17.9 and 4.17.10..

Unless the expected NF and NF service information is locally configured on the requester NF, e.g. when the expected NF service or NF is in the same PLMN as the requester NF, the NF and NF service discovery is implemented via the NRF. The Network Repository Function (NRF) is the logical function that is used to support the functionality of NF and NF service discovery as specified in clause 6.2.6.

NOTE 1:	NRF can be colocated together with SCP e.g. for communication option D, depicted in Annex E.

In order for the requested NF type or NF service to be discovered via the NRF, the NF instance need to be registered in the NRF. This is done by sending a Nnrf_NFManagement_NFRegister containing the NF profile. The NF profile contains information related to the NF instance, such as NF instance ID, supported NF service instances (see clause 6.2.6 for more details regarding the NF profile). The registration may take place e.g. when the producer NF instance and its NF service instance(s) become operative for the first time. The NF service registration procedure is specified in TS 23.502 [3], clause 4.17.1.

In order for the requester NF or SCP to obtain information about the NF and/or NF service(s) registered or configured in a PLMN/slice, based on local configuration the requester NF or SCP may initiate a discovery procedure with the NRF by providing the type of the NF and optionally a list of the specific service(s) it is attempting to discover. The requester NF or SCP may also provide other service parameters e.g. slicing related information. For the detailed service parameter(s) used for specific NF and NF service discovery refer to clause 5.2.7.3.2 of TS 23.502 [3].

For some Network Functions storing subscriber data (UDM, UDR, AUSF, HSS) the NRF (or SCP) may need the association between an individual subscriber identifier and a NF Group ID  – this can be done using local configuration or retrieving the NF Group ID associated with the subscriber identifier from the UDR  using the Nudr_DM_Query service operation.

In the case of Indirect Communication, a NF Service Consumer employs an SCP which routes the request to the intended target of the request.

If the requester NF is configured to delegate discovery, the requester NF may omit the discovery procedure with the NRF and instead delegate the discovery to the SCP; the SCP will then act on behalf of the requester NF. In this case, the requester NF adds any necessary discovery and selection parameters to the request in order for the SCP to be able to do discovery and associated selection. The SCP may interact with the NRF to perform discovery and obtain discovery result.

Depending on the chosen message routing model, the NRF may provide the IP address or the FQDN of NF instance(s) or the Endpoint Address(es) of relevant NF services instance(s) to the requester NF. The NRF provides a list of NF instances and NF service instances relevant for the discovery criteria. The result of the NF and NF service discovery procedure is applicable to any subscriber that fulfils the same discovery criteria. The entity that does the discovery A requester (NF or SCP) may cache the NF profile(s) received from the NF/NF service discovery procedure. The stored NF profile(s) may be used for NF selection for any subscriber matching the NF profile(s).

In the case of Direct Communication, the requester NF uses the discovery result to select NF instance and a NF service instance that is able to provide a wanted NF Service (e.g., an instance of the PCF that can provide Policy Authorization).

In the case of Indirect Communication without Delegated Discovery, the requester NF uses the discovery result to select a NF instance while the associated NF service instance selection may be done by the requester NF and/or an SCP on behalf of the requester NF.

In both the cases, the requester NF may use the information from a previously cached discovery result for subsequent selections (i.e. the requester NF does not need to trigger a new NF discovery procedure to perform the selection).

In the case of Indirect Communication with Delegated Discovery, the SCP will select a suitable NF service instance based on discovery and selection parameters provided by the requester NF and optional interaction with the NRF. The NRF to be used may be provided by the NF consumer as part of the discovery parameters, e.g. as a result of a NSSF query.

NOTE 2:	In a given PLMN, Direct Communication, Indirect Communication, or both may apply.

If the end point address provided by a NF service producer points to a specific target NF producer/NF service producer instance within a Set, the NF consumer or SCP may re-select another service instance of the same NF service within the same NF service Set in that same NF instance, or another NF instance in the same NF Set for subsequent transaction, and get the same results as if the original selected NF service instance was used. Re-selecting may be needed if the requester NF fails to contact the original NF service instance. If the end point address provided by the NF service producer points to an NF Set or NF Service Set, the NF consumer or SCP is allowed to reselect a different NF service instance of the same NF service within that same Set for the UE and its subsequent transaction.

Editor's note:	To be updated if needed based on the outcome of the ongoing discussions regarding the SET concept.

The requester NF or SCP may subscribe to receive notifications from the NRF of a newly updated NF profile of an NF (e.g. NF service instances taken in or out of service), or newly registered de-registered NF instances. The NF/NF service status subscribe/notify procedure is defined in TS 23.502 [3], clauses 4.17.7 and 4.17.8.

For NF and NF service discovery across PLMNs, the NRF in the local PLMN interacts with the NRF in the remote PLMN to retrieve the NF profile(s) of the NF instance(s) in the remote PLMN that matches the discovery criteria. The NRF in the local PLMN reaches the NRF in the remote PLMN by forming a target PLMN specific query using the PLMN ID provided by the requester NF. The NF/NF service discovery procedure across PLMNs is specified in clause 4.17.5 of TS 23.502 [3].

NOTE:	See TS 29.510 [58] for details on using the target PLMN ID specific query to reach the NRF in the remote PLMN.

For topology hiding, see clause 6.2.17.

*** End of Changes ***