3GPP TSG-SA WG2 Meeting #133	S2-1906084

13 - 17 May 2019, USA, Reno	merge of 5261, 5541 and 5066

Source:	ORANGE, Huawei, HiSilicon, KDDI, Toyota Motor Corporation

Title:	Network Performance Analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes additions to “Network Performance analytics”.

# Discussion

This discussion addresses the following topics:

- Request parameters

It is stated in the previous version of the text that only Group ID is possible as target of Reporting Information.

We propose to add also an individual UE, or any UE. In the latter case, the Analytics Filter Information must be provided, containing the Area of Interest. 

-	 Limitation of results

For large UE groups or for UEs moving on large areas, it may be necessary to limit the size of results. This can be done by requesting results on a dedicated area.

- Inputs of performance analytics

The purpose is to analyse which information is available in OAM, with the reference document 28.552.

The load information collected is restricted to RAN nodes.

The performance information collected focuses on network integrity (handover and communication set-up).

- Outputs of performance analytics

The purpose is to define what are the output of the NWDAF, focusing on load and basic network efficiency.

The performance information produced focuses on network integrity (handover and communication set-up).

# Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

## 6.6	Network Performance Analytics

### 6.6.1	General

Network Performance Analytics provides either statistics or predictions on the load in an Area of Interest; in addition it may provide statistics or predictions on the number of UEs that are located in that Area of Interest.

The service consumer may be an NF (e.g. PCF, NEF), an AF, or the OAM.

The consumer of these analytics may indicate in the request:

-	Analytics ID set to "Network Performance";

-

Target of Analytics Reporting containing either a UE, or an Internal Group Identifier that refers to the group for which the analytics on the number of UEs that are located in the Area of Interest at the time indicated in the Observation period is requested.  Otherwise the Target of Analytics Reporting shall be set to "any UE";

-	Analytics Filter Information containing:

-	optional maximum number of results,

-	Area of Interest (list of TA or Cells) which restricts the area in focus (mandatory if Target Of Event Reporting is set to "any UE", optional otherwise),

-	optional DNN(s),

-	optional S-NSSAI(s),

-	optional list of analytics subset(s), i.e. gNB resource usage, Number of UEs, Communication performance, Mobility performance;

-	An Observation period indicates the time when the statistics or prediction are requested; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.6.3.

### 6.6.2	Input Data

The NWDAF collects Load and Performance information (or subset thereof depending on what the consumer requested) in an Area of Interest from the sources listed in Table 6.6.2-1 and number of UEs within Area of Interest from the sources listed in Table 6.6.2-2.

NOTE:	The tables below shows the information provided to the NWDAF, in other words the information provided by NWDAF is not shown here but in the procedures in clause 6.6.4.

Table 6.6.2-1:  Network Performance information collected by NWDAF

| Load and performance information |  |  |  | Source |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Load and performance information |  |  |  | OAM |  | Statistics on RAN load and performance per CellId in the Area of Interest. |  |
| > Virtualized Resource usage of NF (5G NF or NB) |  |  |  |  |  | Usage of assigned virtualized resources currently in use for a specific NF instance (mean usage of virtual CPU, memory, disk; see TS 28.552 [8] clause 5.7). |  |  |  |
| > Number of PDU Sessions requested to setup |  |  |  |  |  | Number of PDU Sessions in the PDU SESSION RESOURCE SETUP REQUESTs received by the gNB from AMF (see TS 28.552 [8] clause 5.1.1.5). |  |
| > Number of PDU Sessions successfully setup |  |  |  |  |  | Number of PDU Sessions successfully setup by the gNB for the PDU SESSION RESOURCE SETUP REQUESTs received from AMF (see TS 28.552 [8] clause 5.1.1.5). |  |
| > Number of requested handover preparations |  |  |  |  |  | Number of handover preparations requested by the source gNB (see TS 28.552 [8] clause 5.1.1.6). |  |  |  |
| > Number of successful handover preparations |  |  |  |  |  | Number of successful handover preparations received by the source NR cell CU (see TS 28.552 [8] clause 5.1.1.6). |  |  |  |
| > Number of requested handover resource allocations |  |  |  |  |  | Number of handover resource allocation requests received by the target NR cell CU (see TS 28.552 [8] clause 5.1.1.6). |  |  |  |
| > Number of successful handover resource allocations |  |  |  |  |  | Number of successful handover resource allocations at the target NR cell CU for the handover (see TS 28.552 [8] clause 5.1.1.6). |  |
| User plane activity information |  |  |
| > UE Throughput |  | Per TAI or per Cell ID measurements related to information transferred over the user plane (see TS 28.552 clause 5.1.1.3). |
| > RRC Connection Number |  | Per TAI or per Cell ID (see TS 28.552 clause 5.1.1.4). |
| > Radio Resource Utilization |  | Per TAI or per Cell ID measurements related to information transferred over the user plane (see TS 28.552 clause 5.1.1.2). |
| > Upstream Throughput for Network and Network Slice Instance |  | Per TAI or per Cell ID measurements related to information transferred over the user plane (see TS 28.554 clause 6.3.2). |
| > Downstream Throughput for Single Network Slice Instance |  | Per TAI or per Cell ID measurements related to information transferred over the user plane (see TS 28.554 clause 6.3.3). |
| > RAN UE Throughput |  | Per TAI or per Cell ID measurements related to information transferred over the user plane (see TS 28.554 clause 6.3.6). |

Table 6.6.2-2:  Number of UEs in Area of Interest information collected by NWDAF

| Number of UEs information | Source | Description |
| --- | --- | --- |
| Number of UEs | AMF | Number of UEs in an Area of Interest |

### 6.6.3	Output Analytics

The NWDAF shall be able to provide both statistics and predictions on Network Performance.

Network performance statistics are defined in Table 6.6.3-1.

Table 6.6.3-1: Network performance statistics

| Information | Description |
| --- | --- |
| List of performance information (1..max) | Observed statistics during the observation period |
| > Area subset | TA or Cell ID within the requested area of interest |
| > gNB resource usage | Usage of assigned resources (CPU, memory, disk) (average, peak) |
| > Number of UEs | Number of UEs observed in the area (TA or Cell) (average) |
| > Communication performance | Ratio of successful setup of PDU Sessions (average) |
| > Mobility performance | Ratio of successful handover (average) |

Network performance predictions are defined in Table 6.6.3-2.

Table 6.6.3-1: Network performance predictions

| Information | Description |
| --- | --- |
| List of performance information (1..max) | Predicted analytics during the observation period |
| > Area subset | TA or Cell ID within the requested area |
| > gNB resource usage | Usage of assigned resources (CPU, memory, disk) (average, peak) |
| > Number of UEs | Number of UEs predicted in the area |
| > Communication performance | Ratio of successful setup of PDU Sessions |
| > Mobility performance | Ratio of successful handover |
| > Confidence | Confidence of this prediction |

The number of results shall be limited by the input parameter "maximum number of results".

Editor's Note: Output analytics related to User plane activity information are FFS.

### 6.6.4	Procedures

OAM

NF

NWDAF

1-Nnwdaf_AnalyticsInfo_Request

/ Nnwdaf_AnalyticsSubscription_Subscribe

AMF

4a-Namf_EventExposure_Subscribe

4b-Namf_EventExposure_Notify

2a-Nnrf_NFDiscovery_Request (AMF Region)

2b-Nnrf_ NFDiscovery_Request Response

6-Nnwdaf_AnalyticsInfo_Response

/ Nnwdaf_AnalyticsSubscription_Notify

5- NWDAF derives new analytics

NRF

3a -Noam NFStatus Subscribe

3b-Noam  Nstatus Notify

10- Nnwdaf_AnalyticsSubscription_Notify

9- NWDAF derives new analytics

7- Nnrf_Management_NfStatus_Notify

8- Noam  Nstatus Notify

2c-Nnrf_Management_NfStatus_Subscribe

2d-Nnrf_Management_NfStatus_Notify

OAM

NF

NWDAF

1-Nnwdaf_AnalyticsInfo_Request

/ Nnwdaf_AnalyticsSubscription_Subscribe

AMF

4a-Namf_EventExposure_Subscribe

4b-Namf_EventExposure_Notify

2a-Nnrf_NFDiscovery_Request (AMF Region)

2b-Nnrf_ NFDiscovery_Request Response

6-Nnwdaf_AnalyticsInfo_Response

/ Nnwdaf_AnalyticsSubscription_Notify

5- NWDAF derives new analytics

NRF

3a -Noam NFStatus Subscribe

3b-Noam  Nstatus Notify

10- Nnwdaf_AnalyticsSubscription_Notify

9- NWDAF derives new analytics

7- Nnrf_Management_NfStatus_Notify

8- Noam  Nstatus Notify

2c-Nnrf_Management_NfStatus_Subscribe

2d-Nnrf_Management_NfStatus_Notify

Figure 6.6.4-1: Procedure for subscription to network performance analytics

1.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request  to the NWDAF, including some of the information specified in clause 6.6.1.

2.	The NWDAF discovers from NRF the AMF(s) belonging to the AMF Region(s) that include(s) the Area of Interest and other NFs if required based on Analytics Filter Information.  .

3.	The NWDAF subscribes to OAM to collect the information stated in Table 6.6.2-1on the Area of Interest, pertaining to the RAN and to the NFs discovered in step 2, following the procedure captured in Clause 6.2.3.2.

4.	The NWDAF collects the number of UEs located in the Area of Interest from AMF using Namf_EventExposure_Subscribe service, including the Target Event Reporting provided as an input parameter (i.e. any UE or Internal Group Identifier).

5.	The NWDAF derives the requested analytics.

6. 	The NWDAF sends Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response (one or more instances of Performance Information SubscriptionCorrelationId, Probability of assertion).

7-8.	A change of network performance information, e.g. change in the load in the area of interest at the observed period, is detected by OAM and is notified to NWDAF.

9.	The NWDAF derives new analytics based on the most recent data collected.

10.	The NWDAF provides a notification using Nnwdaf_AnalyticsSubscription_Notify (one or more instances of Performance Information) SubscriptionCorrelationId, Probability of assertion.

* * * End of changes * * * *