3GPP TSG-SA WG2 Meeting #133	S2-1906155

13 - 17 May 2019, USA, Reno	rev of 6084

Source:	ORANGE, Huawei, HiSilicon, KDDI, Toyota Motor Corporation, Ericsson

Title:	Network Performance Analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes additions to “Network Performance analytics”.

# Discussion

This discussion addresses the following topics:

- Request parameters

It is stated in the previous version of the text that only Group ID is possible as target of Reporting Information.

We propose to add also an individual UE, or any UE. In the latter case, the Analytics Filter Information must be provided, containing the Area of Interest. 

-	 Limitation of results

For large UE groups or for UEs moving on large areas, it may be necessary to limit the size of results. This can be done by requesting results on a dedicated area.

- Inputs of performance analytics

The purpose is to analyse which information is available in OAM, with the reference document 28.552.

The load information collected is restricted to RAN nodes.

The performance information collected focuses on network integrity (handover and communication set-up).

- Outputs of performance analytics

The purpose is to define what are the output of the NWDAF, focusing on load and basic network efficiency.

The performance information produced focuses on network integrity (handover and communication set-up).

# Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

## 6.6	Network Performance Analytics

### 6.6.1	General

Network Performance Analytics provides either statistics or predictions on the load in an Area of Interest; in addition it may provide statistics or predictions on the number of UEs that are located in that Area of Interest.

The service consumer may be an NF (e.g. PCF, NEF), an AF, or the OAM.

The consumer of these analytics may indicate in the request:

-	Analytics ID set to "Network Performance";

-

Target of Analytics Reporting containing either a UE, or an Internal Group Identifier that refers to the group for which the analytics on the number of UEs that are located in the Area of Interest at the time indicated in the Observation period is requested;

-	Analytics Filter Information containing:

-	optional maximum number of results,

-	Area of Interest (list of TA or Cells) which restricts the area in focus (mandatory if Target Of Event Reporting is set to "any UE", optional otherwise),

-	An Observation period indicates the time when the statistics or prediction are requested; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.6.3.

### 6.6.2	Input Data

The NWDAF collects Load and Performance information in an Area of Interest from the sources listed in Table 6.6.2-1 and number of UEs within Area of Interest from the sources listed in Table 6.6.2-2.

NOTE:	The tables below shows the information provided to the NWDAF, in other words the information provided by NWDAF is not shown here but in the procedures in clause 6.6.4.

Table 6.6.2-1:  Load and Performance information collected by NWDAF

| Information |  |  |  | Source |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Load and performance information |  |  |  | OAM |  | Statistics on RAN load and performance per CellId in the Area of Interest. |  |
| Load information | NRF | Load per NF |

Table 6.6.2-2:  Number of UEs in Area of Interest information collected by NWDAF

| Number of UEs information | Source | Description |
| --- | --- | --- |
| Number of UEs | AMF | Number of UEs in an Area of Interest |

### 6.6.3	Output Analytics

The NWDAF shall be able to provide both statistics and predictions on Network Performance.

Network performance statistics are defined in Table 6.6.3-1.

Table 6.6.3-1: Network performance statistics

| Information | Description |
| --- | --- |
| List of performance information (1..max) | Observed statistics during the observation period |
| > Area subset | TA or Cell ID within the requested area of interest |

Network performance predictions are defined in Table 6.6.3-2.

Table 6.6.3-2: Network performance predictions

| Information | Description |
| --- | --- |
| List of performance information (1..max) | Predicted analytics during the observation period |
| > Area subset | TA or Cell ID within the requested area |
| > gNB resource usage | Usage of assigned resources (CPU, memory, disk) (average, peak) |
| > Number of UEs | Number of UEs predicted in the area |
| > Communication performance | Ratio of successful setup of PDU Sessions |
| > Mobility performance | Ratio of successful handover |
| > Confidence | Confidence of this prediction |

The NWDAF provides Network Performance Analytics to a consumer that includes the load in the area of interest and the number of UEs located in the area of interest or both at the time requested by the consumer in the Observation Period:

-	Analytics ID set to "Network Performance".

-	Notification Target Address including the address of the consumer.

-	Notification Correlation Id, for the consumer to correlate notifications from NWDAF if subscription applies.

-	Analytics specific parameters including the load in the Area of Interest and the number of UEs in the Area of Interest, at the time indicated in the Observation period.

### 6.6.4	Procedures

OAM

NF

NWDAF

1-Nnwdaf_AnalyticsInfo_Request

/ Nnwdaf_AnalyticsSubscription_Subscribe

AMF

4a-Namf_EventExposure_Subscribe

4b-Namf_EventExposure_Notify

2a-Nnrf_NFDiscovery_Request (AMF Region)

2b-Nnrf_ NFDiscovery_Request Response

6-Nnwdaf_AnalyticsInfo_Response

/ Nnwdaf_AnalyticsSubscription_Notify

5- NWDAF derives new analytics

NRF

3a -Noam NFStatus Subscribe

3b-Noam  Nstatus Notify

10- Nnwdaf_AnalyticsSubscription_Notify

9- NWDAF derives new analytics

7- Nnrf_Management_NfStatus_Notify

8- Noam  Nstatus Notify

2c-Nnrf_Management_NfStatus_Subscribe

2d-Nnrf_Management_NfStatus_Notify

Figure 6.6.4-1: Procedure for subscription to network performance analytics

1.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request (Analytics ID="Network Performance", AnalyticsFilter="Area of interest" or "Area of interest" and "Internal Group Identifier", Observation Period(s)) to the NWDAF.

2.	The NWDAF discovers from NRF the AMF(s) belonging to the AMF Region(s) that include(s) the Area of Interest  and subscribes to NF load information from NRF about these AMF(s).

3.	The NWDAF subscribes to OAM to get the load information on the Area of Interest, following the procedure captured in Clause 6.2.3.3.

Editor's note:	The exact SA5 service, granularity (NF, eNB) and measurement or KPI that are used for load

4.	The NWDAF collects the number of UEs located in the Area of Interest from AMF using Namf_EventExposure_Subscribe service, including the Target Event Reporting provided as an input parameter (i.e. any UE or Internal Group Identifier)..

Editor's note: An Event ID such as "Number of UEs present in a geographical area" needs to be added to TS 23.502 clause 5.2.2.3.1, as currently this Event ID is only specified for NEF.

5.	The NWDAF derives the requested analytics.

6. 	The NWDAF sends Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response (one or more instances of the n-tuple (Load in Area of Interest and number of UEs, timestamp) SubscriptionCorrelationId, Probability of assertion).

7-8.	A change of network performance information, i.e. change in the load in the area of interest at the observed period, is detected by OAM, or a change in the NF load information is reported by NRF, and is notified to NWDAF.

9.	The NWDAF derives new analytics based on the most recent data collected.

10.	The NWDAF provides a notification using Nnwdaf_AnalyticsSubscription_Notify (one or more instances of the n-tuple (Load in Area of Interest and number of UEs, timestamp) SubscriptionCorrelationId, Probability of assertion.

* * * End of changes * * * *