SA WG2 Meeting #133	S2-1906464

13 – 17 May, 2019, Reno, Nevada, USA	(was S2-1905457+5318+5193)

Source:	CATT, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

Title:	GMLC selection

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	5G_eLCS / Rel-16

Abstract of the contribution: This paper proposes to add description of GMLC selection.

1.	Discussion

The analysis of GMLC selection described in TS 23.273 is shown in Table 1.

Table 1 Analysis of GMLC selection in TS 23.273

| Clause | GMLC selection Description | Analysis |  |  |
| --- | --- | --- | --- | --- |
|  |  | NF to Select GMLC | Methods | Parameters |
| Step 4 in clause 6.1.2 | For a non-roaming case, this step is skipped. In the case of roaming, the H-GMLC may receive an address of a V-GMLC (together with the network address of the current serving AMF) from the UDM in step 3, otherwise, the H-GMLC may use the NRF service in the HPLMN to select an available V-GMLC in the VPLMN, based on the VPLMN identification contained in the AMF address received in step 3. | GMLC | Requesting UDM or NRF | PLMN ID (When NRF is used) |
| Step 2 in clause 6.2 | For an LCS 5GC-MO-LR requesting location transfer to an LCS Client, the AMF shall assign a GMLC address, i.e. V-GMLC address, which is stored in the AMF. If a V-GMLC address is not available, the AMF may reject the location request. | AMF | Using configuration information in AMF |  |
| Step 28 in clause 6.3.1 | In the case of roaming, the LMF selects a V-GMLC (which may be different to the V-GMLC for steps 3-8 and steps 19-21), The LMF then invokes an Nlmf_EventNotify service operation towards the selected V-GMLC or (H-)GMLC. NOTE 10: In the case of roaming, the LMF may select the V-GMLC for step 28 using the NRF service or using configuration information in the LMF or may use the same V-GMLC as for steps 3-8 (e.g. if the LMF acts as a serving LMF and received the V-GMLC address from the AMF as part of step 14). | LMF | Using NRF service or configuration information in LMF |  |
| Step 3 in clause 6.3.2 | In the case of roaming, the LMF selects a V-GMLC. The LMF then invokes an Nlmf_EventNotify service operation towards the selected V-GMLC or (H-)GMLC. NOTE 1: In the case of roaming, the LMF may select the V-GMLC for step 3 using the NRF service or using configuration information in the LMF or may use the same V-GMLC as for steps 3-8 of clause 6.3.1 (e.g. if the LMF acts as a serving LMF and received the V-GMLC address from the AMF as part of step 14 of clause 6.3.1). | LMF | Using NRF service or configuration information in LMF |  |
| Step 5 in clause 6.10.1 | For emergency services, the AMF selects an GMLC based on NRF query or configuration in AMF. The information regarding the endpoint in the GMLC to deliver the event notification, is obtained from the NRF as specified in clause 7.1.2 of TS 23.501 [18] or from local configuration in the AMF. | AMF | Using NRF service or configuration information in AMF |  |

As shown in Table 1, GMLC, AMF or LMF performs GMLC selection by using local configuration information, NRF service or UDM service. When NRF service or UDM service is used, neither new parameter nor new mechanism is introduced for GMLC selection. So it is proposed to reuse these services described in TS 23.502 for GMLC selection.

Furthermore, the clause 5.2 in TS 23.273 is empty. Because no specific function is introduced for 3GPP access, so it is proposed to rename the title of the clause 5.2 to GMLC selection.

2.	Proposal

It is proposed to update TS 23.273 as follows. All are new texts.

*** First change all new text***

5.x	GMLC Discovery and Selection

More than one GMLC in the HPLMN can serve the location requests for a single UE. GMLC discovery and selection functionality is supported by AMF, LMF, NEF, LCS client and GMLC.

A LCS client may be configured with GMLC address(es). It may also determine the GMLC address by performing a DNS query.

A NEF, LMF, AMF or GMLC may be configured with GMLC address(es). Those NF  may also query the NRF to get GMLC address(es).

In the following scenarios, information about the GMLC instance may be provided by UE, in such case, this GMLC instance is used:

- In the deferred MT-LR procedure, when UE reports the detected event to the AMF, it may also include the (H-)GMLC contact address.

- In the MO-LR procedure, when UE initiates the LCS service request, it may also include the (H-)GMLC contact address if the location estimation is reported to the (H-)GMLC.

*** End of change ***