3GPP TSG-SA2 Meeting #133	S2-1906741

# Reno, Nevada, United States, 13th May 2019 - 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1120 | rev | 9 | Current version: | 16.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving the EN in Bindings between the consumer and producer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Huawei, Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-03-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Align the binding procedures with the NF Set and NF Service Set concepts, and resolve the related Editor’s Note. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe the supported binding addresses, aligned with the NF Set and NF Service Set concepts. Add a new clause for update of the bindings between the consumer and producer. Rev 4: Only the bindings to NF instance, NF service instance, and NF Set are possible when the binding is created as part of service request. Rev 7: Added resource information. Clarified in 4.17.11.3, step 3 has difference with step 3-4 in figure 4.17.11.2-1. Clarified the procedures regarding subscription/notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in the specification. Unclear how the bindings are updated and released. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.11, 4.17.11.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************** First Change ****************

*** BEGIN CHANGES ***

### 4.17.11 Binding between NF service consumer and NF service producer

#### 4.17.11.1 General

This clause describes the procedures to establish binding between the NF service consumer and producer.

Direct Communication or Indirect Communication procedures may be used between the Consumer and Producer. In case of Indirect Communication, an SCP is located between the Consumer and Producer.

#### 4.17.11.2 Binding created as part of service response

When the NF service consumer communicates with the NF service producer, the producer may return a binding indication to the consumer. The consumer stores the received binding indication and uses it for the subsequential requests concerning the data context.

Figure 4.17.11.2-1: Binding created as part of service response

1. 	If Direct Communication is used, the NF service consumer selects the NF service producer and sends the request to the selected NF service producer. If Indirect Communication mode is used the NF service consumer sends the request to the SCP and provides within the service request to the SCP the discovery and selection parameters necessary to discover and select a NF cervice producer.

2. 	The NF service producer sends a response to the NF service consumer. In the response the NF service producer may include a a binding indication. If the NF service consumer receives a  resource information and binding indication as specified in Table 6.3.1.0-1 in TS 23.501 [2] it uses  them for subsequent requests regarding the concerned resource. Otherwise, the procedure ends here.

3.	The NF service consumer uses the binding indication and resource information received in the previous step for subsequent requests regarding the concerned resource.

4. 	The NF service producer sends a response to the consumer. The NF service producer may respond with an updated resource information and binding indication or both, different to the one received in the previous response.

#### 4.17.11.3 Binding created as part of service request

If the NF service consumer can also be as a NF service producer for later communication from the contacted producer, a service request sent to  the producer may include binding indication

Figure 4.17.11.3-1: Binding created as part of service request

1. 	Instance A, as an NF service consumer sends a service request using either Direct Communication or Indirect communication via SCP and Instance B is selected as NF service producer. If Instance A, can also be NF service producer for later communication for the concerned data context, it may include binding  indication referring to NF service instance, NF instance or NF Set as specified in Table 6.3.1.0-1 in TS 23.501 [2] in the request sent to the NF service producer.

2. 	Instance B as the NF service producer sends a response to the NF service consumer.

3.	When Instance B as NF service consumer needs to invoke the service provided by Instance A, Instance B sends a request using the binding indication received in step 1 as described in Steps 3-4 in Figure 4.17.11.2-1 with the following difference:

- Based on the received binding indication, if delegated discovery is not used, the Instance B may need to discover the corresponding endpoint address of the Instance A.

#### 4.17.11.4 Binding for subscription requests

Binding can be created as part of the implicit subscription request. In this case, the response to the service request that creates the implicit subscription may include a binding indication as in 4.17.11.2. For subsequent operations on the created implicit subscription, bindings may be used. The description in 4.17.11.1 applies for Figure 4.17.11.4.-1.

If the binding needs to be updated, the NF service consumer may initiate a new request to the NF service producer with an updated Notification Correlation ID and binding indication.

Figure 4.17.11.4-1: Binding for subscription requests

*** END CHANGES ***