3GPP TSG-SA WG2 Meeting #134 	S2-1906926

Sapporo, Japan, 24 – 28 June, 2019	(revision of S2-190xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1461 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to QoS support for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIOT |  |  |  |  |  | Date: |  |  | 2019-06-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 has requested feedback from SA2 in incoming LS on support for flow based QoS & slicing for NB-IoT & eMTC connected to 5GC (S2-1906863/R2-1908264). The LS brings up multiple considerations for - QoS model in NB-IoT - Network Slicing support in NB-IoT S2-190924 describes a way forward for both cases. This contribution provides the required clarifications to QoS model and Network slicing in NB-IoT in line with the proposals in S2-190924. |  |  |  |  |  |  |  |  |
| Summary of change: |  | QoS model for NB-IoT: - New section to include requirement of only a default QoS rule per PDU session, maximum of 2 Data Radio Bearers. - Inter-RAT idle mode mobility to Nb-IoT: SMF needs to initiate PDU sesion modification to remove any non-default QoS rule. Network slicing for NB-IoT: - The UE shall not provide NSSAI in the Access stratum for NB-IoT. - The NAS Requested NSSAI limited to 2 S-NSSAIs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QoS requirements: unnecesary additional complexity which is very detrimental given that NB-IoT is meant to enable low cost UEs. Network slicing: excessive signalling overhead over NB-IoT due to Network Slicing, increasing power consumption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.31.11, 5.31.X (new), 5.31.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 5.31.11	Inter-RAT idle mode mobility to and from NB-IoT

Tracking Areas are configured so that they do not contain both NB-IoT and other RATs cells, so when the UE is changing RAT type to or from NB-IoT while remaining registered with 5GC, the UE will perform the Mobility Registration Update procedure, see clause 5.3.2.3. When the UE is changing RAT type to or from NB-IoT and moving between 5GC and EPC, during the Registration, Attach or TAU procedure the RAT type change is determined.

PDU session handling is controlled by "PDU Session continuity at inter RAT mobility" in the UEs subscription data, which indicates per DNN/S-NSSAI whether to;

-	maintain the PDU session,

-	disconnect the PDU session with a reactivation request,

-	disconnect the PDU session without reactivation request, or

-	leave it up to local VPLMN policy

when the UE moves between a "broadband" RAT (e.g. NR or WB-E-UTRAN) and a "narrowband" RAT (NB-IoT).

During PDU session establishment the SMF retrieves the "PDU Session continuity at inter RAT mobility" subscription information (if available) from the UDM. Local SMF configuration is used if "PDU Session continuity at inter RAT mobility" is not available for a PDU Session.

The AMF informs the SMF at an inter-RAT idle mobility event, e.g. to or from NB-IoT connected to 5GC about the RAT type change in the Nsmf_PDUSession_UpdateSMContext message during the Registration procedure. Based on this (H-)SMF handles the PDU session according to "PDU session continuity at inter RAT mobility information" subscription data or based on local policy.

NOTE:	The "PDU Session continuity at inter RAT mobility" and "PDN continuity at inter-RAT mobility" subscription should be the same so that the PDU sessions/PDN connections are handled the same by both CN types.

During inter-RAT idle mode mobility to NB-IoT, if a PDU session has more than one QoS rule, the SMF shall initiate a PDU session modification procedure as described in TS 23.502 [3] to remove any non-default QoS rule, and maintain only the default QoS rule.

*** NEXT CHANGES***

### 5.31.X	QoS model for NB-IoT

5GC QoS model described in clause 5.7 applies to NB-IoT with the following requirements:

-	The default QoS rule shall be the only QoS rule of a PDU Session for a UE connected to 5GC via NB-IoT. There is only one QoS flow (corresponding to the default QoS rule) per PDU session.

-	Reflective QoS is not supported over NB-IoT.

-	For NB-IoT, there is a 1:1 mapping between the QoS flow corresponding to the default QoS of a PDU session and a Data Radio Bearer when user plane resources are active for that PDU session.

-	A maximum of two Data Radio Bearers are supported over NB-IoT. Therefore, a maximum of two PDU sessions can have active user plane resources at the same time.

*** NEXT CHANGES***

### 5.31.Y	Network Slicing for NB-IoT

Network Slicing as described in clause 5.15 applies to NB-IoT with the following additional requirements:

-	The UE shall not provide NSSAI in the Access stratum for NB-IoT. The AMF may still provide the Access Stratum Connection Establishment NSSAI Inclusion Mode parameter as described in clause 5.15.9 while the UE is camping on NB-IoT.  In that case the UE ignores it while camping in NB-IoT, but the UE uses de mode received in the Access Stratum Connection Establishment NSSAI Inclusion Mode parameter in case of inter-RAT idle mobility to a different RAT, e.g. WB-E-UTRAN or NR.

-	The UE shall provide a maximum of 2 S-NSSAIs in the NAS Requested NSSAI. How the UE reduces the number of S-NSSAI in Requested NSSAI to 2 S-NSSAIs if needed is up to UE implementation.

*** END CHANGES ***